| oneM2M Technical Report |  |
| --- | --- |
| Document Number | oneM2M-TR-0026-V-4.8.0 |
| Document Name: | Vehicular Domain Enablement |
| Date: | 2019-12-17 |
| Abstract: | This oneM2M Technical Report examines how the current oneM2M System can be used in the Vehicular Domain and includes a study of advanced features which the future oneM2M release(s) could support for this vertical domain. |
| Template Version: 08 September 2015 (Dot not modify) |  |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

# About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

# More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

© 2017, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TSDSI, TTA, TTC).

All rights reserved.

The copyright and the foregoing restriction extend to reproduction in all media.

# Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

1	Scope	13

2	References	13

2.1	Normative references	13

2.2	Informative references	13

3	Definitions, symbols and abbreviations	14

3.1	Definitions	14

3.2	Symbols	14

3.3	Abbreviations	14

4	Conventions	15

5	Introduction to Vehicular Domain	15

5.1	Vehicular Domain Overview	15

5.2	Technology Trends in Vehicular Domain	15

5.3	The focus of oneM2M in Vehicular Domain	16

5.4	Levels of Driving Automation	18

6	Vehicular Domain Use Cases	19

6.1	Vehicle Diagnostic & Maintenance Report	19

6.1.1	Description	19

6.1.2	Source	19

6.1.3	Actors	19

6.1.4	Pre-conditions	20

6.1.5	Triggers	20

6.1.6	Normal Flow	20

6.1.7	Alternative Flow	21

6.1.8	Post-conditions	21

6.1.9	High Level Illustration	21

6.1.10	Potential Requirements	21

6.2	Remote Maintenance Services	21

6.2.1	Description	21

6.2.2	Source	22

6.2.3	Actors	22

6.2.4	Pre-conditions	22

6.2.5	Triggers	22

6.2.6	Normal Flow	22

6.2.7	Alternative Flow	23

6.2.8	Post-conditions	23

6.2.9	High Level Illustration	23

6.2.10	Potential Requirements	24

6.3	Traffic Accident Information Collection	24

6.3.1	Description	24

6.3.2	Source	24

6.3.3	Actors	24

6.3.4	Pre-conditions	25

6.3.5	Triggers	25

6.3.6	Normal Flow	25

6.3.7	Alternative Flow	26

6.3.8	Post-conditions	26

6.3.9	High Level Illustration	27

6.3.10	Potential Requirements	27

6.4	Fleet Management Service using DTG (Digital Tachograph)	28

6.4.1	Description	28

6.4.2	Source	28

6.4.3	Actors	28

6.4.4	Pre-conditions	28

6.4.5	Triggers	28

6.4.6	Normal Flow	29

6.4.7	Alternative Flow	31

6.4.8	Post-conditions	31

6.4.9	High Level Illustration	31

6.4.10	Potential Requirements	31

6.5	Electronic Toll Collection (ETC) service	32

6.5.1	Description	32

6.5.2	Source	32

6.5.3	Actors	32

6.5.4	Pre-conditions	32

6.5.5	Triggers	33

6.5.6	Normal Flow	33

6.5.7	Alternative Flow	34

6.5.8	Post-conditions	34

6.5.9	High Level Illustration	34

6.5.10	Potential Requirements	34

6.6	Taxi Advertisement	35

6.6.1	Description	35

6.6.2	Source	35

6.6.3	Actors	35

6.6.4	Pre-conditions	35

6.6.5	Triggers	35

6.6.6	Normal Flow	35

6.6.7	Alternative Flow	35

6.6.8	Post-conditions	36

6.6.9	High Level Illustration	36

6.6.10	Potential Requirements	36

6.7	Vehicle Data Service	36

6.7.1	Description	36

6.7.2	Source	36

6.7.3	Actors	36

6.7.4	Pre-conditions	37

6.7.5	Triggers	37

6.7.6	Normal Flow	37

6.7.7	Alternative Flow	39

6.7.8	Post-conditions	39

6.7.9	High Level Illustration	40

6.7.10	Potential requirements	40

6.8	Smart Automatic Driving	41

6.8.1	Description	41

6.8.2	Source	41

6.8.3	Actors	41

6.8.4	Pre-conditions	41

6.8.5	Triggers	41

6.8.6	Normal Flow	42

6.8.7	Alternative flow	42

6.8.8	Post-conditions	42

6.8.9	High Level Illustration	42

6.8.10	Potential requirements	42

6.9	Use Case on Vehicle Data Wipe Service	43

6.9.1	Description	43

6.9.2	Source	43

6.9.3	Actors	43

6.9.4	Pre-conditions	43

6.9.5	Triggers	44

6.9.6	Normal Flow	44

6.9.7	Alternative flow	44

6.9.8	Post-conditions	44

6.9.9	High Level Illustration	44

6.9.9.1	Data Request and Response	44

6.9.9.2	Data Request and Response	45

6.9.9.3	Issue of Bigger Data	45

6.9.9.4	Pre-condition of Data Wipe (and Post-condition of Data Request and Data Response)	46

6.9.9.5	Data Wipe	46

6.9.9.6	Data Wipe with Authentication	47

6.9.9.7	Post Condition of Data Wipe	47

6.9.10	Potential requirements	47

6.10	Vehicle Management based on Geo-Fence	48

6.10.1	Description	48

6.10.2	Source	48

6.10.3	Actors	48

6.10.4	Pre-conditions	48

6.10.5	Triggers	48

6.10.6	Normal Flow	49

6.10.7	Alternative flow	49

6.10.8	Post-conditions	49

6.10.9	High Level Illustration	50

6.10.10	Potential requirements	50

6.11	Secure Over-The-Air Firmware Update for Automotive ECUs	50

6.11.1	Description	50

6.11.2	Source	50

6.11.3	Actors	51

6.11.4	Pre-conditions	51

6.11.5	Triggers	52

6.11.6	Normal Flow	52

6.11.7	Alternative flow	53

6.11.8	Post-conditions	54

6.11.9	High Level Illustration	54

6.11.10	Potential requirements	55

6.12	Car/Bicycle Sharing Services	55

6.12.1	Description	55

6.12.2	Source	55

6.12.3	Actors	56

6.12.4	Pre-conditions	56

6.12.5	Triggers	56

6.12.6	Normal Flow	56

6.12.7	Alternative Flow	60

6.12.8	Post-conditions	60

6.12.9	High Level Illustration	61

6.12.10	Potential Requirements	61

6.13	Smart Parking	61

6.13.1	Description	61

6.13.2	Source	62

6.13.3	Actors	62

6.13.4	Pre-conditions	62

6.13.5	Triggers	62

6.13.6	Normal Flow	62

6.13.7	Alternative Flow	64

6.13.8	Post-conditions	65

6.13.9	High Level Illustration	66

6.13.10	Potential Requirements	66

6.14	Vehicle Broadcasting without Registration	66

6.14.1	Description	66

6.14.2	Source	66

6.14.3	Actors	67

6.14.4	Pre-conditions	67

6.14.5	Triggers	67

6.14.6	Normal Flow	67

6.14.7	Alternative Flow	68

6.14.8	Post-conditions	68

6.14.9	High Level Illustration	68

6.14.10	Potential Requirements	68

6.15	Vehicle location privacy protection	68

6.15.1	Description	68

6.15.2	Source	69

6.15.3	Actors	69

6.15.4	Pre-conditions	69

6.15.5	Triggers	69

6.15.6	Normal Flow	69

6.15.7	Alternative flow	70

6.15.8	Post-conditions	70

6.15.9	High Level Illustration	71

6.15.10	Potential requirements	71

6.16	Vehicle Domain service continuity	71

6.16.1	Description	71

6.16.2	Source	71

6.16.3	Actors	72

6.16.4	Pre-conditions	72

6.16.5	Triggers	72

6.16.6	Normal Flow	72

6.16.7	Alternative flow	73

6.16.8	Post-conditions	74

6.16.9	High Level Illustration	74

6.16.10	Potential requirements	74

6.17	Optimal Speed Recommendation	74

6.17.1	Description	74

6.17.2	Source	74

6.17.3	Actors	75

6.17.4	Pre-conditions	75

6.17.5	Triggers	75

6.17.6	Normal Flow	76

6.17.7	Alternative flow	77

6.17.8	Post-conditions	77

6.17.9	High Level Illustration	77

6.17.10	Potential requirements	77

6.18	Autonomous driving	78

6.18.1	Description	78

6.18.2	Source	78

6.18.3	Actors	78

6.18.4	Pre-conditions	79

6.18.5	Triggers	79

6.18.6	Normal Flow	79

6.18.7	Alternative Flow	79

6.18.8	Post-conditions	79

6.18.9	High Level Illustration	80

6.18.10	Potential Requirements	80

6.19	Accident Notification using Edge/Fog Traffic Monitoring Service	80

6.19.1	Description	80

6.19.2	Source	81

6.19.3	Actors	81

6.19.4	Pre-conditions	81

6.19.5	Triggers	81

6.19.6	Normal Flow	81

6.19.7	Alternative Flow	82

6.19.8	Post-conditions	82

6.19.9	High Level Illustration	83

6.19.10	Potential requirements	83

6.20	High-precision Road Map using Edge/Fog Computing	83

6.20.1	Description	83

6.20.2	Source	84

6.20.3	Actors	84

6.20.4	Pre-conditions	84

6.20.5	Triggers	85

6.20.6	Normal Flow	85

6.20.7	Alternative Flow	86

6.20.8	Post-conditions	86

6.20.9	High Level Illustration	87

6.20.10	Potential requirements	87

6.21	Smart Transportation with Edge/Fog computing	87

6.21.1	Description	87

6.21.2	Source	89

6.21.3	Actors	89

6.21.4	Pre-conditions	89

6.21.5	Triggers	90

6.21.6	Normal Flow	90

6.21.7	Alternative Flow	90

6.21.8	Post-conditions	91

6.21.9	High Level Illustration	91

6.21.10	Potential requirements	92

6.22	Automated Parking	92

6.22.1	Description	92

6.22.2	Source	92

6.22.3	Actors	92

6.22.4	Pre-conditions	93

6.22.5	Triggers	93

6.22.6	Normal Flow	93

6.22.6.1	Example of data published by APS	93

6.22.7	Alternative Flow	94

6.22.8	Post-conditions	94

6.22.9	High Level Illustration	94

6.22.10	Potential Requirements	94

6.23	Platooning	94

6.23.1	Description	94

6.23.2	Source	95

6.23.3	Actors	95

6.23.4	Pre-conditions	96

6.23.5	Triggers	96

6.23.6	Normal Flow	96

6.23.6.1	Normal Flow 1: Finding platoon - Platooning service	97

6.23.6.2	Normal Flow 2: Joining the platoon - Platooning manager	97

6.23.6.3	Normal Flow 3: Driving - Platooning	97

6.23.6.4	Example data for platoon formation advice	97

6.23.6.5	Example data on platoon status	98

6.23.6.6	Example data for platoon navigation	98

6.23.7	Alternative Flow	99

6.23.8	Post-conditions	99

6.23.9	High Level Illustration	99

6.23.10	Potential Requirements	99

6.24	Highway pilot	100

6.24.1	Description	100

6.24.2	Source	101

6.24.3	Actors	101

6.24.4	Pre-conditions	101

6.24.5	Triggers	101

6.24.6	Normal Flow	101

6.24.6.1	Example of data published by vehicle on detected obstacle	102

6.24.7	Alternative Flow	102

6.24.8	Post-conditions	102

6.24.9	High Level Illustration	103

6.24.10	Potential Requirements	103

6.25	Car rebalancing service	103

6.25.1	Description	103

6.25.2	Source	104

6.25.3	Actors	104

6.25.4	Pre-conditions	104

6.25.5	Triggers	104

6.25.6	Normal Flow	104

6.25.6.1	Example of data published by CRS	105

6.25.7	Alternative Flow	105

6.25.8	Post-conditions	105

6.25.9	High Level illustration	105

6.25.10	Potential Requirements	106

6.26	Urban driving	106

6.26.1	Description	106

6.26.2	Source	106

6.26.3	Actors	106

6.26.4	Pre-conditions	107

6.26.5	Triggers	107

6.26.6	Normal Flow	107

6.26.6.1	Example of data published by UDS	107

6.26.7	Alternative Flow	108

6.26.8	Post-conditions	108

6.26.9	High Level Illustration	108

6.26.10	Potential Requirements	109

6.27	Traffic Light Control and Coordination using Reliable Edge/Fog Service	109

6.27.1	Description	109

6.27.2	Source	109

6.27.3	Actors	109

6.27.4	Pre-conditions	110

6.27.5	Triggers	110

6.27.6	Normal Flow	110

6.27.7	Alternative Flow	111

6.27.8	Post-conditions	111

6.27.9	High Level Illustration	111

6.27.10	Potential requirements	111

6.28	Vulnerable Road Users Discovery Service using Edge/Fog Computing	112

6.28.1	Description	112

6.28.2	Source	112

6.28.3	Actors	112

6.28.4	Pre-conditions	112

6.28.5	Triggers	112

6.28.6	Normal Flow	112

6.28.7	Alternative Flow	114

6.28.8	Post-conditions	114

6.28.9	High Level Illustration	114

6.28.10	Potential requirements	115

6.29	Car-to-Home service based on Geo-Fence	115

6.29.1	Description	115

6.29.2	Source	116

6.29.3	Actors	116

6.29.4	Pre-conditions	117

6.29.5	Triggers	117

6.29.6	Normal Flow	117

6.29.7	Alternative Flow	118

6.29.8	Post-conditions	118

6.29.9	High Level Illustration	118

6.29.10	Potential Requirements	119

6.30	Vehicle Remote Control using External Platform	119

6.30.1	Description	119

6.30.2	Source	120

6.30.3	Actors	120

6.30.4	Pre-conditions	121

6.30.5	Triggers	121

6.30.6	Normal Flow	121

6.30.7	Alternative Flow	122

6.30.8	Post-conditions	122

6.30.9	High Level Illustration	122

6.30.10	Potential requirements	122

6.31	Vehicular Data Service with a vehicular data model and Edge/Fog Computing	123

6.31.1	Description	123

6.31.1.1	Sensor Data Ingestion Interface (SDII)	123

6.31.2	Source	124

6.31.3	Actors	124

6.31.4	Pre-conditions	124

6.31.5	Triggers	124

6.31.6	Normal Flow	124

6.31.7	Alternative Flow	126

6.31.8	Post-conditions	126

6.31.9	High Level Illustration	127

6.31.10	Potential requirements	127

6.32	Protocol Selection in Gateway for Vehicle	127

6.32.1	Description	127

6.32.2	Source	128

6.32.3	Actors	128

6.32.4	Pre-conditions	129

6.32.5	Triggers	129

6.32.6	Normal Flow	129

6.32.7	Alternative Flow	130

6.32.8	Post-conditions	130

6.32.9	High Level Illustration	130

6.32.10	Potential requirements	130

6.33	Software-Defined Radios Application for Vehicle	130

6.33.1	Description	130

6.33.2	Source	132

6.33.3	Actors	132

6.33.4	Pre-conditions	132

6.33.5	Triggers	132

6.33.6	Normal Flow	133

6.33.7	Alternative Flow	133

6.33.8	Post-conditions	134

6.33.9	High Level Illustration	134

6.33.10	Potential requirements	134

6.34	Dashcam Application	134

6.34.1	Description	134

6.34.2	Source	134

6.34.3	Actors	134

6.34.4	Pre-conditions	135

6.34.5	Triggers	135

6.34.6	Normal Flow	135

6.34.7	Alternative flow	136

6.34.8	Post-conditions	136

6.34.9	High Level Illustration	136

6.34.10	Potential requirements	136

6.35	Volatile Data Management for Instant IoT Services	136

6.35.1	Description	136

6.35.2	Source	137

6.35.3	Actors	137

6.35.4	Pre-conditions	137

6.35.5	Triggers	137

6.35.6	Normal Flow	137

6.35.7	Alternative Flow	138

6.35.8	Post-conditions	138

6.35.9	High Level Illustration	138

6.35.10	Potential requirements	139

6.36	Offloading Service Continuity between Edge/Fog Nodes	139

6.36.1	Description	139

6.36.2	Source	139

6.36.3	Actors	139

6.36.4	Pre-conditions	139

6.36.5	Triggers	139

6.36.6	Normal Flow	139

6.36.7	Alternative Flow	140

6.36.8	Post-conditions	140

6.36.9	High Level Illustration	140

6.36.10	Potential requirements	141

6.37	Handling Data Protection-related Regulations	141

6.37.1	Description	141

6.37.2	Source	142

6.37.3	Actors	142

6.37.4	Pre-conditions	142

6.37.5	Triggers	142

6.37.6	Normal Flow	142

6.37.7	Alternative Flow	143

6.37.8	Post-conditions	143

6.37.9	High Level Illustration	144

6.37.10	Potential requirements	144

6.38	Vehicle Idling Alarming Service	144

6.38.1	Description	144

6.38.2	Source	145

6.38.3	Actors	145

6.38.4	Pre-conditions	145

6.38.5	Triggers	145

6.38.6	Normal Flow	145

6.38.7	Alternative Flow	146

6.38.8	Post-conditions	146

6.38.9	High Level Illustration	146

6.38.10	Potential Requirements	146

7	Overview of Potential Requirements	147

8	High Level Architecture	157

8.1	Introduction	157

8.2	Vehicular Architecture Type 1	158

8.3	Vehicular Architecture Type 2	159

8.4	Vehicular Architecture Type 3	160

8.5	Vehicular Architecture Type 4	160

9	Key Issues for Enablement of Vehicular Domain	161

9.1	Key Issues 1: Location	161

9.1.1	Accuracy of geographic location	161

9.1.2	Latency	164

9.2	Key Issue 2: Maintaining AE contact information	164

9.3	Key Issue 3: Registration management	165

9.4	Key Issue 4: Security	165

9.4.1	Secure communication	165

9.4.2	Lightweight Encryption	166

9.4.3	Security for credential	166

9.5	Key Issue 5: Cross-Resource Subscription	168

9.6	Key Issue 6: Subscription Aggregation	168

9.7	Key Issue 7: Time synchronization	169

9.8	Key Issue 8: V2V network interface switching management	169

9.9	Key Issue 9: Simultaneous registration with multiple CSEs	170

10	Potential Solutions for the Key Issues	171

10.1	Solution A: Maintaining AE contact information - IN-CSE Notifies all CSEs	171

10.1.1	Solution Description	171

10.1.2	Solution Applicability	172

10.2	Solution B: Maintaining AE contact information - IN-CSE Notifies only impacted CSEs	172

10.2.1	Solution Description	172

10.2.2	Solution Applicability	173

10.2.3	Solution Details	173

10.2.3.1	Impacted Resources and Attributes	173

10.2.3.1.1	Overview	173

10.2.3.1.2	Modified <AE> resource	173

10.2.3.1.3	Modified <AEAnnc> resource	173

10.2.3.1.4	New Resource Type: AEContactList	174

10.2.3.1.5	New Resource Type: AEContactListPerCSE	174

10.2.3.2	Impacted Information Flows	175

10.2.3.2.1	Overview	175

10.2.3.2.2	Procedure for Managing Change in AE Registration Point	176

10.2.3.2.2.1	Procedure at IN-CSE	176

10.2.3.2.2.2	Procedure at any CSE	176

10.3	Solution C: Cross-Resource Subscription	176

10.3.1	Solution Description	176

10.3.2	Solution Applicability	177

10.3.3	New Resources and Procedures	178

10.3.3.1	Introduction	178

10.3.3.2	New <crossResourceSubscription> Resource to Enable Cross-Resource Subscription Functionality	178

10.3.3.3	Procedure for Creating a Cross-Resource Subscription	179

10.3.3.4	Procedure for Generating Cross Resource Notification	180

10.4	Solution D: Subscription Aggregation	180

10.4.1	Solution Description	180

10.4.2	Solution Applicability	181

10.5	Solution E: Secure Channel Establishment	181

10.5.1	External communication and inter-vehicle communication	181

10.5.1.1	Solution Description	181

10.5.2	Intra-vehicle communication	182

10.5.2.1	Solution Description	182

10.5.3	Solution Applicability	182

10.6	Solution F: Hardware Secure Element	182

10.6.1	Solution Description	182

10.6.2	Solution Applicability	183

10.7	Solution G: Cross-Resource Subscription #2	183

10.7.1	Solution Description	183

10.7.2	Solution Applicability	183

10.7.3	New Resources and Procedures	183

10.7.3.1	Introduction	183

10.7.3.2	New subscriptionAssociation resource type	184

10.7.3.3	Extension to subscription resource type	184

10.7.3.4	Procedure to create subscription association	184

10.7.3.5	Procedure to manage subscription association	185

10.7.3.6	Procedure to generate notifications of cross-resource subscription	185

10.8	Solution H: Time Synchronization using beacons	185

10.8.1	Solution Description	185

10.8.2	Solution Applicability	185

10.8.3	Solution Details	185

10.9	Solution I: Time Synchronization using request and response parameters	187

10.9.1	Solution Description	187

10.9.2	Solution Applicability	187

10.9.3	Solution Details	187

10.10	Solution J: Network Communication Management	188

10.10.1	Network Communication Management function overview	188

10.10.2	Detailed Procedure	189

10.10.2.1	Network communication management procedure for a single Node	189

10.10.2.1.1	Network Communication Planning Procedure	189

10.10.2.1.2	Network communication mode switching procedure	192

10.10.2.2	Network communication management procedure for a Group	192

10.10.2.2.1	Network communication planning procedure	192

10.10.2.2.2	V2X network communication switching procedure	193

10.10.2.2.2.1	Option 1: The lead vehicle sends switching message to each member to update the current communication mode	193

10.10.2.2.2.2	Option 2: IN-CSE updates the current communication mode for each member	195

10.10.2.2.2.3	Option 3: Each member updates the current communication mode	196

10.10.2.2.3	V2X network communication switching procedure	197

10.10.3	Solution Applicability	197

10.10.4	New Resources and Procedures	198

10.10.4.1	Introduction	198

10.10.4.2	Modified <node> resource	198

10.10.4.3	New <commModePlan> resource type	198

10.10.4.4	Modified <group> resource	199

10.11	Solution K: Time Synchronization using currentTime	199

10.11.1	Solution Description	199

10.11.2	Solution Applicability	199

10.11.3	Solution Details	199

10.12	Solution L: Cross Resource Subscription Management	200

10.12.1	Solution Description	200

10.12.2	Solution Applicability	200

10.12.3	Solution Details	200

10.12.4	Overview	200

10.12.5	Modified attribute	201

10.12.5.1	New attribute	201

10.13	Solution M: Time Compensation	202

10.13.1	Solution Description	202

10.13.2	Solution Applicability	202

10.13.3	Solution Details	202

Annex A: oneM2M data model for vehicular domain	205

A.1	AUTOPILOT	205

A.2	AUTOPILOT and use of IoT	205

A.3	AUTOPILOT, oneM2M and other IoT platforms	205

A.4	oneM2M data model for vehicular domain	206

A.5	Proposal for oneM2M data model for vehicular domain	206

History	207

- Scope

The present document examines how the current oneM2M System can be used in the Vehicular Domain and includes a study of advanced features which the future oneM2M release(s) could support for this vertical domain. The present document also analyses use cases and the potential requirements pertaining to the use cases with regard to vehicular domain.

- References

  - Normative references

# Normative references are not applicable in the present document.

  - Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules.

NOTE:	Available at http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf.

[i.2]	oneM2M TS-0002: "Requirements".

[i.3]	oneM2M TS-0001: "Functional Architecture"

[i.4]	oneM2M TS-0011: "Common Terminology".

[i.5]	Recommendation ITU-T X.1362: "Simple encryption procedure for Internet of Things (IoT) environments".

[i.6]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[i.7]	ISO/IEC 29192:2012 "Lightweight cryptography".

[i.8]	PRESERVE: "V2X Security Architecture v2".

[i.9]	3GPP TR 33.885: "Study on security aspects for LTE support of Vehicle-to-Everything (V2X) services".

[i.10]	3GPP TS 33.185: "Security aspect for LTE support of Vehicle-to-Everything (V2X) services".

[i.11]	EVITA: "Secure on-board architecture specification".

[i.12]	EVITA: "Secure on-board protocols specification".

[i.13]	Trusted Computing Group: "TCG TPM 2.0 Automotive Thin Profile".

[i.14]	oneM2M TS-0003: "Security Solutions".

[i.15]	GSMA: "SGP.01 - Embedded SIM Remote Provisioning Architecture".

[i.16]	oneM2M TS-0004: "Service Layer Core Protocol Specification".

[i.17]	W. Chen, Telcordia Technologies: “Vehicular Communications and Networks: Architectures, Protocols, Operation and Deployment”

[i.18]	AUTOPILOT: http://autopilot-project.eu/

[i.19]	SAREF: The Smart Appliances REFerence (SAREF) ontology.

NOTE:	Available at http://ontology.tno.nl/saref/.

[i.20]	HERE: Vehicle Sensor Data Cloud Ingestion Interface Specification(v2.0.2)

[i.21]	ETSI TR 102 863 : “Intelligent Transport Systems (ITS); Vehicular Communications; Basic Set of Applications; Local Dynamic Map (LDM); Rationale for and guidance on standardization”

[i.22]	5GAA White paper - Toward fully connected vehicles: Edge computing for advanced automotive communications, Dec. 2017

NOTE:	Available at http://5gaa.org/news/toward-fully-connected-vehicles-edge-computing-for-advanced-automotive-communications/

[i.23]		HERE: Sensor Data Ingestion Interface (SDII) Data Specification

NOTE:　Available at https://developer.here.com/olp/documentation/sdii-data-spec/topics/message-components.html

[i.24]        ETSI TS 102 894-2 : “Intelligent Transport Systems (ITS); Users and applications requirements; Part 2: Applications and facilities layer common data dictionary”

- Definitions, symbols and abbreviations

  - Definitions

For the purposes of the present document, the terms and definitions given in oneM2M TS-0011 [i.4] apply.

  - Symbols

For the purposes of the present document, the symbols given in oneM2M TS-0011 [i.4] apply.

  - Abbreviations

For the purposes of the present document, the abbreviations given in oneM2M TS-0011 [i.4] and the following apply:

ADApp	Autonomous Driving Application

ADS	Automated Driving System

ARIB	Association of Radio Industries and Businesses

ATIS	Alliance for Telecommunications Industry Solutions

CCSA	China Communications Standards Association

DDT	Dynamic Driving Task

DSRC	Dedicated Short Range Communications

DTG 	Digital Tachograph

ETC	Electronic Toll Collection

ETSI	European Telecommunications Standards Institute

GPS	Global Positioning System

HSM	Hardware Security Modules

ITS	Intelligent Transportation System

LIDAR	Light Detection and Ranging, Laser Imaging Detection and Ranging

OBU	On Board Unit

ODD	Operational Design Domain

OEDR	Object and Event Detection and Response

RSU	Road Side Unit

TIA	Telecommunications Industry Association,

TPM	Trusted Platform Module

TSDSI	Telecommunications Standards Development Society

TTA	Telecommunications Technology Association

TTC	Telecommunication Technology Committee

V2I	Vehicle to Infrastructure

V2V	Vehicle to Vehicle

V2X	Vehicle to Everything

- Conventions

The key words "Shall", "Shall not", "May", "Need not", "Should", "Should not" in the present document are to be interpreted as described in the oneM2M Drafting Rules [i.1].

- Introduction to Vehicular Domain

  - Vehicular Domain Overview

Existing ITS (Intelligent Transport Systems) services in automotive industry have been provided through the architecture which is composed of connected vehicles to various infrastructures such as the ETC (Electronic Toll Collection) service, VICS (Vehicle Information and Communication System) or telematics service centers. Those systems have been growing by improving mobility convenience for drivers. However, the infrastructure systems are required to change and become more collaborative, in order to meet rising social demands such as energy saving, traffic congestion resolution and fatal accidents avoidance.

With rising computational capabilities of participants in traffic - pedestrians (with their smartphones), vehicles that have communication and computing capabilities, smart traffic lights, smart streets and crossings, etc., corresponding vehicular infrastructure systems are becoming more collaborative, capable of exchanging information with other participants in traffic, with the goal of  meeting rising social demands such as energy saving, managing traffic congestion and minimizing and avoiding fatal accidents.

In order to make optimal decisions in traffic, participants, smart traffic lights, vehicles, smart roads, etc. do so by creating their own 'world model' [i.17]. It is vehicle's representation of the state of the external environment (which other participants are, what their state of movement is for given moment). Based on world model, vehicle can make optimal decisions on its own actions. If we look at an example of vehicle, to create world model, vehicle relies on measured values it gets from its own sensors (like ABS sensor, radar, LIDAR, brake switch, tire pressure sensors, accelerator switch, info on vehicle speed from motor management, etc.). By processing data from these sensors, vehicle creates awareness on its own state, and can therefore manage movement of vehicle in better way.

When adding V2V/V2I communication with other vehicles, it is possible to have even better, more complete world model. Immediate environment of vehicle can be better mapped. For example, if vehicle immediately in front brakes, that vehicle's brake switch, and thus motor management is immediately aware of braking process. It takes some time to driver of vehicle behind it to observe that braking lights have turned ON, and it takes further time before driver of vehicle processes that information and takes corrective action (hitting own brakes). By allowing vehicles to communicate directly and exchange part of their own world models that are relevant for other participants in traffic, each vehicle is therefore able to get quickly more complete world model, and allow for vehicles to take corrective measures immediately.

The same reasoning applies to other 'smart' elements in traffic like traffic lights, roads, road crossings, merging lanes on highways, etc.

  - Technology Trends in Vehicular Domain

In order to develop vehicular domain architecture as ITS platform, many worldwide organizations are discussing technology and standardization related to this field.

ISO (International Organization for Standardization) is an international standard-setting body which standardized technologies for ITS systems through the work of TC (Technical Committee)  204 (ITS) and 22 (Road Vehicles). In recent years,  ISO has focused on CITS (Cooperative ITS) using V2V/V2I communication for some new services such as automated driving and Urban ITS which improves mobility in urban area. Furthermore, it has made an effort to develop gateways between in-vehicle area network and nomadic devices like smartphones.

ITU-T (International Telecommunication Union Telecommunication Standardization Sector) is one of the three sectors of the ITU and it coordinates standards for telecommunications. In SG16 (multimedia) of  ITU-T, VGP (Vehicle Gateway Platform) has been standardized in terms of telecommunication. In particular, reference architecture and functional architecture based on supposed applications and interface protocol to vehicles or ICT devices are discussed.

ETSI (European Telecommunications Standards Institute) is a standardization organization in the telecommunications industry (equipment makers and network operators) in Europe. In TC (Technical Committee) ITS, standards, specifications and other deliverables to support the development and implementation of ITS Service provision across the network have been developed. The scope includes communication media, and associated physical layer, transport layer, network layer, security, lawful intercept and the provision of generic web services.

The W3C (World Wide Web Consortium) is the main international standards organization for the World Wide Web. In 2013, it has established the Automotive and Web Platform Business Group and prepared a draft of Vehicle Information Access and Vehicle Data Spec. Currently these topics are in discussion to standardize in the Automotive Working Group. Responding to the growing needs of web services for a connected car, the W3C focuses on interfaces for application vendors to access vehicle data using a standard and secure method.

In reference to V2V/V2I communication technologies which support M2M services in vehicular domain, DSRC (Dedicated Short Range Communications) has been discussed in ITU-R (International Telecommunication Union Radio-communications Sector) and IEEE (The Institute of Electrical and Electronics Engineers). Moreover cellular communication for vehicular domain is also discussed in 3GPP (Third Generation Partnership Project). These wireless communication technologies are expected to resolve some problems such as cost, coverage, latency and power consumption, while a unified communication standard is expected to accelerate the growth of M2M services in business.

Table 5.2-1 Vehicular Domain Standards

| No | Organization | Sector | Focus point | Major topics |
| --- | --- | --- | --- | --- |
| 1 | ISO | TC204, TC22 | ITS services | Cooperative System, In-vehicular gateway |
| 2 | ITU-T | SG16 | Telecommunication | Vehicle Gateway Platform, Communication protocol |
| 3 | ETSI | TC ITS | Network system, Radio Technique | Cooperative ITS, DSRC |
| 4 | W3C | Automotive WG | Web services | Web-API for vehicles |
| 5 | ITU-R | SG5/WP5A | Wireless communication | V2V/V2I communication (DSRC) |
| 6 | IEEE | 802.11p + P1609 | Wireless communication | V2V/V2I communication (DSRC) |
| 7 | 3GPP | - | Wireless communication | V2V/V2I communication (Cellular) |

  - The focus of oneM2M in Vehicular Domain

The changing trends are enabled by improvements of IT technologies such as V2V/V2I communication, cloud systems or OSS (Open Source Software). The M2M architecture for vehicle domain described in this TR accelerates wide collaboration between not only auto manufacturers or suppliers but also between telecommunication providers or government, which results in improvement of safety, comfort and eco-friendliness. Figure 5.3-1 is an example of reference architecture.

Figure 5.3-1: Vehicular Domain Architecture

In this architecture, vehicles can connect to various services via M2M systems. Gateways which collect sensing data could be located both inside (left of figure) and outside of the vehicle (center of figure). A vehicle may act as a gateway and collect also data generated by sensors located outside of a vehicle (right of figure). Furthermore vehicles may connect to each other and a vehicle may connect to services via other vehicles.

This vehicular domain architecture provides the basic functions for analysing huge amounts of data collected from large number of vehicles which makes it possible to create future services as a result of collaboration with other industries.

Furthermore, multiple security levels according to features of collected data make it possible to provide critical control and management for vehicles such as remote maintenance or automated driving control via communication networks.

In the context of progress achieved through various standardization efforts, oneM2M is continuing to develop a common platform which can be used for information transport functions between vehicles and backend servers over a variety of transport technologies (e.g. cellular or DSRC) to satisfy vehicular domain requirements. The following capabilities are of key importance to the vehicular domain.

Data management

Since vehicles may move fast, the operation and management in this domain will be dynamic, complex and difficult. For example, if the nodes located on roadside act as gateway, status of a connection between device and gateway is varying frequently. Therefore, management of data (including sensing and device information, data streams) as well as filtering and pre-processing functions (at devices or gateways) are required in order to provide efficient analysis services in the complicated environments.

Communication management

M2M services in vehicular domain need to deal with many and widespread devices. On the other hand, the target devices of the operation are depending on the services. For instance, it is expected that the content is distributed to the vehicles in a specific geographical area. Therefore, the communication management function is needed to select appropriate communication protocols and modes (e.g. unicast, multicast or broadcast), according to features of data, for efficient utilization of network resources.

Location

Geographical location information is important in vehicular services. To improve safety, up-to-date location information is important for collision avoidance. Therefore, accurate location providing/managing functions are needed in this vehicular domain. Furthermore, the inherent node mobility in the vehicular domain creates greater variability in the services available along the way. Mobility management functionality is required to allow maintaining the vehicular services as the node moves, and to provide means for selection and migration of services between nodes.

Security

To mitigate the life-threatening risk resulting from malicious control and management of vehicles via network, components of vehicular services should support strict authentication functionality. Furthermore, integrity and encryption functionalities for communication are required, with consideration for on system scalability and resource constraints at the devices.

  - Levels of Driving Automation

As mentioned in previous clauses, vehicles can exchange data on its own state, as well as receive data from surroundings (infrastructure - traffic lights, roads, other participants - vehicles, pedestrians, cyclists, etc.) and its state.

For vehicles exchanging data with its surroundings, we can distinguish number of levels of connectivity.

One is connected car - meaning car is connected to the Internet, typically for use of mobile network services. Examples of this are use of mobile network to Wi-Fi AP gateway, enabling passengers within vehicle to access internet via Wi-Fi. Other use is for collecting data from GPS unit within car to send information on current vehicle's location and speed. Data collected in this way is typically used for road management and routing advice (detecting congestion in roads).

Next level of connectivity is cooperative car - where car is communicating with other vehicles, either directly or indirectly (via RSU, or some other infrastructure element), with the goal of creating LDM (Local Dynamic Map) which contains all vehicles in vicinity and their current state (speed, acceleration/deceleration, changing lanes, …). Further, each vehicle is announcing (via local broadcast, within range of used communication technology) its own state. Based on received information from other vehicles, and own sensors, vehicle can make decision on actions it needs to take. Typically, cooperative driving is providing driver with additional information on state of traffic in immediate vicinity, and vehicle can also take some of actions (keeping distance, braking) itself.

Next stage is automated driving, which refers to the capability of the vehicle to drive from one location to another, without intervention from humans, and in a safe way, without incurring damage to surroundings (pedestrians, buildings, other vehicles) and to its (vehicle's) passengers. Here vehicle has all information like in cooperative driving (state of other vehicles in vicinity), but gets information from other participants in traffic as well - infrastructure (roads, cameras,  traffic lights, etc.) but also pedestrians, cyclists, etc.

Different levels of automation in driving are shown in Table 5.4-1. From top to bottom we can see increasing levels of automation, with ' Full Driving Automation ' being autonomous driving.

| SAE Level | Name | Definition | DDT |  | DDT fallback | ODD |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Sustained lateral and longitudinal vehicle motion control | OEDR(Object and Event Detection and Response) |  |  |
| Driver performs part or all of the DDT |  |  |  |  |  |  |
| 0 | No Driving Automation | The performance by the driver of the entire DDT, even when enhanced by active safety systems. | Driver | Driver | Driver | N/A |
| 1 | Driver Assistance | The sustained and ODD-specific execution by a driving automation system of either the lateral or the longitudinal vehicle motion control subtask of the DDT (but not both simultaneously) with the expectation that the driver performs the remainder of the DDT. | Driver and System | Driver | Driver | Limited |
| 2 | Partial Driving Automation | The sustained and ODD-specific execution by a driving automation system of both the lateral and longitudinal vehicle motion control subtasks of the DDT with the expectation that the driver completes the OEDR subtask and supervises the driving automation system. | System | Driver | Driver | Limited |
| ADS (“System”) performs the entire DDT (while engaged) |  |  |  |  |  |  |
| 3 | Conditional Driving Automation | The sustained and ODD-specific performance by an ADS of the entire DDT with the expectation that the DDT fallback-ready user is receptive to ADS-issued requests to intervene, as well as to DDT performance-relevant system failures in other vehicle systems, and will respond appropriately. | System | System | Fallback-ready user (becomes the driver during fallback ) | Limited |
| 4 | High Driving Automation | The sustained and ODD-specific performance by an ADS of the entire DDT and DDT fallback without any expectation that a user will respond to a request to intervene. | System | System | System | Limited |
| 5 | Full Driving Automation | The sustained and unconditional (i.e., not ODD-specific) performance by an ADS of the entire DDT and DDT fallback without any expectation that a user will respond to a request to intervene. | System | System | System | Unlimited |

Table 5.4-1 Summary of levels of driving automation

DDT (Dynamic Driving Task): all the real-time operational and tactical functions required to operate a vehicle in on-road traffic, excluding the strategic functions such as trip scheduling and selection of destinations and waypoints, including without limitation: Motion control via steering, acceleration and deceleration; Monitoring the driving environment via object and event detection, recognition, classification and response execution, etc.

DDT Fallback: given the circumstances that the occurrence of a DDT performance-relevant system failure or upon operational design domain(ODD) exit, the response by the user to perform the DDT or the response to achieve a minimal risk condition by the user or an ADS.

Using IoT for vehicular domain, vehicles will be able to go beyond cooperative driving, and move into automated driving, levels 3-5 in Table 5.4-1.

Rest of the present document will refer to this figure, when applicable.

- Vehicular Domain Use Cases

  - Vehicle Diagnostic & Maintenance Report

    - Description

The Vehicle Service Center wants to help the vehicle owner to be aware of the status of the vehicle and remind them to maintain the vehicle in a timely manner to avoid any damages.

Hence the Vehicle Service Center needs to obtain and analyse data from the vehicle periodically. Based on the analysis result, it will notify to the vehicle owner showing what's going on with the vehicle, in simple language and images together with some maintenance suggestions.

    - Source

oneM2M-REQ-2012-0067R03 Vehicle Stolen and Vehicle Diagnostics.

    - Actors

Vehicle Owner:

By reading the Vehicle Diagnostic & Maintenance Report sent from the Vehicle Service Center, the vehicle owner would decide whether to maintain his/her vehicle.

Vehicle Service Center:

It operates a service platform for diagnostics and maintenance of vehicles, obtains and analyses the diagnostics data from the vehicle. It will also send vehicle Diagnostic & Maintenance Report in e-mail together with maintenance suggestions to the vehicle owner.

Mobile Communication Network Operator:

As the transmission medium, it supports the network services between Vehicle Service Center and Vehicle for the information transmission.

M2M Device:

It is embedded in a vehicle, which is used to send information to Vehicle Service Center and implement diagnostics function from Vehicle Service Center.

    - Pre-conditions

The vehicle supports the diagnostics pre-configured to report the diagnostics data collected from sensors within the vehicle periodically.

The vehicle is already ignited.

    - Triggers

None.

    - Normal Flow

Figure 6.1.6-1: Vehicle Diagnostics Normal Flow

- The vehicle collects the diagnostics data from sensors within the vehicle and sends it to the Vehicle Service Center. The diagnostics data includes information from Engine and Transmission System, Stability Control System, Air Bag System, Emission System, Antilock Brake System and so on. The information includes tyre pressure, odometer data, life of engine oil, engine and gear-box status, antilock braking system status, etc.

The Vehicle Service Center sends the diagnostics data to the "Vehicle Detection M2M Application". This M2M application receives and analyses the diagnostics data.

The "Vehicle Detection M2M Application" finds that the Brake pads need to be replaced. It queries the maintenance services provided by "Vehicle Resolution M2M Application" and gets the information of the company who can provide the components.

The "Vehicle Detection M2M Application" finally sends the Diagnostic & Maintenance Report to the vehicle owner together with the suggested component providers either by email or alert message displayed in the vehicle terminal.

The vehicle owner will decide whether to maintain his/her vehicle based on the Diagnostic & Maintenance Report.

    - Alternative Flow

None.

    - Post-conditions

For normal flow, the vehicle owner maintains his/her vehicle according to the Diagnostic & Maintenance report in time.

    - High Level Illustration

Figure 6.1.9-1 Vehicle diagnostics Normal Flow

    - Potential Requirements

- The M2M System shall enable the M2M Devices to exchange M2M application to diagnostic data periodically with the M2M Application in the network domain.

The M2M System shall enable the M2M Application to configure the notification interval in the M2M Devices.

The M2M system shall support a mechanism to describe the syntax and semantics format of the M2M application diagnostics data exchanged between the M2M Devices and the M2M Application in the network domain.

  - Remote Maintenance Services

    - Description

This use case introduces a remote maintenance service for the automobiles (cars).

Because integrity of the cars is a matter of human life, the remote maintenance service of the car (treated as M2M Gateway in this use case) should be strongly secured.

Therefore, the integrity measurements both before and after the remote maintenance operation should also be securely performed.

One of the methods to endorse the measurement process might be guaranteed by HSM (Hardware Security Modules) in the M2M Gateway. This method provides a higher reliability level than that by a software emulator, the decision on the level of security is based on the information sent to the center. In the HSM method, the integrity measurement report can be made through an internal the mechanism in the HSM and put in the electronic signature using the key in the HSM.

This use case is derived from the automobiles, but similar cases of remote maintenance services could be considered with Medical equipment, Household applications, financial transaction terminals and Industrial control and machinery.

    - Source

oneM2M-REQ-2013-0188R06 Use Case Remote Maintenance.

    - Actors

Relevant to the name in the figure in clause 6.2.9, High Level Illustration:

Car: the machine works as a M2M Gateway in which M2M Device(s) is implemented as the parts of it.

Center: the M2M Platform which provides remote maintenance.

The Hardware Security Module (HSM): a module in the M2M Gateway (e.g. Trusted Platform Module) that helps determining the level of security functions to endorse the integrity measurement process and holds the electronic signature key.

A white list: data base which is accessed by the center may be used for verifying the integrity measurement report from the M2M Gateway (car), using a secure communication protocol e.g. Trusted Network Connect TNC protocol.

Support software: installable software module to check the integrity of the Car assisted by TPM or the emulator and to support the newly implemented M2M Device(s) (i.e. sensor(s)).

    - Pre-conditions

Center recognizes the software which is installed in the Car that shall be updated.

    - Triggers

None.

    - Normal Flow

- Mutual authentication between the Car (M2M Gateway) and the Center (M2M Platform) is performed.

Center requests the Car to report the integrity check on that Car.

Support software which is installed in the Car runs integrity check of the Car assisted by TPM or the emulator.

Generated integrity status/configuration information report is endorsed by the hardware key which is protected by TPM. This report may contain a detection of the newly implemented sensor(s) (M2M Device(s)).

Support software sends the report based on TNC (Trusted Network Connect, which is application level secure communication protocol) to the Center.

Center verifies the report securely based on the White list which is based outside the M2M network.

Center determines whether the Car contains the software which shall be updated.

Center selects corresponding software modules.

Center delivers the support software module to the Car.

The support software is applied at the Car.

The applied result endorsed by the device key (actual process is done by TPM or the emulator) is reported to the Center.

Center side confirms the completion of delivery/embedding.

Center side stores the sequence of operations log as certifiable evidence for indemnity.

    - Alternative Flow

None.

    - Post-conditions

Newly installed software/sensor(s) is correctly identified as authorized part(s) on the Car, and working correctly with installed support software. The Car's integrity status/configuration information data which is endorsed by the hardware key which is protected by TPM or the emulator is sent to the Center side.

    - High Level Illustration

Figure 6.2.9-1: Remote Maintenance Flow

Figure 6.2.9-2: Remote Maintenance High Level Illustration

    - Potential Requirements

- The M2M service shall be able to provide the mechanism for authorization for integrity-checking and installing processes of software/hardware/firmware component(s) on M2M Device(s) ([i.2], SER-013).

The M2M system shall be able to support authentication using device key on the integrity check for M2M Device(s).

The M2M Device shall be able to support HSM (Hardware Security Module) to protect its integrity depending on the security level requirement.

  - Traffic Accident Information Collection

    - Description

The Intelligent Transportation System (ITS) is mainly used for avoiding collision of vehicles. If doing some extension an ITS can also be used for other purposes such as electronic payment of road tolls, traffic information collection and broadcast, local service advertisements, etc.

It is for sure that the ITS will save a lot of lives, but some traffic accidents will occur any way. So we still need rescue teams to go to the accident sites to help the victims and police to ease the traffic jam caused by the accident. A rescue team can make a more proper rescue plan if they are able to see the scene of accident. Similarly police can make a better traffic control plan if they are able to get an overview of traffic situation near the accident site.

This use case will show how the M2M technologies can help people to timely access to the detailed information of a traffic accident.

    - Source

oneM2M-REQ-2013-0264R05 Use Case Traffic Accident Information Collection.

    - Actors

M2M Platform: It stores M2M data and runs M2M applications. It provides various M2M services to M2M service subscribers.

ITS Center: It is responsible for managing ITS on M2M Platform. It decides what service is provided to an ITS service subscriber.

Police Station: It is a subscriber of ITS service on M2M platform and responsible for controlling the traffic.

Rescue Center: It is a subscriber of ITS service on M2M platform and responsible for carrying out rescue missions.

ITS-Station (ITS-S): It is a kind of M2M Device installed in vehicles. It broadcast its travel status in a fixed interval or upon specific events in order to inform other ITS-S where it is. The ITS-S is equipped with a digital camera used for taking pictures according to the command given by a driver, ITS center or ITS-S itself. The ITS-S is able to communicate with M2M Platform through wireless network or a RSU using DSRC.

Road Side Unit (RSU): It is a kind of M2M Gateway installed at roadside. The RSU is able to communicate with ITS-S using DSRC and communicate with M2M Platform through wired or wireless network.

    - Pre-conditions

The ITS-Ss are equipped with a digital camera.

The ITS-Ss nearby the accident site are able to connect to M2M platform through either the wireless network or a RSU.

Police Station and Rescue Center are the subscribers of ITS services.

    - Triggers

There are two ways to start an accident reporting process. One is the ITS-S involved in an accident detects the crash and then starts an accident reporting process automatically; the other is a driver in a passing by vehicle manually starts an accident reporting process through giving a command to the ITS-S in his vehicle. Alternately, RSUs (e.g. a RSU involved in collision avoidance at an intersection) may be able to detect an accident and start the reporting process automatically, or they may be remotely requested to trigger such process. This use case only considers the case of a vehicle involved in an accident independently of the ITS infrastructure.

    - Normal Flow

- The ITS-S in the vehicle that is directly involved in an accident detects a crash has happened, and then starts an accident reporting process automatically.

An accident reporting process may also be started manually. For example, a driver of a vehicle that is passing by the accident site stops and then manually starts an accident reporting process through giving a command to the ITS-S in his vehicle.

The ITS-S first takes some pictures with its digital camera, and then uses these pictures together with current time and geographical coordinates to generate an accident report. This report shall be signed by the ITS-S.

The ITS-S tries to connect to M2M Platform and then sends the accident report to the M2M Platform. (step 1 in Figure 6.3.9-1).

There are two ways for an ITS-S to connect to the M2M Platform. One is through wireless network; the other is through a nearby RSU using DSRC.

The M2M Platform receives and verifies the accident report, and then does some necessary analysis. The analysis result will be pushed to the subscribers, i.e. the Police Station and the Rescue Center.

The subscribers receive, verify and parse the information coming from M2M platform, and then do some necessary analysis. Based on different situation the subscribers may ask the M2M Platform to provide further information.

In this scenario the Police Station asks the M2M Platform to provide an overview of the traffic situation near the accident site, and the Rescue Center asks the M2M Platform to provide more visual information about the accident. These service requirements are submitted to the M2M Platform.

The M2M Platform receives and verifies the service requirements from Police Station and Rescue Center, and then sends data collection commands to the ITS-S that originally sends the accident report. (step 2 in Figure 6.3.9-1).

The command generated for Police Station requires the ITS-Ss near the accident site to report their travel status.

The command generated for Rescue Center requires the ITS-Ss around the accident site to provide pictures.

The ITS-S that originally sent the accident report receives the commands sent from the M2M Platform. It verifies and parses the commands, and then broadcasts the commands that should be broadcasted. (step 3 in Figure 6.3.9-1).

In this scenario the broadcasted commands are generated by the M2M platform for Police Station and Rescue Center respectively.

The ITS-Ss nearby the accident site receive, verify, parse and execute received commands, i.e. take pictures, get current travel status, generate reports, sign the reports and upload signed reports to M2M Platform. These reports could be sent anonymously. (step 4 in Figure 6.3.9-1).

Some commands need to be rebroadcasted within a predetermined area and predetermined period of time. (step 5 in Figure 6.3.9-1).

In this scenario the command generated for the Police Station needs to be rebroadcasted. The ITS-Ss receiving this command will only report their travel status. (step 6 in Figure 6.3.9-1).

M2M Platform accumulates and verifies the reports uploaded by the ITS-Ss, and then generates a report containing visual information about the accident scene for the Rescue Center and a report about traffic situation near the accident site. These reports will be pushed to Rescue Center and Police Station respectively.

The Rescue Center analyses the report about the accident scene, and then makes a proper rescue plan. The Police Station analyses the report about traffic situation, and then makes a proper travel control plan.

    - Alternative Flow

None.

    - Post-conditions

Based on the detailed information provided by the ITS service on the M2M platform, the rescue team can make a proper rescue plan, and the police can make a proper travel control plan.

    - High Level Illustration

Figure 6.3.9-1: High Level Illustration of Traffic Accident Information Collection

    - Potential Requirements

- A M2M System shall support communication between M2M Platform and a M2M device either directly or via a gateway.

A M2M System shall be able to exchange information between M2M applications via M2M Platform.

A M2M System shall be able to take actions according to the received service requests from M2M Applications.

A M2M system shall be able to support service requests from M2M applications for communication with QoS requirement, such as, higher delivery priority, reliable delivery, etc.

A M2M System shall support mutual-authentication among M2M device, M2M gateway, M2M platform and M2M Application ([i.2] SER-040).

The information sent by a M2M device or the M2M platform or a M2M application shall use cryptographic technology to ensure information authentication and information integrity.

A M2M system shall permit information being provided in anonymous way.

A command issued by a M2M System shall be able to have time expiration or geography restriction.

  - Fleet Management Service using DTG (Digital Tachograph)

    - Description

"DTG-based fleet management service" is the fleet management services utilizing DTG data and related service, to facilitate extensive service features of fleet management.

DTG provides vehicle data such as driving speed, RPM (Revolution Per Minute), brake's status, and mileage, etc.

DTG data management service, based on M2M gateway and DTG data management server, reports and manages DTG data in real-time to store it in the memory of M2M device in vehicle at a certain rate (i.e. one second in this case) to submit it to the national authority or transfer it to central office managing the data in a server.

The fleet management service utilizing the above mentioned service functionality provides advanced service features such as the precise quest of vehicles based on location and the tracking of cargo along with the route of the carrier vehicle, by means of the capability of remote monitoring and control of vehicle status provided by the DTG data management service.

    - Source

oneM2M-REQ-2013-0219R01 Use case - Fleet management using DTG.

    - Actors

DTG device manufacturer to provide DTG devices and DTG management system.

M2M device manufacturer to provide M2M gateway and related functionalities.

The service provider for fleet management service using DTG.

The network provider supporting the communication for fleet management service.

The national agency that manages and operates DTG data (in case of Korea).

    - Pre-conditions

The DTG device records the DTG data occasionally or periodically to transfer it to an application server through M2M Gateway.

M2M service gateway delivers the DTG data, useful to fleet management service, from terminal system to the application server.

Application server provides fleet management service, using DTG data, to customer.

A taxi call service provider operates fleet management service using DTG data, such as for reporting the taxi location and passenger status and for call arrangement.

A bus traffic service provider operates fleet management service using DTG data, including for providing guide information on bus arrival/estimated time, bus schedules on web-site, and status information such as route and air pressure of tire, etc.

A fleet management service provider of truck operates fleet management service based on vehicle information (location, route, gas, tire pressure, etc.) and peripheral device information (temperature, humidity, door lock and goods weight, etc.)

    - Triggers

The following triggers could initiate the information exchanging process according to the flows described hereafter followings:

Creation of DTG data that M2M device occasionally or periodically transfers to an application server.

Arrangement of taxi service calls delivered to a DTG device.

Report of information about vehicle location and route to application server.

    - Normal Flow

DTG service (Common service):

DTG data is periodically (normally once in a second in this case) transferred and stored into DTG management server, and in case of an accident event, the data is stored at an immediate mode (within 10ms in this case).

The DTG data stored in DTG device will be transferred to DTG management server periodically, and once after the engine stopped.

The DTG management server stores DTG data and accident event file which is to be posted onto the web site of national agency.

Analysis of DTG data and accident data to provide driving behavioural habits (quick start/stop, excessive speed) or the accident causes.

Figure 6.4.6-1: Taxi Call Service Normal Flow

Taxi call service:

Terminal system occasionally or periodically reports location, passenger status information to application server (FMS server).

Customer requests taxi call service to the taxi call l through a phone call or smart phone application.

Taxi call center sends call request to a terminal system in the taxi through the application server.

The taxi driver accepts the call request through the terminal system, and then the taxi will come to the customer's location.

Figure 6.4.6-2: Normal Flow - Fleet Management Service (Truck)

Fleet Management Service (Truck):

Terminal system occasionally or periodically reports the vehicle status information including the location, current route, ignition status, terminal version, and driver information to application server (FMS server).

When the application server receives the information, it delivers it to logistic management center.

Terminal system also reports the peripheral information (air pressure of tire, gas gauge, temperature, humidity, door lock, etc.) to the logistics management center through the application server.

Logistic management center can request the information about vehicle itself or peripheral device, to enforce possible controls to them when it is needed.

Terminal system reports the emergency events, such as fire in car, unlocked doors when unattended, and puncture while driving, etc. to FMS server

Figure 6.4.6-3: Normal Flow - Fleet Management Service (Bus)

Fleet Management Service (Bus):

When the application server receives the vehicle information (engine ignition, terminal version, car S/N, and driver ID, etc.) from terminal system, it provides the received information to the BTS management server.

BTS management server sends time schedule, route of bus and the fare information to terminal system through the application server (FMS server).

Terminal system sets the time schedule, the route, and the fare information. And then it occasionally or periodically reports its location and the driving route to application server.

Terminal system also reports the information about peripheral devices such as air pressure of tire, gas gauge level, and bus fare status to BTS management server occasionally or periodically.

BTS management server provides an arrival/estimated time and a bus schedule on web-site.

    - Alternative Flow

None.

    - Post-conditions

None.

    - High Level Illustration

Figure 6.4.9-1: High Level Illustration Fleet Management

    - Potential Requirements

- Provisioning, installation, configuration and registration method of terminal system:

Especially for the case of overlapping two different system for DTG management system (owns and manages the device) and the application system using DTG data (utilizing the data from the device).

DTG/FMS data storing method and delivery protocol:

There is no dominant standard specifying data formats and protocols for vehicle related applications.

Vehicle location based service method:

M2M service platform is expected to provide the service capability supporting location based service.

Control, configuration, error logging, and management method for the terminal system Over-The-Air:

M2M service platform is expected to provide the service capability supporting the Over-The-Air management.

  - Electronic Toll Collection (ETC) service

    - Description

ETC is an important part of Intelligent Transportation System (ITS). It is vigorously promoted in many countries.

ETC aims to eliminate the delay on toll roads by collecting tolls electronically. ETC determines whether the vehicles passing are enrolled in the program, alerts enforcers for those that are not, and electronically debits the accounts of registered vehicle owners without requiring them to stop.

In the ETC Use Case, On Board Unit (OBU) is a dedicated device located in the vehicle and it can communicate with the local RSUs only. Stated differently, the communication between OBU and ETC platform is via the RSUs. With a vehicle moving, the OBU would connect to the next RSU and release connection with the previous RSU. It is necessary for the OBU to 'register' with ETC Service Platform for receiving M2M system services. The term 'registration' here refers to an OBU having its contextual information available at the ETC Service Platform. Such registration information is for the duration of an OBU being subscribed to services from the ETC Service Platform. At the same time, OBU 'registers' with local RSUs also, as OBU connects to different RSUs. Such registration context at the RSUs is temporary, and gets released as the OBU moves out of the range of the RSU and connects to the next RSU. The OBU uses its connectivity with local RSUs for achieving communication with ETC Service Platform.

As regards the 'registration context' between the OBU and the ETC Service Platform, the contextual information needs to reflect the complete profile of the OBU at the ETC Service Platform. Such information can include OBU identity, credentials, service subscription information, payment history, account balance, etc. This is referred to here as 'full registration'.

As regards the 'registration context' between the OBU and the RSU, such contextual information is a subset of the contextual information at the ETC Service Platform for the said OBU. The information in this subset needs to be sufficient for the RSU to identify the OBU to the ETC Service Platform for the OBU to receive desired services without compromising any sensitive information to the RSU. This is referred to here as 'lightweight registration'.

    - Source

REQ-2014-0431R03 Use cases for Electronic Toll Collection (ETC) service.

REQ-2014-0449R02 Use cases for Electronic Toll Collection (ETC) service.

    - Actors

Vehicle Owner enrols for ETC service.

On Board Unit (OBU) is a M2M device used to store information such as identifier of the vehicle. The OBU typically does not communicate with the ETC Service Platform directly while receiving services. Direct communication between OBU and the ETC Service Platform may however be supported, out of band, for example for subscribing for ETC services.

Road Side Unit (RSU) is a device which is an intermediate entity and is available to connect from OBU to the ETC Service Platform. Typical role of RSU can be implemented as an M2M gateway, and it may provide other functions as well (e.g. traffic light control, barrier control, etc.).

ETC Service Platform is responsible for collecting the information regarding the OBU via the RSU, including information such as location information from the RSU.

ETC Service Provider provides its own M2M services for the user (OBU) through the ETC Service Platform.

    - Pre-conditions

Vehicle owner enrols his/her vehicle for ETC services.

All RSUs have their respective context (registration information) available at the ETC Service Platform.

    - Triggers

A vehicle equipped with OBU drives close to an electronic road charging station which equipped with RSU.

    - Normal Flow

Figure 6.5.6-1: Normal Flow for Electronic Toll Collection (ETC) service

- Vehicle owner subscribes to ETC services and the OBU is 'registered' with the ETC Service Platform. All RSUs are also registered with the ETC Service Platform.

NOTE:	The term 'registration' refers to communication and resulting contextual information of the registering entity at the registered entity.

When the vehicle goes to a highway entrance (location A), OBU registers with RSU-A.

The Vehicle (OBU) sends a request to ETC Service Platform via RSU-A asking for permission to pass through.

RSU-A forwards the request from the OBU along with its own identifying information e.g. location information, to the ETC Service Platform.

ETC Service Platform processes the request for the OBU, received from RSU-A, and responds to RSU-A. The processing at ETC Service Platform can include charging the 'toll' to OBU account and updating account balance etc., for the OBU. The response to RSU-A includes the recommended action for the OBU for pass through.

RSU-A decides whether vehicle can pass through according to the response from the ETC Service Platform.

The vehicle drives to the highway exit at location B, and OBU registers to RSU at location B (RSU-B).

The vehicle sends a request to ETC Service Platform via RSU-B asking for permission for pass through.

RSU-B forwards the request for the OBU along with its own information e.g. location information to the ETC Service Platform.

ETC Service Platform processes the request for the OBU received from RSU-B. The processing at ETC Service Platform can include charging the 'toll' to OBU and updating account balance etc. for the OBU. The ETC Service Platform may communicate with the ETC Service Provider, as needed, while processing the request from the OBU.

ETC Service Platform responds to RSU-B by including information such as the recommended action for the OBU to pass through.

RSU-B decides whether vehicle can pass through according to the response from the ETC Service Platform.

    - Alternative Flow

None.

    - Post-conditions

None.

    - High Level Illustration

Figure 6.5.9-1: High Level Illustration for Electronic Toll Collection (ETC) service

    - Potential Requirements

The M2M System shall provide the capability for an M2M device to maintain registration with multiple entities simultaneously.

The registration shall be able to include information that identifies the peer entity, and other information necessary for the establishment of the respective peer relationships (e.g. management privilege, subscription).

The M2M System shall be able to hold the complete set of information context about the peer entity for some registrations (i.e. "full registration").

The M2M System shall be to hold only a subset of information context about the peer entity for some registration (i.e. "lightweight registration").

The M2M System shall be able to perform "lightweight registration" at different entities pertaining to a common peer entity and to hold different sets of information about the common peer entity.

The M2M System shall be able to correlate the "full registration" and the "lightweight registration" pertaining to a common peer entity.

The M2M System shall be possible to distinguish the "full registrations" and the "lightweight registrations" pertaining to a common peer entity.

The M2M System shall enable each of many M2M devices to perform mutual authentication with each of many M2M gateways.

The M2M System shall enable mutual authentication of M2M devices and M2M gateways in a timely manner.

  - Taxi Advertisement

    - Description

For the taxi advertisement device, it does NOT need to access to the network during the day when the network is busy (e.g. from 8:00 to 23:00), but need to access the network at night when the network is idle (e.g. from 23:00 to 8:00) to download a large number of advertisement data.

    - Source

REQ-2014-0467R02 Use case for taxi advertisement.

    - Actors

Taxi advertisement device, which can download advertisement data from advertisement data server through M2M service platform and show to passengers.

The M2M service platform, which can control the taxi advertisement device and its access to the network.

Advertisement data server, which can provide advertisement data for the advertisement device to download.

    - Pre-conditions

The taxi advertisement device and the advertisement data server registered to the M2M service platform.

    - Triggers

The taxi advertisement device accesses to the advertisement data server though M2M platform to download data.

    - Normal Flow

The taxi advertisement device accesses to the advertisement data server through M2M platform.

The M2M service platform checks the time policy. If current time is permitted, it allows the device to access the advertisement data server. Otherwise, it denies access and instructs the taxi advertisement device not to access the advertisement data server until the time when access is permitted.

When access is permitted the taxi advertisement device downloads data from advertisement data server.

    - Alternative Flow

None.

    - Post-conditions

None.

    - High Level Illustration

Figure 6.6.9-1: High Level Illustration of Taxi advert use case

    - Potential Requirements

- The M2M service platform shall be able to support the time-based policies to access the Underlying network ([i.2] CMR-014).

  - Vehicle Data Service

    - Description

This use case introduces several services based on various data collected by sensor devices via smart vehicle (vehicle with on-board communication system) which is regarded as M2M gateway.

The sensor devices may be located in the vehicle but may also be located outside the vehicle, e.g. on the road side. Some sensor devices are equipped with M2M area network module and measure individual data. The smart vehicle connects to the sensor devices and collects data from sensor devices by using the M2M area network technology such as Wireless LAN, ZigBee, Bluetooth, etc., and sends the data to application server in infrastructure domain via mobile network.

Management server and Application Server in the M2M infrastructure domain connect to the smart vehicle via a mobile network in order to control its configurations updating software and exchanging M2M data (e.g. updating a map).

It is important to observe that the smart vehicle as M2M gateway has mobility. For instance, there are possibilities for a mobile device to simultaneously connect too many sensor devices, and to newly connect to sensor devices which have never been connected before.

This use case illustrates potential requirements from the use case of services utilizing mobile device.

    - Source

REQ-2014-0472R06: Use Case on Vehicle Data Services.

    - Actors

M2M Device: In-vehicle sensor device and outdoor sensor device. In-vehicle sensor includes on-board (built-in) sensor used for monitoring of machine health and also sensor on user carry-on device, such as smart phone and wearable device like Fitbit™, Nike Fuelband™, Sony SmartBand™ for example. In-vehicle sensor may monitor status of machine health (diagnosis), mobility, passenger's health and environment. Outdoor sensor device may be located outdoor and monitor roadway infrastructure, agriculture, property (surveillance) and utility (telemetering), for example. It may be equipped with several kind of communication protocol.

M2M Area Network: Area network which connects M2M Device with M2M Gateway and also provides connectivity among M2M Gateways. It may include Wireless LAN, Bluetooth, ZigBee and Ethernet.

M2M Gateway: On-board communication system equipped on the smart vehicle, which communicates with M2M Devices and other M2M Gateways via M2M Area Network and also communicates with Application Servers and Management Servers via Mobile Network.

Mobile Network: Network which has functions to transfer data and control messages between M2M Gateway and M2M Application Server/Management Server. It may include cellular base station.

M2M Management Server: Server which manages M2M applications in M2M Gateway and M2M Device by installing, uninstalling and updating them.

M2M Application Server: Server which maintains database and provides the data access services such as accepting data publication from and issuing data subscription to M2M Gateway/Device. This server also manages non-M2M applications such as navigation system as well as contents such as map data in M2M Gateway and M2M Device by installing, uninstalling and updating them.

    - Pre-conditions

It is possible to establish a connection among M2M Application Server, M2M Management Server and Smart Vehicle via Mobile Network.

It is possible to establish a connection between Smart Vehicle and M2M Device and among Smart Vehicles via M2M Area Network.

The M2M Gateway has been configured by the M2M Management Server.

The M2M Device has not yet been configured by the M2M Management Server.

    - Triggers

Subject to capabilities of the M2M Area Network, the M2M Device detects a M2M Gateway that can be associated, or vice versa. The association may require explicit operation (permission) for that association by users (e.g. owners of Gateway and/or Device) or administrator (a manufacturer of Gateway and/or Device and/or Management Server). Examples are below:

Sensors are built into the smart vehicle at the factory (Permanent Association, no triggering is needed).

Sensors are equipped to the smart vehicle after market, e.g. at auto dealers or auto parts stores. Triggering occurs when the smart vehicle is being equipped with the new sensor.

Users bring sensors with user carry-on devices into the smart vehicle.

The smart vehicle detects nearby outdoor sensors.

The smart vehicle detects other smart vehicles nearby.

    - Normal Flow

M2M Configuration step: Management Server configures M2M Device.

Upon triggering, i.e. when the M2M Device detects a M2M Gateway that can be associated or the M2M Gateway detects a M2M Device that can be associated the following sequence is initiated:

M2M Gateway establishes a connection to M2M Device (or the M2M Device establishes the connection to the M2M Gateway) via the M2M Area Network.

M2M Device sends its attribute information (e.g. type of device, service certificates of the device, required application software, etc.) to the M2M Gateway.

M2M Gateway establishes a connection to the M2M Management Server via Mobile Network.

M2M Gateway relays the attribute information to Management Server.

M2M Management Server provides M2M Gateway with the appropriate software and configuration data for the M2M Device.

M2M Gateway relays the software and configuration data to M2M Device.

M2M Device configures itself according to the software and configuration data.

Transmission of data between M2M Device and M2M Application Server:

Data Publication of M2M Device.

M2M Gateway (re-)establishes a connection to M2M Device via M2M Area Network.

M2M Device publishes measured data to M2M Gateway.

M2M Gateway establishes a connection to M2M Application Server via Mobile Network.

M2M Gateway relays the data to M2M Application Server. M2M Gateway may distribute the data (i.e. data publication) to multiple M2M Application Servers whose data subscriptions have been previously grouped/aggregated at M2M Gateway.

M2M Application Server receives the data.

Data Subscription from M2M Application Server:

M2M Gateway establishes a connection to M2M Application Server via Mobile Network.

M2M Application Server sends M2M Gateway attribute information of data in need.

M2M Gateway (re-)establishes a connection to M2M Device via M2M Area Network.

M2M Gateway relays the attribute information to M2M Device. M2M Gateway may group/aggregate the attribute information (i.e. data subscription) from multiple M2M Application Servers.

M2M Device publishes measured data that meets the attribute information to M2M Gateway.

M2M Gateway relays the data to Application Server. M2M Gateway may distribute the data (i.e. data publication) to multiple M2M Application Servers whose data subscriptions have been previously grouped/aggregated at M2M Gateway.

Application Server receives the data.

Update of non-M2M Application 1: M2M Application Server and M2M Gateway:

M2M Gateway establishes a connection to M2M Application Server via Mobile Network.

M2M Gateway sends its attribute information to M2M Application Server.

M2M Application Server provides M2M Gateway with the appropriate software, its configuration data and contents such as map.

M2M Gateway configures itself according to the software, configuration data, etc.

Update of non-M2M Application 2: M2M Application Server and M2M Device:

M2M Device establishes a connection to M2M Gateway via M2M Area Network.

M2M Device sends its attribute information to M2M Gateway.

M2M Gateway establishes a connection to M2M Application Server via Mobile Network.

M2M Gateway relays the attribute information to M2M Application Server.

M2M Application Server provides M2M Gateway with the appropriate software and configuration data.

M2M Gateway relays the configuration software and data to M2M Device.

M2M Device configures itself according to the software and configuration data.

    - Alternative Flow

Alternative Flow 1

This alternative flow may occur in the case where the M2M Gateway only occasionally connects to devices and servers - e.g. via M2M Area networks that can only occasionally be used:

M2M Gateway Opportunistic Communication (Store and Forward):

M2M Gateway may store data that are destined to M2M Device, M2M Management Server and M2M Application Server.

M2M Gateway may send the stored data to other M2M Gateway.

Both, the originator and the other M2M Gateway may deliver the stored data to the destination when connected.

M2M Gateway may erase the stored data that has been already sent to the destination.

Alternative Flow 2

This variant flow may occur in the case where the M2M Gateway processes the data flow between M2M, Gateway and M2M Application Server:

M2M Gateway Data Processing:

M2M Gateway may aggregate, statistically summarize (e.g. average) and/or erase data based on criteria that are indicated in the data subscription or the configuration data from M2M Application Server.

M2M Gateway Data Subscription:

M2M Application Server may be interested in joint changes to multiple resources (either on the same M2M Device or multiple M2M Devices). In other words, M2M Application Server makes a data subscription where automatic notification (i.e. data publication) depends on two or more resources, not a single resource. Notifications are then generated when the expected changes occur within each of the resources.

Alternative Flow 3

This variant flow may occur in cases where the M2M Gateway broadcasts its interest to subscribe to specific data to all M2M Devices in its vicinity:

M2M Gateway Cross Layer Optimization:

M2M Gateway may indicate a full or part of data subscription (attribute information of data in need) in wireless pre-association information such as beacon.

Smart Vehicle and Wi-Fi Hotspot may periodically broadcast beacon that contains the subscription in order to prevent sensor devices from establishing unnecessary connection and wasting radio resources as well as battery power.

NOTE:	M2M Application Data Security (TBD).

    - Post-conditions

M2M Application Server stores data and provides data access service via API for user applications.

M2M Devices and M2M Gateways are well maintained by M2M Management Server and M2M Application Server.

    - High Level Illustration

Figure 6.7.9-1 High Level Illustration - Vehicle Data Service

    - Potential requirements

- The oneM2M System shall enable discovery of M2M Application Servers, M2M Management Servers and M2M Devices available to an M2M Gateway for data exchange ([i.2] OSR-086).

The oneM2M System shall enable discovery of M2M Gateways available to a M2M Management Server and an M2M Device for data exchange ([i.2] OSR-087).

The oneM2M System shall be able to support the capabilities for data repository (i.e. to collect/store) and for data transfer from one or more M2M Devices or M2M Gateways, for delivery to one or more M2M Gateways via M2M Area Network without any assistance or instruction of M2M Management Servers and M2M Application Servers ([i.2] OSR-088).

Upon request from M2M Application Server, an M2M Gateway or M2M device shall enable functions that pre-process (e.g. average) M2M data before providing them to the recipient ([i.2] OSR-104).

Upon request, an M2M Gateway or M2M device shall enable functions that erase M2M data (e.g. that have been sent or could not be sent to the recipient within a certain time) based on criteria from an M2M Application ([i.2] OSR-105).

An M2M Gateway and/or an M2M Device shall be able to broadcast to all M2M Devices and/or M2M Gateways in the vicinity its need to receive/deliver specific data ([i.2] OSR-106).

M2M Gateway and/or M2M Device shall be able to establish a connection to each other if it is able to receive/deliver the required specific data ([i.2] OSR-107).

The oneM2M System shall enable M2M Gateways to set conditions used for processing jointly data subscriptions and distribute the resulting notifications according to the set conditions ([i.2] OSR-108).

The oneM2M System shall enable subscriptions to changes to multiple resources which aim to generate notifications if and only if the expected changes to those resources occur concurrently ([i.2] OSR-109 and OSR-110).

  - Smart Automatic Driving

    - Description

Attention to automatic driving is increasing. An automatic driving technology normally superimposes dynamic information gathered by car-mounted sensors and/or cameras on a static high-definition map. It makes possible to calculate the way which is appropriate for the vehicle to take.

Because an automatic driving vehicle gathers dynamic surrounding circumstances by sensors and/or cameras, the coverage of the vehicle's perception is normally almost a few hundred meters. Therefore, the automatic driving vehicle may be forced to make a sharp lane change, a sudden stop or a return of its control to a driver at the worst, in case of unusual state such as a crash, a road work or a dropping on the road.

In order to make a smooth lane change, a gradual slow down or a fully prepared return of its control to a driver even in such unusual states, the system which can collect current road conditions and feedback credible information to relevant vehicles is required. The information can be used by each vehicle to calculate the driving way such as a lane, path or speed.

    - Source

REQ-2015-0554-Smart Automatic Driving.

    - Actors

Vehicle Driving Support Center

It distributes high-definition maps to vehicles supporting an automatic driving. Furthermore, it collects unusual states from vehicles on the road, validates its credibility and feeds back credible information to relevant vehicles. If there are devices/GWs located at the roadside, some of the functions of vehicle driving support center may be provided via these devices/GWs.

OneM2M System

It connects between the vehicle driving support center and vehicles. Furthermore, the devices/GWs located at the roadside may provide some of the functions of vehicle driving support center.

Vehicles

The automatic driving vehicles have multiple on-board sensors and cameras for the automatic driving which superimposes dynamic information gathered by the sensors and/or cameras on a static high-definition map distributed by the vehicle driving support center. Furthermore, the automatic driving vehicles use unusual states information from the center to calculate the driving way such as a lane, path or speed.

    - Pre-conditions

- Some vehicles support automatic driving.

The automatic driving vehicles have a mobile communication module and a function to notice their own location and on-board camera image to the vehicle driving support center.

    - Triggers

The automatic vehicles report occurrence of an unusual state such as a crash, a road work or a dropping on the road.

    - Normal Flow

- The automatic driving vehicles reaching the location where the unusual state occurs detect the state by on-board sensors and cameras. As a result, the vehicles change a lane, slow down, stop or return the control to the driver, which become sudden operations depending on those situations. Additionally, the vehicles report the unusual state to the vehicle driving support center. The state is reported as on-board camera image with additional information such as current time and locations of the vehicle.

The center receiving those reports identifies the location where the unusual state occurs, compares the reports which seem to be sent from same location and validates its credibility.

The center notices the unusual state to the vehicles which are reaching the location of the unusual state after finishing the credibility validation. The notice is protected in an appropriate way such as digital signature, because this information is used by each vehicle to calculate the way of the automatic driving.

The automatic driving vehicles receiving the notice from the center make a smooth lane change, a gradual slow down or a fully prepared return of its control to the driver before reaching the location of the unusual state.

    - Alternative flow

None.

    - Post-conditions

None.

    - High Level Illustration

Figure 6.8.9-1: High Level Illustration of smart automatic driving

    - Potential requirements

- OneM2M System shall be able to send the information to intended vehicles by unicast, multicast and/or broadcast.

OneM2M System shall be able to securely transfer the information by using an appropriate means such as digital signature.

OneM2M System shall be able to transfer the information on real-time basis for feeding back current road states to automatic driving control. The feedback time should be less than a few seconds (the distance between vehicles normally corresponds to a few seconds) to avoid unnecessary slow down/stop of following vehicles.

OneM2M system shall be able to guarantee its reliability in order to receive/feedback messages from/to related vehicles.

oneM2M System shall enable sharing of service information between devices/GWs based on proximity.

oneM2M System shall enable sending and receiving service information between devices/GWs with minimized interruption.

  - Use Case on Vehicle Data Wipe Service

    - Description

This use case introduces vehicle data wipe services in addition to the use case on vehicle data services as REQ-2014-0472R06.

Background: A data center on the cloud collects sensor data from vehicles using the mobile network (e.g. cellular and wireless LAN). The data may relate to diagnosis, mobility and context of vehicles. The diagnostic data are useful for vehicle design improvement and the mobility data for dynamic route guidance services. The contextual data are captured by stereo cameras and radar scanners in the automated driving system. And then those data may constitute the complete 3D roadway map which is also essential for the automated driving system in turn. In the use case of REQ-2014-0472R06, the data center, that is the M2M application server, requests data by sending a meta-data to vehicles. Then vehicles prepare data which meet the criteria of the meta-data and publish it to the server. The meta-data may describe attributes of requested data such as time period, geographical area, data type, statistic process options and so forth. Figure 6.9.9.2-1 shows how this simply works.

Problem statement: Though a certain volume of data (big data) obtains utility value, the bigger data gradually show less increase of the value (as is shown in Figure 6.9.9.3-1 and Figure 6.9.9.3-2). For instance, no more samples are required for statistical analysis once the number of samples gets sufficient. Duplicated collections of collected pieces of map are also redundant. Such wasteful data deliveries consume multiple expensive resources of vehicles, mobile network, network backhaul and servers. Data in the vehicle may also lose its value when it gets delivered, obsolete, false or even malicious.

Solution: Such unwanted data collection needs to be cut off in order to spare those resources for other useful data delivery. In addition, unwanted data in a limited capacity of the vehicle storage need to be wiped out. These operations should be carried on under the right authorization.

    - Source

REQ-2015-0589R04-Usecase_on_vehicle_data_wipe_service.

    - Actors

M2M Device: Data source node such as sensors in vehicles.

M2M Gateway: Data relay node such as a vehicle on-board communication system with storage.

M2M Application Server: Data requesting node such as a data center on the cloud.

    - Pre-conditions

The M2M Application Server has disseminated a data request with the meta-data to vehicle(s) (as is shown in Figure 6.9.9.4-1).

NOTE:	The meta-data may describe attributes of requested data such as time period, geographical area, data type, statistic process options and so forth.

    - Triggers

The M2M Application Server decides to cancel the data request and erase the data in the vehicles. (One of the reason for this could be that the data has been collected.)

    - Normal Flow

- The M2M Application Server transmits a wipe request with the same meta-data in the target data request to the vehicle(s).

- On arrival of the wipe request, the vehicle deletes the meta-data and data which meet the criteria of meta-data from in its storage.

    - Alternative flow

If it is additionally instructed in the wipe request, the wipe request can be delivered from the vehicle to other vehicles.

If it is additionally instructed in the wipe request, the vehicle may only stop publishing data which meets the criteria of meta-data, instead of deleting.

The M2M Application Server and vehicles may share a credential and use the credential to authenticate the wipe request (as is shown in Figure 6.9.9.6-1). The credential could be a pre-shared key or could be based on public key infrastructure. The credential could also be identified in the data request.

    - Post-conditions

There is no data relating to the cancelled data request in the vehicle storage.

    - High Level Illustration

      - Data Request and Response

Figure 6.9.9.1--1 Deployment

      - Data Request and Response

Figure 6.9.9.2-1 Data Request and Response

      - Issue of Bigger Data

Figure 6.9.9.3-1 Less increase of value

Figure 6.9.9.3-2 Stop further collection

      - Pre-condition of Data Wipe (and Post-condition of Data Request and Data Response)

Figure 6.9.9.4-1 Pre-condition of data wipe

      - Data Wipe

Figure 6.9.9.5-1 Data wipe

      - Data Wipe with Authentication

Figure 6.9.9.6-11 Data wipe with authentication

      - Post Condition of Data Wipe

Figure 6.9.9.7-1 Post condition of data wipe

    - Potential requirements

- The oneM2M System shall enable the cancellation of continuous data collection and/or the deletion of collected data when pre-defined conditions are met ([i.2] OSR-089).

The oneM2M System shall enable pre-defined conditions to be protected from unauthorized modification ([i.2] SER-050).

The oneM2M System shall enable the deletion of M2M data produced/stored by the M2M Devices/Gateways based on request from an authorized entity ([i.2] OSR-051).

  - Vehicle Management based on Geo-Fence

    - Description

Since a vehicle is a nomadic object, localization of vehicles and collecting information for management are crucial features for vehicle management.

From the holistic viewpoint, vehicle management is a generic term and may address various vehicle and smart car services such as diagnostic, maintenance and so on.

Specifically, in this use case, vehicle management based geo-fence feature is about how to monitor the location and movement of a target vehicle efficiently (e.g. judge where the movement is illegal or legal for example).

    - Source

REQ-2016-0002R01 Vehicle Management based on Geo-fence.

    - Actors

Vehicle

Vehicle is a nomadic object which is monitored by oneM2M infrastructure node or Location Server and is equipped with hardware devices for localization (e.g. GPS, Cellular modem).

oneM2M Infrastructure Node

A system exposes the service functions for vehicle management such as Location, Device management and potentially additional services.

Location Server

A Location Server can localize the target device(s) using collected measurements (e.g. RSSI, AP ID) and detect Geo-Fence event based on set configurations.

LBS/IoT Application

An Application requests Geo-Fence based location service toward oneM2M Infrastructure Node.

    - Pre-conditions

N/A.

    - Triggers

A LBS/IoT Application requires to set Geo-Fence-based Location-Based services depending its demand.

    - Normal Flow

Figure 6.10.6-1 Normal Flow - Vehicle Management based on Geo-Fence

- A vehicle Device registers with oneM2M Infrastructure.

A LBS/IoT Application requests the Geo-Fence Service to oneM2M Infrastructure. The Application set the configuration regarding Geo-Fence Service such as Geo-Fence area, event criteria, quality of event detection and so on.

oneM2M Infrastructure acquires Measurement and/or Position fix from the Vehicle devices.

The Measurement and/or Position fix are/is forwarded to Location Server by oneM2M infrastructure.

Location Server runs Geo-Fence detection engine and analyses whether the Geo-Fence event has occurred or not. The detection is operated following the configuration of application.

When any event has occurred, the event reports to the LBS/IoT Application.

    - Alternative flow

N/A.

    - Post-conditions

N/A.

    - High Level Illustration

Figure 6.10.9-1 High Level Illustration - Vehicle Management based on Geo-Fence

Vehicle Devices can gather the signal measurement and/or position fix.

The Devices are registered and connected to oneM2M Infrastructure.

GeoFence Server can detect any movement event based on the Application's configuration.

    - Potential requirements

- The oneM2M System shall support reporting of Geo-Fence based Location event of the target M2M Device to the M2M Application based on the Application's configuration ([i.2] OSR-047).

The oneM2M System shall support the M2M Application setting the configuration for Geo-Fence based location service.

  - Secure Over-The-Air Firmware Update for Automotive ECUs

    - Description

This use case introduces secure remote Over-The-Air (OTA) Firmware Update scheme for M2M Devices such as ECUs (Electronic Control Units) through on-board M2M Gateway in automobiles. An automobile which has an on-board M2M GW is called "Smart Vehicle".

Each M2M Device is connected to the on-board M2M Gateway Unit by M2M Area Network such as Controller Area Network (CAN).

M2M Management Server in Infrastructure Domain collects information on version of currently installed firmware in ECUs and distributes updated firmware dedicated for each M2M Device through the M2M Gateway using Mobile Network.

    - Source

REQ-2016-0012R01 Use Case on OTA Firmware Update for ECUs.

    - Actors

M2M Device: Electronic Control Unit (ECU) in an automobile is assumed as an M2M Device. There are many kinds of on-board ECUs in automobile, e.g. Power train ECU, Transmission ECU, Brake ECU, etc.

M2M Gateway (M2M GW): M2M GW is connected with M2M Devices to collect M2M Data and distribute firmware through multiple M2M Area Networks depending on kinds of functionality or security requirements. M2M GW is equipped with a data communication module to communicate with M2M Management Servers and external servers in Infrastructure Domain. Also, it has a display device in order to confirm user's authorization for the firmware update operation.

M2M Area Network: CAN (Controller Area Network) or LIN (Local Interconnect Network) is used as wired M2M Area Network in an automobile.

Mobile Network: Mobile Network is assumed to transfer M2M Data and messages between M2M GW and M2M Management Server.

M2M Management Server: This server is utilized to conduct firmware update operation and management of version of firmware currently installed in M2M Devices.

OEM: Original Equipment Manufacturer means automobile manufacturer who is ultimately responsible for the product (e.g. firmware for ECU).

Firmware Developer: A developer of firmware dedicated for on-board M2M Device (ECU).

User: A person who makes a decision for conducting firmware update of ECU, when a request for the update is notified from M2M Management Server. This assumes to be an owner of automobile.

Figure 6.11.3-1: High Level Structure for Over-The-Air Firmware Update for Automotive ECUs

    - Pre-conditions

M2M GW is connected to M2M Management Server via Mobile Network.

M2M GW is connected to M2M Device through CAN.

M2M GW has capability to manage status of on-board equipment in Smart Vehicle.

    - Triggers

Trigger from Server:

When a new firmware becomes available, M2M Management Server notifies it to M2M GW and M2M GW notifies it to User via a display device of M2M GW.

Trigger from User:

User can operate M2M GW to check availability of updated firmware for ECUs in M2M Management Server.

    - Normal Flow

A: Generation of updated firmware

- Firmware Developer generates updated firmware for an ECU and submits it to OEM for approval.

- OEM verifies the updated firmware and attaches OEM's Digital Signature to it as token of approval.

- OEM uploads the updated firmware with the Digital Signature to M2M Management Server.

B: Authentication

M2M Management Server and M2M GW conduct mutual authentication. It is possible to avoid unintentional outflow of firmware by distributing it to the right M2M GW using the authentication mechanism. M2M GW can conduct firmware update operation based on request from the trusted Management Server. Widely used authentication procedures such as Challenge and Response using shared key cryptosystem and public key cryptosystem are applicable.

C: Establishment of Secure Channel

Encryption of communication channel between M2M Management Server and M2M GW is carried out after the mutual authentication. Encryption procedure such as SSL is applicable.

D: Verification of current version of Firmware installed in M2M Device

- M2M Management Server sends a request to M2M GW in order to verify version information of firmware currently installed in M2M Device.

M2M GW transfers the verification request to M2M Device.

M2M Device sends version information of firmware currently installed back to M2M GW together with its own Digital Signature.

M2M GW transfers the version information with the Digital Signature of M2M Device to M2M Management Server.

Management Server verifies the version information with the Digital Signature and checks presence of firmware to be updated for the M2M Device.

E: Distribution of updated Firmware

- M2M Management Server distributes the updated firmware with OEM's Digital Signature to M2M GW.

M2M GW carries out verification of the Digital Signature of the firmware.

F: User Authorization for Firmware Update

M2M GW sends a message to User in order to confirm permission from the User to conduct firmware update operation, with information regarding firmware update operation.

G: Firmware Update Implementation

- M2M GW transfers the updated firmware to M2M Device.

M2M Device verifies Digital Signature of the firmware and gets updated with the firmware if it is the right firmware.

H: Notification of Firmware Update completion

- M2M Device sends version information of the updated firmware with its Digital Signature when reboot with the updated firmware is completed.

- M2M GW transfers the firmware version information with Digital Signature of M2M Device to M2M Management Server.

- M2M Management Server verifies the Digital Signature of M2M Device and then the operation of Firmware Update is finally completed.

    - Alternative flow

(1) User Initiated Firmware Update Operation

Steps A through C are the same as shown in Normal Flow.

Step D: User Verification of  Presence of Firmware to be updated:

1)	User operates Control Display Device of M2M GW in order to check availability of updated firmware for M2M Device.

2)	M2M GW sends a request to M2M Device in order to verify version information of firmware currently installed in M2M Device.

3)	M2M Device sends version information of firmware currently installed back to M2M GW together with its own Digital Signature.

4)	M2M GW transfers the version information with the Digital Signature of M2M Device to M2M Management Server.

5)	Management Server verifies the version information with the Digital Signature and checks presence of firmware to be updated for the M2M Device.

6)	If there is firmware to be updated, follow Steps E - H of Normal Flow.

7)	If there is no firmware to be updated, M2M GW notifies User of it on the display.

(2) Constrained Device (ECU)

This alternative flow is provided for a Constrained Device which is unable to conduct attachment of its own Digital Signature or verification of Digital Signature of other servers and devices by itself due to limitation of capability. In this case, it is possible to conduct Firmware Update operation by trusting M2M GW on the assumption that M2M GW and M2M Device are mutually trusted:

Steps A through C are the same as shown in Normal Flow.

Step D: Verification of current version of Firmware installed in M2M Device:

1)	M2M Management Server sends a request to M2M GW in order to verify version information of firmware currently installed in M2M Device.

2)	M2M GW transfers the verification request to M2M Device.

3)	M2M Device sends version information of firmware currently installed back to M2M GW.

4)	M2M GW transfers the version information with its Digital Signature to M2M Management Server.

5)	M2M Management Server verifies the Digital Signature and version information, and checks presence of firmware to be updated for the M2M Device.

Step E: Distribution of updated Firmware (same as Step E in Normal Flow).

Step F: User Authorization for Firmware Update (same as in Step F in Normal Flow).

Step G: Firmware Update Implementation:

1)	M2M GW transfers updated firmware to M2M Device.

2)	M2M Device gets updated with the firmware trusting M2M GW.

Step H: Notification of Firmware Update completion:

1)	M2M Device sends version information of the updated firmware when reboot with the updated firmware is completed.

2)	M2M GW transfers the firmware version information with its Digital Signature to M2M Management Server.

3)	M2M Management Server verifies the Digital Signature and then the operation of Firmware Update is finally completed.

    - Post-conditions

M2M Management Server has to verify the status of M2M Device (firmware version, number of update failures, error information, etc.) and manage these types of information elements.

M2M Device has to have rollback function in order to restore to the original state, when the Firmware Update operation fails.

    - High Level Illustration

Figure 6.11.9-1 High Level Illustration - Secure Over-The-Air Firmware Update for Automotive ECUs

    - Potential requirements

- The oneM2M System shall be able to prevent unauthorized modification of the firmware of M2M Device ([i.2] SER-064).

The oneM2M System shall be able to detect unauthorized modification of the firmware of M2M Device ([i.2] SER-065).

The oneM2M System shall be able to stop operation of M2M device when it is updated with wrong firmware.

The oneM2M System shall be able to support security mechanisms to protect their cryptographic keys and cryptographic operations by using tamper resistant elements such as TPM (Trusted Platform Module), HSM (Hardware Security Module) and SIM (Subscriber Identity Module) ([i.2] SER-066).

The oneM2M System shall be able to prevent malfunction of M2M Device caused by receiving unsolicited messages or information ([i.2] SER-067).

  - Car/Bicycle Sharing Services

    - Description

As seen clearly, automation already penetrates all aspects of life even in our urban life. The goal of this use case is to describe several automation services which are occurred in different urban space in different life style, e.g. bicycle/car sharing services.

Brief Features of Services.

Car Sharing Service

Car Sharing is to offer a new service model for automobile transportation. Simply, Car Sharing is a self-service, on-demand alternative to car ownership; a service that is offered to urban residents (B2C) and businesses (B2B).

This service is mainly designed around a particular user profile - first of all, people who live in cities but do not drive a car every day and secondly tourists who visit cities but do not bring their car. Thus, people who need a car at short notice but take an alternative to car ownership.

The brief procedure of this service is:

- joining the membership;

- unlocking the car door;

- driving away;

- parking to any reserved spot provided by the service provider and/or public; and

- paying as you drive (including gas, insurance, etc.).

Bicycle Sharing Service

Bicycle sharing service is also a new service in which bicycle are made available for shared use to individuals who do not own a bicycle. Generally, bicycle sharing service is organized by a local government agency but may be operated privately.

The procedure of this service is similar to the car sharing service, but different type of services such as healthcare service can be combined.

    - Source

oneM2M-REQ-2012-0132R01 Use Case: Car/Bicycle Sharing Services.

    - Actors

User

A user who makes use of the shared things which are cars or bicycles.

Sensors (or Sensor Devices)

Sensor Devices can vary based on usage, and do not have any direct communication interfaces to the M2M Service Platform:

For Car Sharing Service - Door Control Sensor, Tire Pressure Sensor, Fuel Indication Sensor, GPS.

For Bicycle Sharing Service - Lock Control Sensor, Accelerometer, Tire Pressure Sensor, Heart-rate Sensor.

Smartphone

A device which is an intermediate entity and is available to connect sensors to a M2M Service Platform. The basic role is similar to the general M2M gateway, but it has some sensors and some applications (navigation) itself that may be used by services.

M2M Service Platform

In charge of providing common functionalities for the M2M services. It is mainly in charge of collecting the status and configuration information of sensors and controlling them via the smartphone and/or M2M gateway.

M2M Service Providers

Companies which provide their own M2M services for the user through the M2M Service Platform. The M2M Service Providers can be multiple according to the types of services.

The providers include Car Sharing Service Provider, Insurance Company, Gas Station, Bicycle Sharing Service Provider, and Healthcare Service Provider.

    - Pre-conditions

See sub-case flows.

    - Triggers

See sub-case flows.

    - Normal Flow

Sub use case 1 - Car Sharing Case

Trigger

A user wants to make use of the car.

Pre-conditions

The user preliminary joins a membership of the Car Sharing Service.

Sensors built in the car are required to periodically (normal) and non-periodically (urgent) send sensor data to the M2M Service Platform based on the trigger defined by the M2M Service Providers. When the trigger conditions are finished, the sensors stop to send data to the M2M Service Platform.

The M2M Service Platform collects and manages data and configurations related to the services. Generally, each service has its own data and configuration set, simply called resources.

The M2M Service Providers in the service domain have a service agreement between each other for unified services.

The Smartphone has a navigation and car sharing application.

Figure 6.12.6-1: Car Sharing Normal Flow

Detailed Flow Descriptions

- The Applications of each Service Provider in the service domain register and subscribe to changes of resources (or information) about the Car Sharing Service in the M2M Service Platform.

- Since each resource in the M2M Service Platform is owned by the Car Sharing Provider, Insurance Company or Gas Station, if an application needs to access another resource, it shall request proper access right for the resources and that request will be granted if appropriately based on the service agreement.

As the user finds a shared car, he/she opens the car door and turns on the ignition using interfaces of the Smartphone such as Bluetooth and NFC, if the user is authorized.

The Sensors report the changed status to the M2M Service Platform via the Smartphone as a gateway when the specific condition is triggered (car is just being used).

- 3a)   The smartphone report periodically the changed status that is monitored via the underlying network such as location, reachability, etc.

- 4, 4a) The M2M Service Platform notifies the Car Sharing Service Provider of the changed status.

NOTE 1:	The Car Sharing Service Provider can update the situation that the car is being used on its website.

- (Normal Reporting Case for managing the Service) The Sensors report the changed status to the M2M Service Platform via the Smartphone when the specific condition is triggered. (Periodic location reporting and car health check for maintenance reasons)

The M2M Service Platform notifies the Car Sharing Service Provider of the changed status.

NOTE 2:	Agreement on privacy policy of location is preliminarily confirmed.

(Urgent Reporting Case for handling any emergency) The Sensors report the changed status to the M2M Service Platform via the smartphone as a gateway when the specific condition is triggered (the fuel is low).

The M2M Service Platform immediately notifies the Car Sharing Service Provider of the changed status.

The Car Sharing Service Provider finds out the nearest Gas Station according to the received location information and service agreements between the Car Sharing Service Provider and Gas Stations, and the Provider sends the route information to M2M Service Platform.

The M2M Service Platform notifies the Smartphone of the route information.

After filling the fuel, the user virtually pays the fuel fee by using the Smartphone's NFC tag. The payment information is reported to the M2M Service Platform.

The M2M Service Platform notifies the Car Sharing Provider and the Gas Station of the payment information.

NOTE 3:	This procedure is for the Car Sharing Provider to pay to Gas Station the fuel fee instead of the user.

Afterwards, due to the low battery of the Smartphone (less than 30 % remain), the Smartphone reports the changed status to the M2M Service Platform.

The M2M Service Platform automatically changes the subscription and reporting attributes of the Sensors and the Car Sharing Service Provider.

EXAMPLE 1:	If the Platform changes the subscription attributes to "only emergency case", only emergency subscription case will be notified. The others cannot be notified, but at the end of service, will be sent in batch-mode.

As the user arrives at the destination, and turns off the ignition, the sensors report the accumulated information including location information and normal event subscription information, to the M2M Service Platform via smartphone.

- 15a) After the car turns of the ignition, the sensors may not send data any more. The smartphone stops reporting the changed status which is monitored via underlying network such as location, reachability, etc.

The M2M Service Platform notifies the Car Sharing Providers and Insurance Company of the usage of the shared car.

The Insurance Company claims the insurance fee by writing onto the Car Sharing Service Provider's resource in the M2M Service Platform, in this case the Insurance Company preliminarily acquires proper access right to write the resource.

The M2M Service Platform notifies the Car Sharing Provides of the insurance fee.

The car sharing provider may retrieve a location of car to get information of the number of cars for each parking lots.

The M2M Service Platform notifies a location information depending on car status either step 3a or step 15a.

Post-conditions

The User will pay as he/she drive according to the recorded data.

The Car Sharing Service Provider can update the position and status of the car on its website using the recorded data. Thus, next users can make use of the Car Sharing Service.

There are many parking lots to manage proper number of cars. Due to the vehicle sharing, certain parking lot may run out of cars or exceed. The Car Sharing Service Provider can move cars to maintain a suitable number of cars among the parking lots.

Sub Use Case 2 - Bicycle Sharing Service

Trigger

A user wants to make use of the bicycle.

Pre-conditions

The user preliminary joins a membership of the Bicycle Sharing Service.

The sensors built in the car and in the smartphone are required to periodically (normal) and non-periodically (urgent) send sensor data to the M2M Service Platform based on the trigger defined by the Service Provider.

The M2M Service Platform collects and manages data and configurations related to the services. Generally, each service has its own data and configuration set, simply called resources.

The Smartphone has a navigation and bicycle sharing application.

The M2M Service Providers in the service domain have a service agreement between each other for unified services.

Figure 6.12.6-2: Bicycle Sharing Normal Flow

Detailed Flow Descriptions

- The Applications in the service domain register the service and subscribe to changes of information about the Bicycle Sharing Service.

- Since each resource in the M2M Service Platform is owned by the Bicycle Sharing Service Provider or Health Service Provider, if an Application needs to access another resource, it shall request proper access right to the resources and that request will be granted if appropriately based on the service agreement.

To unlock the bicycle, the user tags the locker of the bicycle through the NFC interface.

The Sensors report the changed status to the M2M Service Platform via the smartphone as a gateway when the specific condition is triggered.

- 3a)   The smartphone report periodically the changed status that is monitored via the underlying network such as location, reachability, etc.

EXAMPLE 2:	The bicycle is being used.

- 4,4a)	The M2M Service Platform notifies the Bike Sharing Service Provider of the changed status.

NOTE 4:	The Bicycle Sharing Service Provider can record the situation on its web-site that the car is being used.

- (Normal Reporting Case for managing the Service) The heart-rate of the user is continuously collected by the heart-rate sensor on the handlebar, and the health-related information such as heart-rate, location, time is reported periodically to the Service Operator.

The M2M Service Platform notifies the Bicycle Sharing Service Provider and the Health Service Provider of the health Service information.

(Urgent Reporting Case for handling any emergency) While riding the bicycle, the tire pressure sensor detects the low pressure of the front tire, the information is immediately sent to the M2M Service Platform via the Smartphone with location information.

The M2M Service Platform notifies the Bicycle Sharing Service Provider of the changed status.

The Bicycle Sharing Service Provider finds out the nearest bike repair shop according to the received location information, and the Provider sends the route information to M2M Service Platform.

The M2M Service Platform forwards the route information to the Smartphone which has a navigation application.

Afterwards, due to the low battery of the Smartphone (less than 30 % remain), the Smartphone reports the changed status to the Service Operator. (Low battery indication)

The M2M Service Platform automatically changes the subscription attributes of sensors and the Bicycle Service Provider such as delay tolerance to reduce battery-consumption.

EXAMPLE 3:	If the Platform changes the subscription attributes to "only emergency case", only emergency subscription case will be notified. The others cannot be notified, but at the end of service, will be sent in batch-mode.

As the user arrives at the destination and parks to the reserved spot, the Sensor reports the accumulated information including location information and normal event subscription information to the M2M Service Platform via the Smartphone.

- 13a)  After the use of bicycle finishs, the sensors may not send data any more. The smartphone stops reporting the changed status which is monitored via underlying network such as location, reachability, etc.

The M2M Service Platform notifies Bicycle Sharing Service Provide and Healthcare Service Provider of the usage of the shared bicycle.

The bicycle sharing provider may retrieve a location of bicycles to get information of the number of bicycles for each parking lots.

The M2M Service Platform notifies a location information depending on bicycle status either step 3a or step 13a.

Post-conditions

The User may pay as he/she drive according to the recorded data (not for the public service case).

The Bicycle Sharing Service Provider can update the position and status of the bicycle on its website using the recorded data. Thus, next users can make use of the Bicycle Sharing Service.

There are many parking lots to manage proper number of bicycles. Due to the bicycle sharing, certain parking lot may run out of bicycles or exceed. The Bicycle Sharing Service Provider can move cars to maintain a suitable number of bicycles among the parking lots.

    - Alternative Flow

None.

    - Post-conditions

See sub-case flows.

    - High Level Illustration

Figure 6.12.9-1: Car/Bicycle Sharing High Level Illustration

    - Potential Requirements

- The M2M System shall support integration of mobile/portable user owned equipment (e.g. smartphone) as M2M Gateway and/or Device.

The M2M System shall support distinguishing multiple event levels for reporting and handle them differently ([i.2] OSR-032).

NOTE:	For example, the event levels may be divided into normal and urgent events.

Based on the condition of the M2M Gateway and/or Device, the M2M System may change the reporting (or subscription) mechanisms and/or configurations related to a service ([i.2] OSR-033).

The M2M System shall support processing of access right requests to a resource and grant the requests if required conditions are met ([i.2] SER-071).

The M2M System shall provide capabilities  to request from the underlying network either the last known location information or current location information, if supported by the underlying networks.

  - Smart Parking

    - Description

Smart parking helps address one of the biggest problems on driving in urban areas; finding empty parking spaces and controlling illegal parking. Parking spaces are wide spread and owned by different providers so that it is not easy to access at one place/time.

With smart parking service, drivers can easily find available parking spaces, pay parking fees and even can make reservations. Making parking reservations would be available for limited people such as VIPs or the disabled, since ordinary parking service needs to satisfy first-come-first-served rule.

In this use case, law enforcement authority is also included as an actor. This implies M2M technologies supports rightness/safety as well as convenience.

    - Source

oneM2M-REQ-2013-0169R03 Use Case Smart Parking.

    - Actors

M2M Service Platform

This is a platform that interacts with M2M Gateways/Devices and M2M Application Service Providers.

Smartphone

This is a M2M Device acts as a car navigator and a wallet to pay parking fee by connecting parking meters.

On-street Parking Meter

This is a M2M Device installed near parking slots to charge drivers parking fees.

In-building Parking Sensor

This is a M2M Device with a small camera that can recognize a plate on cars, and is installed near disabled-only parking spaces.

Parking Provider

This is a M2M Application Service Provider who owns parking lots, in this use case there are two parking providers; in the mall and on street.

Billing Provider

This is a M2M Application Service Provider (e.g. financial institution) who provides billing service for M2M Users, e.g. for parking fees. When bills are issued by M2M Application Service Providers, coupons may be used for compensation schemes. This can also apply for fines issued by police centers.

Police Center

This is a law enforcement authority, one of M2M Application Service Providers, who charges fine to whom break laws.

User

This is a M2M service user who drives a car. In the second sub case, dedicated parking space, there are two users. One originally makes a reservation who is handicapped, and the other who illegally parks a car on disabled-only parking area.

    - Pre-conditions

See sub-case below.

    - Triggers

See sub-case below.

    - Normal Flow

(1) Finding Space, Parking Car & Paying Bill

Pre-condition

User sets a destination such as a shopping mall using the smartphone navigator, and also checks availability of parking space in the building before or while driving.

Triggers

The car approaches near the destination.

Figure 6.13.6-1: Normal Flow - Finding Space, Parking Car & Paying Bill

Normal Flow

- Since the user set the mall as the destination before, when the user is near the mall, the navigator sends the location to the mall parking provider automatically.

- The mall parking provider informs the navigator that there's no empty parking space now.

- Based on the car's location, which is near the mall, the mall parking provider inquires availability of other parking spaces through M2M service platform.

- There are empty spaces on street, so the mall parking provider recommends that parking space.

- The user approaches the smartphone from a parking meter to start parking. Then the street parking provider is notified, as well as the mall parking provider.

- The mall parking provider offers discount coupon for parking outside as compensation.

- The user touches the smartphone on the meter to finish parking.

- The street parking provider bills parking fee. The bill with discount coupon is sent to billing provider through M2M service platform.

(2) Dedicated Parking Space

Pre-condition

Before driving, the user (user A) makes a parking reservation for a slot in a shopping mall, which is especially for the disabled. It is normally assured because there will be fines for illegal parking on this dedicated parking space.

Triggers

None.

Figure 6.13.6-2: Normal Flow 1 - Finding Dedicated Parking Space

Normal Flow

- The other user (user B) parks a car on the parking lot, which is already reserved by user A.

- User B's illegal parking on the disabled-only parking area is reported to police center.

- Police center charges fine on user B.

- User A approaches the mall and notices that reserved parking space is taken and that only choice now is normal parking slots.

- User A parks on a normal parking slot instead of the reserved one.

- The parking provider offers parking discount coupon to the user A as a compensation.

- After shopping, user A leaves the building and finish parking.

- The parking provider bills parking fee for user A, applying the parking coupon.

    - Alternative Flow

Alternative Flow 1 - Dedicated Parking Space

Pre-condition

Before driving, the user (user A) makes a parking reservation for a slot in a shopping mall, which is especially for the disabled. It is normally assured because there will be fines for illegal parking on this dedicated parking space.

Triggers

None.

Figure 6.13.7-1 Alternative Flow 1 - Finding Dedicated Parking Space

- The other user (user B) parks a car on the parking lot, which is already reserved by user A.

- The mall parking provider inquires plate number of the car to a CCTV near the parking space.

- User B's illegal parking on the disabled-only parking area is reported to police center.

- Police center charges fine on user B.

- User A approaches the mall and notices that reserved parking space is taken and that only choice now is normal parking slots.

- User A parks on a normal parking slot instead of the reserved one.

- The parking provider offers parking discount coupon to the user A as a compensation.

- After shopping, user A leaves the building and finish parking.

- The parking provider bills parking fee for user A, applying the parking coupon.

    - Post-conditions

None.

    - High Level Illustration

Figure 6.13.9-1: High Level Illustration of Smart Parking

    - Potential Requirements

- The M2M System shall support mechanisms to correlate or compensate charging data/records from different M2M Application Service Providers.

- The M2M System shall support triggering of M2M Devices to report collected data on-demand.

  - Vehicle Broadcasting without Registration

    - Description

This use case consists for any vehicle driving fast to signal its emergency state which can be for example hard braking state or abnormal vehicle state to other following vehicles or infrastructure, such as a road side unit in its vicinity.

By broadcasting time limited periodic messages which indicate vehicle's emergency state triggered by an event, the vehicle can warn unspecified entities of a sudden danger so limiting the risk of collision. The infrastructure can collect those information and relay them to other entities or servers in the distance.

Since a vehicle is a very quick-moving object and the transferred message is a time-critical information, the transmission speed is crucial for this use case.

    - Source

REQ-2016-0031R03 Vehicle Broadcasting without Registration TR-0026.

    - Actors

Source Vehicle

A Source Vehicle is a moving object which provides communications functions necessary to support connected vehicle operations. It can detect its emergency state and send warning message by broadcasting.

Target Vehicle

A Target Vehicle is a moving object which provides communications functions necessary to support connected vehicle operations for communicating with the Source Vehicle. After receiving warning message, it tries to avoid the potential danger.

Infrastructure Node

An Infrastructure Node is located by a road and provides communications functions necessary to support connected vehicle operations. It can collect information generated in a certain area and partially process it. It is also possible to convey it to the server or other vehicles in a far distance.

    - Pre-conditions

N/A.

    - Triggers

When a vehicle application detects its emergency state, it should trigger broadcasting specific information.

    - Normal Flow

Figure 6.14.6-1 Normal Flow - Vehicle Broadcasting without Registration

0)	Each Vehicle and Infrastructure Node do not register with each other.

- Source Vehicle runs event detection engine and analyses whether the safety-related event has occurred or not.

- Source Vehicle send the safety-related information to unspecified entities by broadcasting.

3.1)	Infrastructure Node gathers the broadcasted information and locally processes and relays to other vehicles or servers in a distance if needed.

3.2)	Target Vehicle receives the safety-related information and re-acts to avoid the potential collision.

    - Alternative Flow

N/A.

    - Post-conditions

N/A.

    - High Level Illustration

Figure 6.14.9-1 High Level Illustration - Vehicle Broadcasting without Registration

Source Vehicle can send time-critical information to unspecified entities.

Infrastructure Node can relay this information to the server which can also re-use that information.

    - Potential Requirements

The oneM2M system shall enable the M2M Infrastructure to facilitate direct communication between two or more different M2M devices without having registered with one another.

  - Vehicle location privacy protection

    - Description

The privacy protection is an issue that is considered for locating in human to human communications. For example, in 3GPP network, privacy protection is implemented by an independent entity such as a privacy profile register (PPR) or a gateway mobile location center (GMLC). In the vehicle domain, the location information of the vehicle represents the location of the vehicle user. Therefore an issue of privacy protection is also closely related to the vehicle location information. The privacy protection refers to that when a vehicle is connected to the M2M system, the user of the vehicle has permission to specify when and where a third party application is allowed to access to the vehicle location information.

After the M2M platform receives a message from a third party application, requesting for the location information of a certain vehicle, the M2M platform determines the entity that should performs the privacy inspection, according to the source of the location information. For example, if the location information of the vehicle is network based and the underlying network is 3GPP, then the entity that performs the privacy inspection should be the 3GPP location server. On the other hand, if the location information of the vehicle is device based, then it should be the M2M platform that performs the privacy inspection.

    - Source

REQ-2016-0040R02-TR-0026-Vehicle_location_privacy_protection.

    - Actors

M2M Device: It is embedded in a vehicle, which is used to connect to the M2M platform.

Vehicle: Vehicle connects to the M2M platform via the embedded M2M device and it is equipped with hardware devices for localization. (e.g. GPS, Cellular modem).

Vehicle Owner: The owner of the vehicle. The owner has privacy agreement with the 3GPP network location server or the M2M platform.

3GPP network location server: When the vehicle connects to the M2M platform via the 3GPP underlying network, the 3GPP network location server can perform the location service and provide the location information to the M2M platform. In this case the 3GPP network location server also performs the privacy inspection function. The 3GPP network location server is configured with location privacy policies according to the privacy agreement with the vehicle owner and the network operator policy. The location privacy policies refer to when and where a third part application is allowed to access to the vehicle location information.

M2M Platform: It connects, manages M2M devices and exposes location services to the M2M application. The M2M platform can collect the vehicle location information either from the underlying network, e.g. 3GPP network location server, or from the device itself, e.g. the vehicle is equipped with any location capable modules or technologies (e.g. GPS) and is able to position itself. The M2M platform is configured with location privacy policies according to the privacy agreement with the vehicle owner. The location privacy policies refer to when and where a M2M application is allowed to access to the vehicle location information.

M2M Application server: The M2M application server requests for the location information of the vehicle for any possible application usage, e.g. fleet management or traffic monitoring.

    - Pre-conditions

The vehicle owner already has privacy protection agreement with the operator of the 3GPP network or the M2M platform.

    - Triggers

The M2M application server requests to access to the location information of the vehicle.

    - Normal Flow

Scenario 1: The vehicle No.1 connects to the M2M platform via the 3GPP network and the location information is collected by the 3GPP network location server:

- The M2M application sends message to the M2M platform to request for the location information of vehicle No.1.

- The M2M platform checks the location information source of the vehicle. If the location information of the vehicle comes from the underlying network location server, e.g. 3GPP network location server, the M2M platform sends privacy inspection requests, including the M2M application identifier which can be recognized by the 3GPP network, to 3GPP network location server to verify whether the M2M application is allowed to access to the location information.

- The 3GPP network location server is configured with the privacy policies of vehicle No.1. When the 3GPP network location server receives the access right verify request, it performs the privacy inspection, checking the privacy policies with the M2M application identifier and other information, e.g. the time and the lasted location of the vehicle. The 3GPP network location server returns the checking result to the M2M platform, that is whether the M2M application is allowed to access to the location information.

- If the M2M application is allowed to access to the location information, after the M2M platform receives the responses from 3GPP network location server, it sends the location information request message to the 3GPP network location server.

- The 3GPP network location server performs the location updating procedure inside the 3GPP network and collects the latest location information of the vehicle. Then the 3GPP network location server returns the location information to the M2M platform.

- The M2M platform forwards the location information to the M2M application.

Scenario 2: The vehicle No.2 connects to the M2M platform via the 3GPP network. It is equipped with location capable modules or technologies (e.g. GPS) and is able to position itself. The M2M platform can collect the location information of the vehicle directly without the help of any other location server.

- The M2M application sends message to the M2M platform to request for the location information of vehicle No.2.

- The M2M platform checks the location information source of the vehicle. If the location information of the vehicle is device based and is collected by the M2M platform directly from the vehicle itself, then the M2M platform checks the privacy policies configured in the M2M platform.

- If the M2M application is allowed to access to the location information, then the M2M platform returns the location information to the M2M application. If the M2M application is not allowed to access to the location information, then the M2M platform sends failure response to the M2M application with appropriate error code.

    - Alternative flow

N/A.

    - Post-conditions

N/A.

    - High Level Illustration

M2M application

Vehicle1

Vehicle2

GPS module

underlying network

Underlying network location server

privacy

M2M platform

privacy

Figure 6.15.9-1: Vehicle location privacy prtection high-level illustration

    - Potential requirements

- The oneM2M system shall be able to support the enforcement and management (e.g. update) of the privacy policies for vehicle location information.

- The oneM2M system shall be able to apply the privacy policies configured in the oneM2M system to those vehicles whose location information is obtained by the oneM2M system itself.

- The oneM2M system shall be able to request the underlying network to perform, on behalf of the oneM2M system, the privacy policy decision concerning the location information if the location information is obtained by the underlying network location service ([i.2] SER-062 and SER-063).

  - Vehicle Domain service continuity

    - Description

Autonomous or self-driving cars have been gaining attention as early versions of such vehicles have become available. A gateway on such a vehicle, may be responsible for movement control (braking, turning, etc.) as well as delivering external services. Such a scenario requires real time monitoring of sensors and fast access to various actuators for accident avoidance. For example, time series data may be gathered which triggers specific actions when data is lost. In such cases service continuity and data transfer of time-critical information are essential.

    - Source

REQ-2016-0052R02-TR-0026_service_use_case.

    - Actors

Vehicle

A Source Vehicle is a moving object which provides support for services necessary for vehicle operations.

Road-Side Unit (RSU)

A Road-side unit is located along vehicular paths and provides localized support for services necessary for multiple vehicle operations. This support is provided for vehicles within the RSU's communication range.

Infrastructure Node

An Infrastructure Node provides centralized support for services provided by a Services Provider to a large number and variety of devices.

    - Pre-conditions

All RSUs are registered to the Infrastructure Node.

    - Triggers

When a vehicle moves it loses connectivity to one road-side unit, and then re-connects to different road-side units along the way.

    - Normal Flow

The Normal Flow depicts the case where a moving vehicle receives notifications.

Figure 6.16.6-1: Normal Flow for Vehicular Domain Service Continuity

0)	Vehicle establishes communication with RSU 1.

- RSU 1 creates a subscription to a resource hosted on the Infrastructure Node on behalf of the Vehicle.

- A notification corresponding to the subscription in step 1 is generated and sent to the Vehicle via RSU 1.

- The vehicle moves, and establishes communication with RSU 2.

- Supporting interactions between RSUs and Infrastructure Node.

- A notification corresponding to the subscription in step 1 is generated. The notification is sent to the Vehicle via RSU 2.

    - Alternative flow

The Alternative Flow depicts the case where a moving vehicle reports observed road/traffic conditions (e.g. vehicles in proximity and their speed, distance to curb, etc.) and receives collision avoidance commands. The RSUs collect and store the information from the vehicles, and also exchange information with each other. This allows the RSUs to better identify the trajectory of vehicles and to help predict potential collisions. As collision avoidance decisions are extremely time sensitive, all decisions are made locally at the RSU. This avoids any unnecessary delays incurred with communication to the infrastructure node.

Figure 6.16.7-1: Alternative Flow for Vehicular Domain Service Continuity

0)	Vehicle establishes communication with RSU 1.

- Vehicle sends road/traffic data to RSU 1.

- The vehicle moves, and establishes communication with RSU 2.

- RSU 2 retrieves traffic data related to vehicle from RSU 1.

- Vehicle sends road/traffic data to RSU 2.

- Collision avoidance algorithm determines that a collision is imminent (based on vehicle data from RSU 2 and from RSU 1)

- RSU 2 sends a collision avoidance command to the vehicle.

    - Post-conditions

N/A.

    - High Level Illustration

Figure 6.16.9-1: High Level Illustration Vehicular Domain Service Continuity

    - Potential requirements

- The oneM2M system shall enable continuity of services to M2M devices as they move across various geographic points in the oneM2M system ([i.2] OSR-099).

  - Optimal Speed Recommendation

    - Description

The target vehicle is approaching the traffic light that is currently displaying red.

A local RSU station or the central server connected with local vehicles, traffic lights and other entities such as weather server, map server and remote vehicles collects the vehicle's precise location and the traffic light phase schedule as well as additional information such as weather, road shape, emergency situation etc. of the front road where the vehicle is heading to.

The local RSU station or the central server then calculates a specific optimal speed for the vehicle based on the collected information to reach the traffic light at the beginning of the next green phase and if necessary to avoid potential dangers.

The resulting speed is finally delivered to the driver who can avoid the unnecessary stop at the traffic light and unforeseeable hazards.

    - Source

REQ-2016-0067R02-new_Vehicle_usecase_TR-0026.DOC.

    - Actors

Target Vehicle

A Target Vehicle is a moving object which periodically provides its movement information such as time, location, velocity, heading, lateral and longitudinal acceleration to the central server so that it can utilize the information to calculate the optimal speed. After the calculation in the server, the Target Vehicle receives back the result directly from the server.

Traffic Light

A Traffic Light is located by a road and provides its phase schedule necessary for the server to calculate the optimal speed of the target vehicle.

Local RSU station or Central Server

A local RSU station or Central Server calculates the optimal speed for the target vehicle based on the gathered necessary local information from various sources including the target vehicle, the traffic light, other information servers and remote RSUs, central server, or vehicles.

Information Servers

There could be many kinds of additional Information Servers that can be connected with and utilized by the local RSU station or central server. For example, a weather server provides local weather data such as time, location, external air temperature, snow, rain and etc. and it is possible to detect potential road hazards via a map server.

Remote Vehicles

Remote vehicles automatically or manually report emergency situation to the local RSU station or server.

    - Pre-conditions

N/A.

    - Triggers

When the local RSU station or central server detects that the target vehicle has approached a few hundred meters (It should be more than 300 meters in order to safely adjust the current speed) in front of the traffic light, the server calculates the optimal speed and sends it to the target vehicle.

    - Normal Flow

Figure 6.17.6-1 Normal Flow - Optimal Speed Recommendation

- As the first step, associated entities register to the ITS system and communicate with the local RSU station or central server except remote vehicles which do not necessarily need to communicate because they are moving object and this scenario works based on local information. It is enough to know just the location of the remote vehicle and what happens around it when it is in the vicinity of the traffic light.

- The local RSU station or central server gathers information to calculate the optimal speed of the target vehicle.

2.1)	The target vehicle periodically provides its movement information including heading, time, location, velocity, lateral and longitudinal acceleration.

2.2)	The traffic light periodically provides its phase schedule when to be red, yellow or green.

2.3)	Additional information server provides further useful data as requested by the local RSU station or central server. Information such as slippery zone, high accident frequency location, or sharp curve could be examples.

2.4)	Remote vehicles can also provide more information to the server in case an accident or something dangerous happens. This can be done automatically as programmed or manually by drivers. Note that the local RSU station or central server does not communicate with remote vehicles since it's impossible to predict which vehicles may have necessary information. Therefore remote vehicles should initiate communication with the local RSU station or central server only when necessary, typically when they reach the vicinity of the traffic light.

When the target vehicle has reached a specific area (It could be 300m in front of the traffic light) ahead of the traffic light, the local RSU station or central server starts calculating the optimal speed for the target vehicle to pass through the traffic light with no stop based on the collected information. If any additional information indicates there could be safety risk, the local RSU station or central server should consider them as well for the calculation.

The final outcome from the server is delivered to the target vehicle and shown to the driver who can follow the speed recommendation.

    - Alternative flow

N/A.

    - Post-conditions

N/A.

    - High Level Illustration

Target vehicle

Remote vehicles

Central server

Additional server

Figure 6.17.9-1 High Level Illustration - Optimal Speed Recommendation

Solid lines in red are mandatory while dashed lines in grey are optional for this use case.

    - Potential requirements

- The oneM2M system shall support real time data transmission for moving object to have enough time to react to the received information.

- The oneM2M system shall be able to verify geographical location information from moving objects regardless of information accuracy.

- The oneM2M system shall be able to verify time synchronization among multiple sources that provide inputs for a single output.

- The oneM2M system shall be able to guarantee network stability and reliability for applications that can have safety impacts

- The oneM2M system shall apply high-level security (e.g. protection from malicious hacker) for applications that can have safety impacts.

- The oneM2M system shall provide privacy protection mechanism at the central server.

  - Autonomous driving

    - Description

Autonomous driving (AD) refers to the capability of the vehicle to drive from one location to another, without intervention from humans, and in a safe way, without incurring damage to surroundings (pedestrians, buildings, other vehicles) and to its (vehicle's) passengers.

Different levels of automation are shown in Table 5.4-1. From top to bottom we can see increasing levels of automation, with 'Full automation' (level 5) being autonomous driving.

For autonomous driving, it is essential for vehicle to have full information on its own state and state of its surroundings. In that respect data that is lost, or which arrives with large delay, is introducing uncertainty in the vehicle's model of the surroundings. This uncertainty will typically result in vehicle taking appropriate measures to deal with it, typically decreasing the speed of the vehicle, or stopping it altogether. Reasoning behind this is that for safety critical applications, safety is in the first place. Modern vehicles are equipped with number of sensors which are increasing comfort, fuel efficiency and safety of vehicles. That is done by using a variety of sensors that measure and collect data about vehicle (for example - ABS sensors, brake switch, speed, location, etc.), but are also equipped with sensors to observe state of its surroundings - using radar, camera, short range distance measurement sensors, LIDAR, etc.

Further, the roads and surrounding infrastructure are becoming more instrumented with sensors and are able to communicate. The possibility of interconnecting infrastructure sensors (cameras, traffic light radars, road sensors , etc.) and thus exchanging data with vehicles may lead to new ways to design autonomous vehicle systems, thereby reducing cost, while increasing robustness and reliability of autonomous driving.

Other connected objects (pedestrian's smartphones, for example) may also act as additional sources of data for autonomous driving vehicle, thereby contributing to improved efficiency, accuracy and safety of the Autonomous driving functions.

System supporting combining all these sources of data will enable pushing the driving automation to the higher levels of automation (as described in clause 5.4), ultimately to one where the driver is out of the (control/driving) loop. Furthermore, by making autonomous cars an IoT entity, this will enable larger groups of developers to create IoT/AD services.

    - Source

REQ-2017-0001R03-Autonomous_driving.

    - Actors

Vehicle Driver:

Driver sits in its normal (driving) position, and is in position to take corrective action (level 3, level 4) when prompted to do so by the autonomous driving system. There is no vehicle driver at level 5 of automation.

IoT platform provider

It operates an IoT platform which is collecting data from vehicles, other participants in traffic (pedestrians, cyclists), from roads and associated infrastructure (traffic lights, cameras, etc.).

Autonomous driving application provider

Party that is providing Autonomous driving application (ADApp) which runs on AS (Application Servers). AS connects to the IoT platform, and from there it collects relevant data needed to run an ADApp - for example LDM (Local Dynamic Map).

Communication Network provider/operator

Provides and facilitates connectivity between vehicles, roads and associated infrastructure. This covers both wireless (for example LTE, LTE-V2X, ITS-G5 and WAVE) and fixed connections. The Network supports receiving requests for data transfers with required latency and with required packet losses. It is not expected or mandated that single network operator provides all of connectivity.

IoT Device

IoT devices are embedded in a vehicles, roads and associated infrastructure, as well as in devices used by other participants in traffic (pedestrians, cyclists). Each IoT device collects and sends data to IoT platform, and can receive data from platform.

    - Pre-conditions

The vehicle supports autonomous driving - meaning it is capable of transmitting and receiving data from other vehicles, road and other infrastructure, other participants in traffic (pedestrians, cyclists), and based on own collected data and received data from ADApp on making decision and acting upon it.

    - Triggers

Autonomous driving mode can be activated by vehicle driver, or it can be activated by default. How the HMI (Human Machine Interface) works and which commands are given to the vehicle is out of scope of the present document.

    - Normal Flow

- The vehicle continuously collects data from sensors within the vehicle and sends it to the IoT platform. The collected data includes information from internal state of the vehicle (ABS status, brake switch, accelerator pedal, etc.) as well as data on surroundings of the vehicle (radar, LIDAR, cameras, etc.).

- Road infrastructure (roads, traffic lights, cameras, etc.) continuously collects data from its sensors and sends it to the IoT platform.

- Devices belonging to other participants in traffic (pedestrians, cyclists, etc.) continuously collect data from its sensors and send it to the IoT platform.

- The IoT platform hosts the collected data from abovementioned sources, and upon request it will send it to AS (Application Server) where Autonomous driving application (ADApp) is running. ADApp is subscribed to the IoT data from all participants in traffic. Note that in order to protect privacy of participants in traffic, the accessibility of the collected data has to be sufficiently restricted and access to collected data is covered by regional regulatory obligations.

- Processed information from ADApp is sent back to IoT platform, and is made available to all ADApp subscribed participants in traffic.

- Each participant in traffic is responsible for interpretation and action based on received ADApp data.

    - Alternative Flow

None.

    - Post-conditions

Vehicle stays in autonomous driving mode until removed from it by driver.

    - High Level Illustration

IoT platform

Application

Platform

(ADApp)

V2V/V2I communication (LTE-V, ITS-G5)

Cellular communication (LTE, 5G)

Fixed connectivity

Figure 6.18.9-1: Example of IoT data streams and corresponding communication networks

    - Potential Requirements

- The M2M system shall support the data to be transmitted to IoT platform with strict timing and packet loss requirements, determined by application ([i.2] CMR-016).

The M2M system shall support the data to be transmitted from IoT platform to subscribed devices with highest priority, with strict timing and packet loss requirements, determined by application ([i.2] CMR-017).

For each source of relevant autonomous driving data which is sent to the IoT platform, the oneM2M System shall be able to detect and report the missing data in time series. For each vehicle receiving data from ADApp, the oneM2M System shall be able to detect and report the missing data in time series ([i.2] CMR-018 and CMR-019).

  - Accident Notification using Edge/Fog Traffic Monitoring Service

    - Description

A Traffic Monitoring Service based on data collected by vehicular on-board cameras/sensors and surveillance cameras (e.g. video camera, radar, LIDAR, GPS) can provide functionality such as: location, detection of traffic accidents, video capture with timestamp, etc.. The generated data is used for example to notify Drivers about accidents and let them take a detour. An Application Provider would like to process this data and provide it to the Driver as soon as possible in order to avoid traffic congestion.

In this use case, an Edge/Fog architecture is used to lower the processing burden on the Cloud Nodes. The Edge/Fog Nodes receive data from vehicles and perform following processing:

Analysing the accident site situation from video data.

Processing video (e.g. editing) and generating video clips with high quality and low quality.

Using pool-based functionalities for sharing/scaling with other nearby Edge/Fog Nodes.

Acquiring appropriate Edge/Fog Node information from Cloud Nodes in order to provide the accident information.

Analysing vehicular metrics and network bandwidth of each vehicles, and sends the appropriate video clip (high or low quality) to vehicles.

The system is designed to mitigate the burdens on Cloud Nodes and core networks, optimise data, and improve system reliability by using Edge/Fog Nodes.

    - Source

REQ-2018-0008R02 Use case for Accident Notification using Edge/Fog traffic Monitoring Service

    - Actors

Cloud Nodes: It is the Nodes which manage Edge/Fog Nodes, maintain database of Edge/Fog Nodes and interacts with Application Provider.

Application Provider: It provides its own services for vehicles.

Edge/Fog Node: It is the Node which computes, stores and analyses data. It is located between Cloud Nodes and end devices.

RSU: It is located along vehicular paths and provides connection between vehicles and Edge/Fog Node in a RSU network.

Driver: It is a user who drives a vehicle.

Surveillance camera: It is located along vehicular paths, captures the path and sends the captured video data to Edge/Fog Node via RSU.

    - Pre-conditions

Drivers subscribe to an Accident Notification Service of Application Provider.

Vehicles are equipped with on-board video camera and sensors for vehicular surrounding monitoring.

    - Triggers

A vehicle clashes with an oncoming vehicle. They can’t send the accident information to an Edge/Fog Node from the accident site automatically due to serious damage to their communication units.

    - Normal Flow

- Vehicular on-board cameras/sensors and surveillance cameras near the accident site recognize the accident. They start to collect video clips with timestamp and location data near the site.

- The vehicles and the surveillance cameras send the video clips and the location data to Edge/Fog Node A via RSU.

- Edge/Fog Node A receives the data and analyses the accident site situation from the data, and processes the video  (e.g. editing)  and generates video clips with high quality and low quality.

- Based on the received system metric and diagnostic information, if Edge/Fog Node A’s video processing capability is not enough or does not work properly, pool-based functionality sharing and scaling with other nearby Edge/Fog Nodes can be utilized.

- Edge/Fog Node A gets the appropriate Edge/Fog Node information from Cloud Nodes in order to provide the accident information.

- Based on the received the Edge/Fog Node information, Edge/Fog Node A sends the video clips with accident information to Edge/Fog Node B.

- Edge/Fog Node B analyses network bandwidth of each vehicles near the accident site together with the system metrics, and sends the appropriate video clip (high or low quality) to vehicles of the coverage area for Edge/Fog Node B via an RSU.

- The Vehicles receive the video clip. Each driver knows accident by in-vehicle infotainment and decides a detour.

Figure 6.19.6-1: Normal Flow - Accident Notification

    - Alternative Flow

None

    - Post-conditions

Drivers can avoid traffic congestion and arrive at their destination as their planned.

    - High Level Illustration

Figure 6.19.9-1: High Level Illustration - Edge/Fog Traffic Monitoring Service

    - Potential requirements

- The oneM2M System shall enable pool-based functionality sharing and scaling between Edge/Fog Nodes.

The oneM2M System shall be able to trigger priority services from the underlying network (e.g.. 3GPP MPS).

The oneM2M System shall enable detection of network bandwidth between Edge /Fog Nodes and M2M devices in order to provide necessary quality of service according to the bandwidth.

The oneM2M System shall enable Edge/Fog Nodes to provide system metrics and diagnostic information to other Edge/Fog Nodes, as required to ensure reliable operations within the oneM2M System.

The oneM2M System shall enable Edge/Fog Nodes to alert other suitable Edge/Fog Nodes that are unable to perform specific services.

  - High-precision Road Map using Edge/Fog Computing

    - Description

This use case introduces a High-precision Road Map Service based on data collected by vehicular on-board cameras/sensors, surveillance cameras (e.g. video camera, radar, LIDAR, GPS), and V2X data from mobile core network. Vehicular on-board cameras/sensors and surveillance cameras collect surrounding data periodically and send them to local Edge/Fog Node. Edge/Fog Node cloud processes the data, generates differential data of existing High-precision Road Map, and provide the data to vehicles in real time.

The High-precision Road Map consists of static data (map data) and dynamic data (e.g. traffic accident, traffic congestion, weather condition, traffic signs, etc.). This concept is based on ETSI TR 102 863 “LDM (Local Dynamic Map)” [i.21]. The differential data generated by Edge/Fog Node corresponds to dynamic data.

The Application Provider would like to process data and provide information to the Driver as soon as possible, thus the service is required to support low latency.

In this use case, an Edge/Fog architecture is used to lower the processing burden on Cloud Nodes. The Edge/Fog Nodes receive data from vehicles and the mobile core network, and perform following processing:

In order to minimize the amount of processing required by the vehicles and the Cloud Nodes, the Edge/Fog Nodes can receive the data from vehicles and perform pre-processing for the vehicles, the Cloud Nodes and other Edge/Fog Nodes. Pre-processing by Edge/Fog Nodes may entail the Edge/Fog node optimizing data that has been collected from vehicles. For example, if vehicles are located in a traffic congestion, the vehicle may send sensor information (e.g. video camera, radar, LIDAR, GPS) to Edge/Fog Node. The Edge/Fog node may detect that some of the data sets are similar and perform data aggregation before sending the data to the Cloud Nodes and other Edge/Fog Nodes. In addition, as the vehicles move, they may store their sensor information in different Edge/Fog nodes – resulting in the sensor information being fragmented. Unfortunately, some applications will require that Edge/Fog nodes have functionality to resolve this data fragmentation. This functionality includes discovery of the sensor information, their retrieval and combination. This functionality may also require: checking for missing sensor readings, integrity check of periodic data (for example for data that is periodic, the Edge/Fog node may validate that data is received when expected), removal of old or redundant data.

In order to minimize the amount of data that needs to be sent from the vehicles and to minimize the amount of processing required by the Cloud Nodes, the Cloud Nodes and/or Edge/Fog nodes may obtain some context information from the mobile core network. Cloud Nodes and/or Edge/Fog nodes may use services that are exposed by the mobile core network to obtain context and create a geographical context for the vehicles, e.g. what networks are accessible at the current location and their congestion level.

Edge/Fog Nodes may compare the collected data with existing High-precision Road Maps and abstracting differential data before sending post-processed information to the Cloud Nodes and other Edge/Fog Nodes.

Edge/Fog Nodes may categorize the differential data into high priority data (e.g. traffic accident, traffic congestion, weather condition) and low priority data before sending post-processed information to the Cloud Nodes and other Edge/Fog Nodes.

Edge/Fog Nodes may incorporate the high priority data into High-precision Road Map.

Data synchronization between Edge/Fog Nodes.

The use case intends to mitigate the burdens on Cloud Nodes and optimises the system by moving processing to the Edge/Fog Nodes and leveraging the capabilities of the Mobile Core Network.

    - Source

REQ-2018-0010R02 Use case for High-precision Road Map Service using Edge/Fog Computing

    - Actors

Cloud Nodes: It is the Nodes which manage Edge/Fog Nodes, maintain database of Edge/Fog Nodes, store master data of High-precision Road Maps and interact with Application Provider.

Application Provider: It provides its own services for vehicles.

Edge/Fog Node: It is the Node which computes, stores and analyses data. It is located between Cloud Nodes and Mobile Base Station.

Mobile Base Station: It provides the connection between vehicles and Edge/Fog Node in a mobile network.

RSU: It is located along vehicular paths and provides connection between vehicles and Edge/Fog Node in a RSU network.

Driver: It is a user who drives a vehicle.

    - Pre-conditions

Drivers subscribe High-precision Road Map Service of Application Provider.

Edge/Fog Node A and Edge/Fog Node B store High-precision Road Map data of each coverage area.

Vehicles are equipped with on-board video camera and sensors for vehicular surrounding monitoring and High-precision Road maps system.

Cloud Nodes and/or Edge/Fog nodes uses services that are exposed by the mobile core network to obtain context and create a geographical context for the vehicles, e.g. what networks are accessible at the current location and their congestion level.

    - Triggers

None.

    - Normal Flow

- Vehicles in the coverage area of an Edge/Fog Node A collect surrounding data by using vehicular sensors (e.g. video camera, radar, LIDAR, GPS).

Vehicles send the collected data to Edge/Fog Node A via RSU.

Edge/Fog Node A processes the data as follow.

If the underlying network exposes services for gathering context, it obtains context and creates a geographical context for the vehicles it communicates with, e.g. what networks are accessible at the current location and their congestion level.

Optimizes the collected data from vehicles (e.g. aggregation, stale or redundant data identification and removal, integrity check, validation, etc.).

Compares the data with existing High-precision Road Map in Node A's coverage area and abstracting differential data.

Categorizes the differential data into high priority data (e.g. traffic accidents, traffic congestions) and low priority data.

Incorporates the high priority data into the existing High-precision Road Map.

Edge/Fog Node A sends the differential data to Edge/Fog Node B with timestamps for data synchronization between Edge/Fog Node A and Edge/Fog Node B. The communication with Edge/Fog Node B may be dependent on the geographical context for the vehicles, i.e. networks accessibility and their congestion level.

Edge/Fog Node B sends the differential data to vehicles in Edge/Fog Node B’s coverage area.

Vehicles in Node B’s coverage area update High-precision Road Map data of in-vehicle navigation system.

Cloud Nodes acquire latest differential data from all Edge/Fog Nodes once a day and incorporate the differential data into existing High-precision Road maps and store as master data.

Figure 6.20.6-1: Normal Flow - High-precision Road Map Service

    - Alternative Flow

None

    - Post-conditions

Drivers know high priority events  (e.g. traffic accident, traffic congestion, weather condition) by updated High-precision Road Map and takes appropriate direction.

Edge/Fog Nodes and Cloud Nodes manage latest High-precision Road Map.

    - High Level Illustration

Figure 6.20.9-1: High Level Illustration -  Edge/Fog Computing for High-precision Road Map

    - Potential requirements

- The oneM2M System shall enable data continuity services to be provided between Edge/Fog Nodes by enabling the discovery, retrieval, and combination of data sets dispersed across the Edge/Fog network.

The oneM2M System shall enable data optimization services to be provided at Edge/Fog Nodes including aggregation, stale or redundant data identification and removal, integrity check, validation, etc. even if the data sets are dispersed across the Edge/Fog network.

The oneM2M System shall enable categorization of the data collected by M2M devices  (e.g. high priority data, low priority data) for differential delivery and processing.

The oneM2M System shall enable timestamp synchronization of the data collected by M2M devices between Edge/Fog Nodes for data synchronization.

The oneM2M System shall enable services to receive and utilize location-based information about available access networks, their congestion level and other related network information, when the information is provided by the Underlying Network.

The oneM2M System shall enable differential processing of data at different nodes, e.g.  Edge/Fog node , Cloud node.

  - Smart Transportation with Edge/Fog computing

    - Description

Fog/edge computing technologies are very suitable for enabling smart transportation because they enable a dynamic and localized environment for deploying services potentially closer to the users and/or data sources. Service providers using fog and edge deployments may actually not instantiate all their available services and in all Fog nodes, rather they may do so when there are users who actually require to use the services in the area serviced by the respective Fog node.

Smart traffic systems include a multitude of devices and sensors and may provide a variety of services. In this case consider the service of delivering a passenger to a particular destination using a self-driving car. As part of the transportation service, the user may also request other services such as stopping at a particular point of service e.g. coffee shop, finding a parking spot at the destination, and getting notifications about an eminent emergency service (See Figure 6.21.1-1). For example, the passenger in the vehicle is watching a movie using a streaming service. The provider knows the destination and the route of the vehicle. Since the vehicle remains under one Fog node’s area only for a certain period of time, it is unnecessary to have the full movie in one Fog node. Instead, the data, in this case video streams, can be prepared across the route of the vehicle up until the destination. The Fog nodes in these areas may be requested to instantiate the service ahead of time and also to provide enough information to predict  when the vehicle will be under a Fog node’s coverage and how much data should be buffered ahead of time. In this use case, it is assumed that the Fog node has roadside units (RSUs) connected to it via which the data is sent to the vehicle.

Figure 6.21.1-1: Normal Flow - Smart Transportation Use Case

In order to provide this functionality, the platform performs:

- Capability status report, which is used by Fog/Edge node to report the status of the “things” that are under their control and from which relevant data can be extracted. For example, a camera installed in a particular street intersection for smart transportation service may stop working and other Fog nodes or services on the controlling Fog nodes need to be updated about this status so that other data sources can be used to maintain the smart transportation service.

- Fog/Edge capability discovery, which is used by Cloud nodes or Fog/Edge nodes to understand the capabilities of neighboring nodes, other Fog/Edge nodes, and/or Cloud nodes at a different level in the Fog/Edge deployment hierarchy.  This also implies a common understanding of the  Fog/Edge capabilities per service or in aggregate, as a profile.

- Reachability verification, which is used by Cloud nodes or Fog/Edge nodes to verify whether  Fog/Edge nodes of interest are still reachable in order to communicate with them when needed.

- Service migration and continuity, which is needed by the Fog/Edge nodes to interact with each other to provide or receive services, e.g. to request video feed from a camera that is under the control of a neighboring Fog node.

- Orchestration of the services provided by Fog/Edge nodes in a dynamic fashion, in order to satisfy operational requirements for availability, scalability, interoperability, etc.

- Service provisioning, which is used by Cloud nodes or Fog/Edge nodes to provide other Fog/Edge nodes with the required functionality to instantiate a service.

- Service differentiation: The Edge/Fog nodes can choose to pass-through traffic to other Edge/Fog nodes or to Cloud nodes based on service type, devices, network conditions, available capabilities, etc.

- Analytics: The Edge/Fog nodes need to be able to pull data from different sources and take actions based on service policies.

    - Source

REQ-2018-0018R02- Smart Transportation with Edge/Fog deployment

    - Actors

User: It is the person who drives a vehicle or passengers who ride the vehicle.

Smart Transportation Service: A service through which the driver or the passengers in a vehicle can leverage services and capabilities provided by Fog/Edge deployment.

Cloud Node: A node with cloud or fog capabilities that manages the operations of other Fog nodes lower in the deployment hierarchy. The Cloud Node interacts with service providers and has service layer agreements and can authorize requests from Fog nodes. Furthermore, the Cloud Node may oversee and manage the interactions between different Fog nodes that together enable a fog service layer.

Fog Node: A node that has any fog resource such as compute, storage, communication, analytics, etc.. For the smart transportation use case, a Fog Node is assumed to be deployed at one level higher than the level of a Local Fog Node. There may be several levels of Fog Node deployments with the Cloud Node being at the highest level. It also assumed that a Fog Node connects to at least one Local Fog Node and that the former also manages at least one Local Fog Node.

Local Fog Node (LFN): A Fog node with numerous capabilities and services that together with other LFNs and the cloud create a fog service layer that provides services for authorized applications. Moreover, the Local Fog Node is connected to more than one Fog Entities that are located in the service area of the Local Fog Node. The Local Fog Node, together with other neighboring Local Fog Node, appear as a Fog Service Layer to the applications that are hosted on the Local Fog Node. In the smart transport use case, the Local Fog Node is located between the Fog Entity and the Fog Node or Cloud Node. The Local Fog Node is assumed to be managed by a Fog Node or a Cloud Node that is one level above the Local Fog Node in the fog deployment hierarchy

Fog Entity (FE):  A node that has a sub-set of the capabilities of a Fog Node. Moreover, a Fog Entity connects to a Local Fog Node (see below) and hence becomes a source of data and service for the Local Fog Node. In addition, the Fog Entity may also request and consume data or services from the Local Fog Node. In the smart transport use case, a Fog Entity may be a sensor, a camera, a traffic light, or any IoT device with basic (relatively small) compute and storage resources. Moreover, the Fog Entity is deployed behind or below the Local Fog Node.

    - Pre-conditions

Passengers subscribe to Service Provider services

Fog nodes provide services to the vehicles

Vehicles are equipped with devices accessing the services

    - Triggers

None.

    - Normal Flow

Figure 6.21.6-1 illustrates the high-level flows of smart transportation use case, which consists of the following steps:

- Step 1: Fog capability status report where capabilities and other context information about a local Fog node are reported to or collected by other local Fog nodes, Fog/Edge nodes, and/or Cloud nodes.

- Step 2: Fog capability discovery where a Cloud node (or a Fog node) can discover a local Fog node including its capabilities and related context information.

- Step 3: Reachability verification where the reachability between two local Fog nodes, between a local Fog node and a Cloud node (or a Fog node) is verified before they start to communicate.

- Step 4: Service provisioning and recommendation where fog services on a local Fog node is provisioned and some service parameters could be configured or recommended by another local Fog node or by a Cloud node or a Fog node.

- Step 5: Service request where the smart transportation service requests fog services directly from a local Fog node or indirectly through a Cloud node (or a Fog node).

- Step 6: Service orchestration where fog services are orchestrated among local Fog nodes, Cloud nodes and/or smart transportation application.

- Step 7: Service instantiation where the smart transportation service can be instantiated onto a local Fog node with or without the coordination of a Cloud node (or a Fog node).

Figure 6.21.6-1: Normal Flow - Smart Transportation System

    - Alternative Flow

None

    - Post-conditions

None

    - High Level Illustration

At the highest level as shown in Figure 6.21.9-1, there is the Cloud Node (CN) that manages all the Fog nodes and is aware of their presence and capabilities. The CN and the Local Fog Nodes (LFNs) interact using the Fog-to-Cloud interface. The CN may be deployed such that it oversees a very large area which is composed of several smaller areas, known as fog areas, each of which is overseen by a LFN. The LFNs together provide a Fog Service Layer that is capable of providing fog services to services running on the LFNs and to each other using a Fog-to-Fog interface.

Services include extracting data from “things”, performing analytics, sharing data, etc. The LFN can be connected to fog entities (FEs) which are nodes with fog capabilities. Their capabilities may be the same as those of the LFNs or they can be smart cameras with fog capabilities (memory, compute, image processing logic, communication that may be wired or wireless, etc) or smart traffic lights, or speed sensors with fog capabilities. In the smart transport use case, the FEs are types of FNs with smaller fog capabilities and may be residing in service points such as a coffee shop and they in turn can control “things” such as cameras, sensors, and can send or receive data to and from these “things”, respectively. FEs can also be nodes residing in other important buildings or departments such as firefighting departments, hospitals, police department, etc. Furthermore, FEs may be basic IoT devices such as sensors, cameras, actuators, etc. As such, these FEs may be aware of reported events and locations to which an ambulance, fire truck, police vehicle, etc, may be dispatched. The FEs can collect such information or data and send it to the LFNs to which they have connections with.

The Application Layer running on LFNs interact with the LFNs only and hence are not aware of the FEs with which the LFNs may be connected. The FEs may appear as “things” to the fog application layer. Note that the FEs may also host their local applications. However, for the smart transportation use case, the application layer refers to the application layer that is residing above the LFNs. Although the FE can also host applications, our future reference to application layer below is about the application layer which runs on the LFNs. Finally, it is assumed in our architecture that the LFNs alone are the primary providers of the fog service layer and may use data or services from FEs or IoT devices in a manner that is transparent to the applications that use the fog service layer.

As an example only Figure 6.21.9-1 shows that LFN 1 is connected to two FEs, 1a and 1b, while LFN 2 is connected to one FE 2a, and LFN 3 connects to FEs 3a and 3b. However, these are just examples that are not intended to limit such connections to the numbers shown. Note that the actual protocol that runs on interfaces between the Fog nodes, and between the Fog nodes and the Cloud, are out of scope of this document.

Figure 6.21.9-1: High Level Illustration - Smart Transportation

    - Potential requirements

- The oneM2M System shall enable the remote instantiation of services across Fog/Edge networks as well as the remote provisioning of information required to instantiate the services.

- The oneM2M System shall enable the sharing and discovery of service capability information across  Fog/Edge networks.

- The oneM2M System shall enable requests for services to be provided by specific Fog/Edge nodes.

- The oneM2M System shall enable service continuity and migration among Fog/Edge nodes.

- The oneM2M System shall enable the orchestration of services provided by Fog/Edge nodes in a dynamic fashion to satisfy operational requirements for availability, scalability, interoperability, etc.

  - Automated Parking

    - Description

The concept of Valet Parking is widely used all over the world; for example, the more  luxurious hotels and restaurants, stores and other businesses.

Once a customer arrives with his/her vehicle at the hotel, he/she gets out of the vehicle and hands over the car-keys to the hotel personnel, which will then drive the vehicle to its parking spot, relieving the customer from that task. In the meantime, the owner of the vehicle can e.g. check-in or attend a meeting. Likewise, the vehicle is returned by the hotel personnel upon the request of the relevant customer. Utilising the technology evolution of self-driving vehicles, it logical next step to also automate the valet parking concept, further referred to as Autonomous Parking, or AP.

In AP (see Figure 6.22.1-1), the autonomous vehicle will park itself after the driver has left the car (step 1) at a drop-off point, which may be located near the entrance of a parking lot. The autonomous vehicle will find an available parking spot (step 2) and drive and park itself (step 3). When the driver wants to leave the site, he/she will simply request from the autonomous vehicle to return by itself (step 4) to the collect point, using (for example) a smartphone app.

Figure 6.22.1-1: Automated Parking Sequence.

To navigate safely around the parking lot to its destination, the Automated Driving (AD) vehicle uses information about the environment around the vehicle. The vehicle can use its own functions and sensors to observe immediate environment, but it can also benefit from gathering additional data going beyond what its sensor can observe – like accessing IoT platforms which can provide data and functions based on IoT enabled sensors like parking cameras, as well as position info from other vehicles driving (or being parked) at parking. Furthermore, IoT platforms may provide information to support services for booking a parking place and arranging (automated) payment.

    - Source

REQ-2018-0012R03 Use case: Automated Valet Parking (AVP)

    - Actors

Vehicle owner

Profits from the AP service by decreasing the required time to park the vehicle (shorter drive), or by choosing cheaper parking garage, or by increasing the probability of finding a free slot in the parking area and decreasing the probability of vehicle accidents in the parking area.

IoT platform provider

It operates an IoT platform which is collecting data from vehicles, parking spots, roads, other participants in traffic present, and from surrounding associated infrastructure (traffic lights, cameras, etc.).

Autonomous Parking Service provider

Party that is providing Automated Parking Service (APS). It collects data from different sources relevant for parking of the vehicle (from parking garage, or smart city on available parking spots in the street). It processes data and gives destination to AD vehicle where to park itself.

Communication Network provider

Provides connectivity between vehicles, roads, parkings and associated infrastructure.

    - Pre-conditions

The vehicle supports autonomous driving, meaning it is capable of autonomously driving.

    - Triggers

AP function (sending vehicle to park itself, as well as calling it back to pick up point) is activated by vehicle driver.

    - Normal Flow

- The APS continuously collects data from vehicles, parking garages, smart city (on available parking spots), Vulnerable Road Users (VRU’s), smart city camera’s,…

- When receiving request from end-user for finding a parking spot for particular vehicle, APS will determine which parking spots fulfils criteria or being part of end-user’s profile. For example, this can be ‘find closest garage’, ‘park on street’, ‘park on restaurant’s plot’,…

- APS publishes, and subscribed vehicles get from oneM2M location of parking spots.

      - Example of data published by APS

Here we present example of data (name-type-value) that can be used.

Table 6.22.6.1-1 Example of data published by APS

| Parking location | Description |
| --- | --- |
| + messageID: uint8 | Unique message ID, as defined in ETSI EN 302 637 |
| + stationID: uint32 | Unique station (vehicle) ID as defined in ETSI EN 302 637 |
| + generationTimestampUTC: uint64 | The elapsed time since midnight January 1st 1970 UTC in miliseconds. (with leap seconds) |
| + latitude: WGS84 double | Coordinate for parking space defined in ETSI EN 302 637 |
| + longitude: WGS84 double | Coordinate for parking space defined in ETSI EN 302 637 |
| + altitude: WGS84 double | Coordinate for parking space defined in ETSI EN 302 637 |

    - Alternative Flow

None.

    - Post-conditions

Vehicle stays in AP mode until it is removed from it by end-user, typically after picking up end-user.

    - High Level Illustration

oneM2M

1.) “Find parking place for my car”

2.) “Here is location to park”

1

2

3

Figure 6.22.9-1: Process of finding a parking space and parking supported by M2M system (finding a place, driving to it, and parking)

    - Potential Requirements

- The oneM2M system shall support the Autonomous Parking (AP) related data (for example available parking places and their location) exchange between the vehicle and the platform.

- The oneM2M system shall support a common information model for AP including model for parking places – both in garage and in streets.

- The oneM2M system should support a profile for parking with a consistent information model based on existing information models (e.g. ETSI ITS, DATEX and Sensoris).

  - Platooning

    - Description

Platooning is function where vehicle is automatically following another vehicle at a relatively close distance. Driving in a platoon requires vehicles to use inter-vehicle communications to anticipate timely on manoeuvres of other vehicles in the platoon.

Figure 6.23.1-1: Platooning vehicles on public road, lead vehicle has driver

Figure 6.23.1-2: Platooning trucks on public road, lead vehicle has driver

Several aims and motivations for vehicular platooning exist, such as improvement of traffic throughput and homogeneity, enhancement of traffic safety due to small speed variations and relative low impact velocities in collisions, and reduction of fuel consumption and emissions due to lowering the air drag. These objectives can to a certain extent already be achieved by non-automated driving systems (i.e. human driver monitors the environment and may execute e.g. the steering task), although a higher level of automation is considered to contribute in a positive way. Automated driving (system performs all aspects of the dynamic driving task) can offer additional benefits in terms of comfort (relieving the driver from driving task) and efficiency (no driver required in vehicles). The vehicles that follow have automated steering and distance control to the vehicle ahead, and the control is supported by advanced V2V communication extended with additional IoT data.

In order to support platooning, in M2M system each vehicle is represented by an object containing elements describing the vehicle (unique ID, type, owner,…), state of the vehicle (location, speed),… Knowing the state of each vehicle is then used by applications that use IoT data to provide service.

Each platoon is longitudinal set of vehicles driving one behind another, and therefore platoon model is collection of data objects of individual vehicles. For practical purposes like route planning, or planning of timing (for example can vehicle pass through green light or not), applications should treat platoon as single virtual vehicle. Note that this means that since vehicles can join and leave a platoon, that means that platoon data object is flexible collection of data objects of vehicles making part of it.

    - Source

REQ-2018-0013R03 Use case: Platooning.

    - Actors

Vehicle owner / driver / passenger in the vehicle

Platooning service is enhancing traffic safety for vehicle driver and passenger, due to small speed variations and relative low impact velocities in collisions, and as well reduction of fuel consumption and emissions due to lowering the air drag.

IoT platform provider

It operates an IoT platform which is collecting data from vehicles, roads, other participants in traffic present, and from surrounding associated infrastructure (traffic lights, cameras, etc.).

Platooning Service provider

Party that is providing Platooning Service (PS) which runs in cloud. It does authentication of vehicles that want to make use of platooning, collects info on currently running platoons, and provides ‘rendezvous’ info needed for vehicle to meet platoon. Note that this service will (very likely) make use of route planning function, not covered here.

Platooning Manager provider

Party that is providing Platooning Manager (PM) which runs on Multi-Access Edge Computing (MEC) node. Note that it is possible to have this function also running on cloud.

Platooning Function provider

Platooning Function (PF) runs in vehicles and is responsible for maintaining a position of the vehicle in the platoon (distance to other vehicles, follow trajectory of lead vehicle), as long as the vehicle is member of platoon.

Communication Network provider

Provides connectivity between vehicles, roads and associated infrastructure. It is not expected or mandated that single network operator provides all of connectivity.

    - Pre-conditions

The vehicle supports autonomous driving as well as platooning - meaning it is capable of autonomously driving, also transmitting and receiving data from other vehicles, road and other infrastructure, other participants in traffic (pedestrians, cyclists).

    - Triggers

Platooning is activated by vehicle driver. Driver can start this function either before starting a trip, by indicating where it needs to go, and its current location is taken as staring point, or it can do it during the (started) driving session.

Platooning Service will match this user (and its vehicle) to one of existing (or yet forming) platoons. After calculating route for this user, platooning service will provide this data so vehicle will rendezvous with chosen platoon.

    - Normal Flow

There are three distinct phases in platooning:

First phase of platooning is finding a platoon. This is done by Platooning Service (PS). Finding a platoon is a non-time critical function and is based on high level (discovery, authentication) information on available platoons, and it typically would run in the service provider’s cloud. Result of this phase is matching of vehicle that want to join a platoon to an existing platoon, or start new platoon with this vehicle in it. If joining an existing platoon, PS will calculate randezvous point for this vehicle and platoon.

Second phase of platooning happens when vehicle that wants to join a platoon comes in vicinity of the platoon, when Platoon Manager (PM) which is running on a MEC node, will assist vehicle by providing information on the current state of the platoon, which goes beyond V2V communication range. Note that since MEC node will be part of base station (eNB or gNB in 5G) there is a question of handover of platoon related data when platoon and vehicle joining it are moving from coverage area of one base station to coverage of another base station. It is important to note that east-west MN handovers are not currently covered in oneM2M.

Finally, third phase of platooning is situation where vehicles are driving in the platoon, and that is platooning application which is running in the vehicle.

      - Normal Flow 1: Finding platoon - Platooning service

- User contacts PS, authenticates itself and states its own destination and leave / arrival time.

- PS looks at all platoons that go to preferred destination, check leaving /arrival times, and matches user (and its vehicle) to one platoon. If no such platoon exists, PS informs vehicle and defines it to be a platoon containg just that one vehicle.

- PS collects current platoon’s state via IoT platform and uses that info to plan a route for user’s vehicle so it can meet / join the platoon.

- PS informs user’s vehicle on chosen route and rendezvous point.

- Vehicle starts riding to rendezvous point.

      - Normal Flow 2: Joining the platoon - Platooning manager

Condition is that vehicle that wants to join a platoon is in vicinity of the platoon, but it has not yet joined it.

- Platoon manager keeps track of the platoon and vehicle that wants to join.

When distance between those two is small enough and it is safe to perform manouver of joining the platoon (enough space, no traffic lights in vicinity, no other participants), it will instruct leading vehicle of the platoon, trailing vehicle of the platoon and vehicle wishing to join that they can start process of joining.

When vehicle and platoon are within V2V communication range, platoon will inform PM about it, and from that moment signalling during the joining process goes via V2V communication and not via PM.

      - Normal Flow 3: Driving - Platooning

- Vehicle receives messages from other vehicles in the platoon and acts accordingly.

      - Example data for platoon formation advice

This is data that is sent from roadside platooning service to vehicle that wants to join a platoon.

Table 6.23.6.4-1 Example data for platoon formation advice

| FormationAdvice | Description |
| --- | --- |
| + messageID: uint8 | as defined in ETSI EN 302 637 |
| + stationID: uint32 | as defined in ETSI EN 302 637 |
| + platoonID: uint32 | Unique platoon ID |
| + generationTimestampUTC: uint64 | The elapsed time since midnight January 1st 1970 UTC in miliseconds. (with leap seconds) |
| + speedAdviceValue: array float32 | Last known latitude of the engaging vehicle in put in here by and updated only by the PlatoonService. |
| + speedAdviceStartTime: array uint64 | Last known longitude of the engaging vehicle in put in here by and updated only by the PlatoonService. |
| + speedAdviceStopTime: array uint64 | The elapsed time since midnight January 1st 1970 UTC in miliseconds. |

      - Example data on platoon status

This is data that leading vehicle of the platoon sends to PS and PM (in which it informs them about state of the platoon).

Table 6.23.6.5-1 Example data on platoon status

| PlatoonStatus | Description |
| --- | --- |
| + messageID: uint8 | as defined in ETSI EN 302 637 |
| + stationID: uint32 | As defined in ETSI EN 302 637 |
| + platoonID: uint32 | Unique platoon ID |
| + generationTimestampUTC: uint64 | The elapsed time since midnight January 1st 1970 UTC in miliseconds. (with leap seconds) |
| + platoonState: uint8 (enum) | None(0), Standalone(1), VehicleEngaging/Assembling(2), Platooning(3), VehicleDisengaging(4), DisengagingAll(5) |
| + platoonDestination: string | Global platoon desitnation eg (cities); Eindhoven..Helmond |
| + vehicleRole: uint8 (enum) | None(0), Standalone/Ready-for-leading(1), Trailing(2), Following(3), Leading(4), Ready-for-leading(5) |
| + vehicleMode: uint8 (enum) | None(0), Standalone(1), Engaging(2), Platooning(3), Disengaging(4), Searching(5), Forming(6) |
| + platoonSize: uint8 | Number of vehicles in platoon |

      - Example data for platoon navigation

This is sent by roadside service to leading vehicle of the platoon. It is end destination of the platoon, with optional waypoints in between.

Table 6.23.6.6-1 Example data for platoon navigation

| PathToPointOnMap | Description |
| --- | --- |
| + stationID: uint32 | As defined in ETSI EN 302 637 |
| + generationTimestampUTC: uint64 | The elapsed time since midnight January 1st 1970 UTC in miliseconds. (with leap seconds) |
| + destinationLatitude: WGS84 double | As defined in ETSI EN 302 637 |
| + destinationLongitude: WGS84 double | As defined in ETSI EN 302 637 |
| + path: t.b.d. (waypoint): array (destinations) |  |

    - Alternative Flow

None.

    - Post-conditions

Vehicle stays in platooning mode until:

- It arrives at destination, or comes close to it, when it leaves platoon and drives the rest of the way autonomously to destination.

- It can be triggered by user to leave a platoon – and continue autonomously.

- It receives information from platooning service that it can / should leave current platoon and join other platoon (multi-hop platooning; member of platoon 1 en route from A to B, and then at point B leave platoon 1 and join platoon 2 that will take it to C). Short time between two platoons can be covered by vehicle driving autonomously, and is not in strict sense stop for the platooning function.

    - High Level Illustration

Figure 6.23.9-1 shows three phases in platooning:

Green lines represent first phase, red lines second phase, and blue lines third phase. Note that what is now shown here is handover from MN running on MEC node to MN on another MEC node (as mentioned before in text).

Finding a platoon

Joining a platoon

Driving in a platoon

Platoon

data

Platoon

data

Platoon

data

Platoon

management

data

Platoon

management

data

cloud

Platoon

service

data

Platoon

service

data

Figure 6.23.9-1: Data flows in platooning

    - Potential Requirements

- The oneM2M system shall support handover (e.g east-west communication) over platoon relevant data from one Platooning Manager (running on edge node) to next Platooning Manager (running on neighboring edge node).

- The oneM2M system shall support the the Platooning data exchange between the vehicles, Platooning Service provider , Platooning Manager and oneM2M platform.

- The oneM2M system  shall support a common information model for Platooning including vehicle state and platooning state.

- The oneM2M system should support a profile for Platooning with a consistent information model based on existing information models (e.g. ETSI ITS, DATEX and Sensoris).

- The oneM2M system shall support group management (e.g .joining, leaving and changing vehicle’s role within the platoon) and group message communication for platooning service.

- The oneM2M system shall support Vehicle-to-Vehicle (V2V) communication via different network interfaces.

- The oneM2M system shall support management of Vehicle-to-Vehicle (V2V) network interface switching.

  - Highway pilot

    - Description

It is expected that autonomous driving will radically reshape transport networks around the world by reducing congestion, fatalities and fuel consumption, and improving other driving conditions, in particular on highway environments. Moreover, it is expected that autonomous driving in highway environments will reduce costs in the line-haul trucking industry by (up to) 40%;

The Highway Pilot function automates highway driving, meaning that steering and speed adjustments are executed by the automated driving system. As the name of the function already implies, the Highway Pilot is intended for use on Highways only. The added-value of this function is its ability to enhance drivers’ and automated vehicles’ awareness on potential road hazards on route and to assist them to adapt their driving accordingly.

In order to react in time to obstacles or damaged road surface, vehicles need to have information on those. Vehicle can subscribe to that information, which must be available on IoT platform.

If the information on obstacles or damaged road surface is not available and vehicle still detects an obstacle – for example pothole, vehicle will then send information on the location of detected obstacle and type (pothole) to IoT platform. To avoid false positives, it is up to application (AE) to determine how to handle collected data, and when to make it available to other vehicles that are subscribed to this information.

Road hazards may refer to several events and situations:

emergency braking vehicles / slow vehicles

stationary vehicles (breakdowns or accidents)

fast approaching emergency vehicles

traffic jams and queues

road works / route modifications

nearby presence of bicycles or pedestrians

fallen objects (from vehicle, trees)

road defects (potholes, bumps, cracks, gravel)

weather related road changes (puddles, ice)

etc.

Receiving anticipated warning information about such events is useful on all types of road environments, including a highway context; where vehicles move at high-speed require shortened reaction time.

Furthermore, anticipated warning information also benefits all modes of driving:

In Manual Driving mode, thanks to experience, drivers learn to handle hazardous situations. However, a sudden action from a driver when avoiding obstacle on the road (ex: trajectory change, quick deceleration) may become another hazard for others.

In Assisted Driving mode, which is more and more used on Highway roads, drivers considerably relax their attention on the road, hence increasing their response time when a hazard occurs, which can lead to collision with obstacle, trajectory change or quick deceleration which may become hazard for others.

In Automated Driving mode, passengers depend on the detection of the vehicle’s own sensors. The hazard must enter the sensors perception range and be identified as such before the vehicle reacts. If the hazard is hidden around the corner, the reaction may be abrupt. For passengers’ comfort and acceptance of AD functions, it is a priority that AD driving is as smooth as possible.

Finally, not all hazards will trigger pre-emptive actions from drivers and vehicles. For example, near missed potholes and slippery surfaces may go unnoticed and be noticed too late.

    - Source

REQ-2018-0014R03 Use case: Highway Pilot

    - Actors

Vehicle owner / driver / passenger in the vehicle

Profits from the Highway Pilot service by decreasing probability of vehicle damage/accidents in the highway environment and increasing safety due to timely taking into account state of the roads and their surface, thereby avoiding creating hazardous situations for itself and other participants.

IoT platform provider

It operates an IoT platform which is collecting data from vehicles and other sources (as for example roadside cameras). Data on obstacles or damaged road surface is provided to vehicles that are subscribed to it.

Highway Pilot Service (HPS) provider

Party that is providing Highway Pilot Service (HPS) which is collecting, processing and publishing data on detected road obstacles or damaged road surface which is coming from vehicles and other sources.

Communication Network provider

Provides connectivity between vehicles, roads and associated traffic infrastructure, provides connectivity to IoT nodes as well as to application servers where services run. It is not expected or mandated that single network operator provides all of connectivity.

    - Pre-conditions

The vehicle has sensors to detect obstacles or road surface damages, and has communication interfaces to send collected data to IoT platform.

    - Triggers

Highway pilot is activated automatically, when vehicle is driving on the highway, which can be determined in different ways (for example location on the map or by speed of the vehicle).

    - Normal Flow

- Vehicle detects that it is on the highway, and starts collecting data on obstacles on the road and damage (potholes, cracks) on the road surface. This information is sent to the IoT platform.

- Highway Pilot Service (HPS) is subscribed to data on obstacles on the road and damage (potholes, cracks) on the road surface, and based on HPS provider defined processing and criteria it will publish to IoT platform data on detected obstacle or road surface damage that includes location of detected obstacle, and type of obstacle (crack in road surface, or patch of ice on road).

- Published data includes at least vehicle ID, state of vehicle (speed, heading, …), time of detection, location of detection, type and values of sensors that have detected obstacle.

- To give an example, if a vehicle’s wheel drive across patch of ice, it can be detected by vehicle’s ABS sensors, and that information is sent to IoT platform. Possible criterion for HPS can be that for example 3 vehicles report the patch of ice on the same location, HPS will then publish that information so other (subscribed) vehicles can adjust their driving accordingly. Note that each (subscribed) vehicle is responsible for interpretation of provided IoT Highway Pilot Service data and action based on it.

      - Example of data published by vehicle on detected obstacle

Here we present example of data (name-type-value) that can be used.

Table 6.24.6.1-1 Example of data published by vehicle on detected obstacle

| Obstacle detection | Description |
| --- | --- |
| + messageID: uint8 | Unique message ID, as defined in ETSI EN 302 637 |
| + stationID: uint32 | Unique station (vehicle) ID as defined in ETSI EN 302 637 |
| + generationTimestampUTC: uint64 | The elapsed time since midnight January 1st 1970 UTC in miliseconds. (with leap seconds) |
| + stationType: (1..225) int | Type of vehicle, defined in ETSI EN 302 637, for example ‘truck’, ‘car’, … |
| + latitude: WGS84 double | defined in ETSI EN 302 637_2 |
| + longitude: WGS84 double | defined in ETSI EN 302 637_2 |
| + heading: WGS84 (0..360) | defined in ETSI EN 302 637_2 |
| + speed: (0..163) m/s | defined in ETSI EN 302 637_2 |
| + sensorID: uint8 | as defined in ETSI EN 302 637 |
| + sensorType: string | Sensor type, for example ‘camera’, ‘LIDAR’,… |
| + sensorValue: double | Value measured by sensor |

    - Alternative Flow

None.

    - Post-conditions

Vehicle stays in highway pilot mode until it leaves highway, which can be determined for example by vehicle’s location.

    - High Level Illustration

Figure 6.24.9-1: Example of IoT data streams and corresponding communication networks

    - Potential Requirements

- The oneM2M system shall support the Highway Pilot data (for example road surface state, detected obstacles) exchange between the vehicle and oneM2M platform.

- The oneM2M system shall support a common information model for Highway Pilot including vehicle state, detected obstacles, road conditions.

- The oneM2M system should support a profile for Highway Pilot with a consistent information model based on existing information models (e.g. ETSI ITS, DATEX and Sensoris).

  - Car rebalancing service

    - Description

The driverless car rebalancing service is targeted to offer rebalancing of several Autonomous Driving (AD) vehicles distributed over several pickup points within a car sharing concept. This is situation that can arise when a large number of customers (end-users) are making use of car sharing service. One example of this is early morning where end-users will take shared vehicles to go to train station, or when they go for a sports game. Shared cars will be present in large numbers at given location.

That can lead us to situation that there are more shared vehicles at one location, while later they might be needed somewhere else.

That is why it is important to have car rebalancing, capability of AD vehicles to drive without passengers to location where they will be needed.

Whether there is a need for vehicles to drive without passengers to another location or not is determined by Car Rebelancing Service (CRS) which is extension of Car Sharing Service (CSS (see relevant use case)).

CRS is coupling data from CSS (request for car sharing), and mostly other sources of data in order to proactively trigger car rebalancing. That input will include:

probabilistic & historical data from car sharing

information on major events like concerts, sports games, where a large number of people can be expected to make use of car sharing. This includes start/ end times of event, number of visitors, location of events, timetables of public transport, …

Get actual weather information from internet.

The key benefits of the car rebalancing use case is increased availability of vehicles in real time, i.e., min. response time between request and delivery.

    - Source

REQ-2018-0016R04 Use case: Car Rebalancing.

    - Actors

Vehicle owner

The car rebalancing service provides increased availability of vehicles in real time, i.e., min. response time between request and delivery, resulting in increased end-user’s satisfaction with service.

IoT platform provider

It operates an IoT platform which is collecting data from vehicles, roads (busy/not busy), historic CSS data, and other sources that can it can use to predict the need for rebalancing.

Car Rebalancing Service provider

Provides the car rebalancing service. The car rebalancing service collects information on the available vehicles on each known pickup points. If rebalancing is needed (i.e., vehicles need to be redistributed) then it will make use of routing and motion planning function, to lead the vehicles to the selected pickup points/parking.

Communication Network provider

Provides connectivity between vehicles, roads and associated infrastructure, and also to other data sources (which data sources are used is matter of how CRS provider is implementing its service). It is not expected or mandated that single network operator provides all of connectivity.

    - Pre-conditions

The car rebalancing service can obtain current shared vehicles info (current location, destination), historic data from CSS, as well as data from other sources (university schedulers, concert agenda, sport game info, weather info,…).

    - Triggers

CRS is continuously monitoring the actual state of shared vehicles, and predicts upcoming need for shared vehicles. Other triggers are possible – for example it can be activated when it is observed that not enough vehicles are available (according to pre-defined threshold) at pickup points/parkings or when a higher (than a predefined threshold) number of vehicles is parked at a pickup point/parking.

    - Normal Flow

- CRS monitors number of shared vehicles that are not occupied.

- Based on prediction of need (coming from CSS and other sources), CRS will per vehicle calculate vehicle’s next location (where they need to get), and send to each of vehicles its new destination.

- Vehicle drives and parks at new location.

      - Example of data published by CRS

Here we present example of data (name-type-value) that can be used.

Table 6.25.6.1-1 Example of data published by CRS

| PathToNextPickup / Parking Point |  |
| --- | --- |
| + stationID: uint32 | Vehicle unique ID, defined in ETSI EN 302 637 |
| + generationTimestampUTC: uint64 | When was command given to start rebalancing |
| + destinationLatitude: WGS84 double | defined in ETSI EN 302 637_2 v010302 |
| + destinationLongitude: WGS84 double | defined in ETSI EN 302 637_2 v010302 |
| + path: t.b.d. (waypoints?) | Optional data – for example that can be command – avoid toll roads. |

    - Alternative Flow

None.

    - Post-conditions

Vehicle stays in car rebalancing mode until the car rebalancing service provider or vehicle owner decide that the vehicle cannot support the car rebalancing service anymore;

    - High Level illustration

Figure 6.25.9-1: Example of rebalancing cars

    - Potential Requirements

- The oneM2M system shall support the Car Rebalancing data (vehicle’s next location, ...) exchange between the vehicle and oneM2M platform.

- The oneM2M system shall support a common information model for Car Rebalancing including vehicle state and target destination (endpoint).

- The oneM2M system should support a profile for Car Rebalancing with a consistent information model based on existing information models (e.g. ETSI ITS, DATEX and Sensoris).

  - Urban driving

    - Description

Autonomous vehicles have the potential to remove human error and reduce instances of accidents caused by driver error, drunk driving or distracted drivers, in particular in Urban Driving environments.

Urban Driving, see Figure 6.26.1-1, assisted by IoT has the main objective to support CAD (Connected and Automated Driving) functions through the extension of the Electronic Horizon of an automated vehicle. That means that the vehicle can process data from external sources which enrich those provided by its own sensors (Camera, LIDAR, Radar…). The type of relevant information that automated vehicles may access as IoT elements concerns:

Traffic lights at intersections

Information from infrastructure cameras (such as pedestrian, bicycle, obstacle presence)

Information from Vulnerable Road Users (VRU) – pedestrians, cyclists, pets,…

Information from other vehicles captured by their own sensors and shared as IoT elements.

Taking this type of information into account the CAD vehicles will adapt their behaviour according to the additional IoT  information about their surroundings, available through their connection to an onboard IoT platform.

Figure 6.26.1-1: Urban driving – lots of different participants

    - Source

REQ-2018-0017R03 Use case: Urban Driving.

    - Actors

Vehicle owner

Profits from the Urban Driving service by decreasing probability of vehicle damage/accidents in the urban environment and at the same time reducing the vehicle  journey time when the road traffic jams on the urban roads are minimized or eliminated;

IoT platform provider

It operates an IoT platform which is collecting data from urban traffic data sources such as:

Traffic lights at intersections

Information from infrastructure cameras (such as pedestrian, bicycle, obstacle presence)

Information from VRUs

Information from other vehicles captured by their own sensors and shared as IoT elements.

Urban Driving Service provider

Party that is providing Urban Driving Service (UDS).

Communication Network provider

Provides connectivity between vehicles, roads and associated infrastructure. It is not expected or mandated that single network operator provides all of connectivity.

    - Pre-conditions

The vehicle supports autonomous driving - meaning it is capable of autonomously driving, also transmitting and receiving data from other vehicles, road and other infrastructure, other participants in traffic (pedestrians, cyclists).

    - Triggers

Urban Driving is activated automatically, when vehicle is driving on the urban environment, which can be determined in different ways (for example location on the map).

    - Normal Flow

- Vehicle detects that it is in urban environment, which leads to having higher sampling rate for its sensors, for example. Vehicles is sending own state info, together with information on detected objects to oneM2M platform.

- Urban Driving Service (UDS) is subscribed to data from vehicles, traffic lights, road sensors (magnetic loops), camera’s, and also to data from other sources like data from pedestrians smartphones (sending their location),….

- UDS is processing data from all available sources, and uses prediction to analyse possible problems (crashes, traffic jams). This information is published on oneM2M platform and it can be accessed by all subscribed devices.

      - Example of data published by UDS

Here we present example of data (name-type-value) that can be used.

Table 6.26.6.1-1 Example-1 of data published by UDS

| Driving advice | Description |
| --- | --- |
| + messageID: uint8 | Unique message ID, as defined in ETSI EN 302 637 |
| + stationID: uint32 | Unique station (vehicle) ID as defined in ETSI EN 302 637 |
| + generationTimestampUTC: uint64 | The elapsed time since midnight January 1st 1970 UTC in miliseconds. (with leap seconds) |
| + speedAdviceValue: array float32 | Speed advice to vehicle. |
| + laneAdvice: string | lane number from left to right in the current road driving direction |

Table 6.26.6.1-2 Example-2 of data published by UDS

| Detected VRU | Description |
| --- | --- |
| + messageID: uint8 | Unique message ID, as defined in ETSI EN 302 637 |
| + stationID: uint32 | Unique station (vehicle) ID as defined in ETSI EN 302 637 |
| + generationTimestampUTC: uint64 | The elapsed time since midnight January 1st 1970 UTC in miliseconds. (with leap seconds) |
| + detectedVRU: enum | Detected VRU, for example ‘pedstrian’, ‘cyclist’, ‘dog’, … |
| + locationVRU: enum | Location from where VRU is approaching, for eample ‘left’, front left’,… |

    - Alternative Flow

None.

    - Post-conditions

Vehicle stays in urban driving mode until it leaves the urban road environment, which can be determined by vehicle’s location for example.

    - High Level Illustration

Figure 6.26.9-1: Urban driving example: cyclist is registered

    - Potential Requirements

- The oneM2M system shall support the Urban Driving data (for example city’s traffic management system, traffic lights state, detected VRUs, …..) exchange between the vehicle and oneM2M platform.

- The oneM2M system shall support a common information model for Urban Driving including vehicle state, city traffic infrastructure state and detected VRUs.

- The oneM2M system should support a profile for Urban Driving with a consistent information model based on existing information models (e.g. ETSI ITS, DATEX and Sensoris).

  - Traffic Light Control and Coordination using Reliable Edge/Fog Service

    - Description

A traffic light control and coordination application provides control and coordination to traffic lights at one or multiple intersections to ensure that traffic and pedestrians move as smoothly and safely as possible. The system collects data from video cameras and motion sensors on poles, inductive-loop traffic detector in the pavement and vehicles on the road. A controller is usually deployed in order to provide quick response and fast control. Service reliability is an essential feature for this system. For example, if the controller at an intersection is down, the traffic light control and coordination system should still provide services. In this use case, an Edge/Fog architecture is used to improve system reliability.

    - Source

REQ-2018-0031R01 Use case for Reliable Edge Fog Computing

    - Actors

Traffic light control and coordination system Application: An application control for the traffic lights at one or multiple intersections based on data collected from video cameras and motion sensors

Cloud Node: A node with cloud or fog capabilities that manages the operations of other fog nodes lower in the deployment hierarchy. It also interacts with service providers and has service layer agreements and can authorize requests from fog nodes related to applications that are hosted on the fog nodes. Furthermore, the Cloud Node may oversee and manage the interactions between different fog nodes that together enable a fog service layer for applications.

Fog Node: A node that has any fog resource such as compute, storage, communication, analytics, etc.. For this use case, Fog/Edge Nodes are assumed to be deployed at higher levels than the level of a Local Fog Node. It also assumed that a Fog/Edge Node connects to at least one Local Fog Node and that the former also manages at least one Local Fog Node.

Local Fog Node (LFN): A Fog Node which is connected to one or more  Fog Entities that are located in the service area of the Local Fog Node. The Local Fog Node, together with other neighbouring Local Fog Node, appear as a Fog Service Layer to the applications that are hosted on the Local Fog Node.

Note: In this use case, the Local Fog Node is located between the Fog Entity and the Fog Node. The Local Fog Node is assumed to be managed by a Fog Node or a Cloud Node that is one level above the Local Fog Node in the fog deployment hierarchy

Fog Entity (FE):  A node that has a sub-set of the capabilities of a Fog Node. In this use case Fog Entities may be sensors, cameras, traffic lights, or any IoT device with basic (relatively small) compute and storage resources. Fog Entities are deployed at the edge of the deployment,  connect to  Local Fog Nodes  and are sources of data and services for the Local Fog Node. Fog Entities may also request and consume data or services from the Local Fog Nodes.

    - Pre-conditions

LFN are deployed at each intersection, controlling the traffic lights at that intersection.

Some or all LFNs may control traffic lights located at nearby intersections .

    - Triggers

A Local Fog Node that controls the traffic light at intersections is not functional and cannot provide traffic light control service.

FN detect that a Local Fog Node that controls the traffic light at an intersection is not functional.

    - Normal Flow

Figure 6.27.6-1 illustrates the high-level flows of smart transportation use case, which consists of the following steps:

- Step 1: FN finds Traffic Light 1 and 2 that are controlled by LFN 1. FN then selects LFN 2 to control Traffic Lights at intersection 1.

- Step 2: FN sends a request to notify LFN 2 to control Traffic Light 1 and 2.

- Step 3: LFN 2 sends response back to FN.

- Step 4: After receiving the request from FN, LFN 2 sends a request to Traffic Light 1 to notify that LFN 1 is non-functional and LFN 2 starts controlling Traffic Light 1.

- Step 5: After receiving the request, Traffic Light 1 sends a response back to LFN 2 and begins to uses the SL provided by LFN 2.

- Step 6: After receiving the request from FN, LFN 2 sends a request to Traffic Light 2 to notify that LFN 1 is non-functional and LFN 2 starts controlling Traffic Light 1.

- Step 7: After receiving the request, Traffic Light 2 sends a response back to LFN 2 and begins to uses the SL provided by LFN 2.

Figure 6.27.6-1: Normal Flow – Traffic Light Control When A Fog Node is Non-functional

    - Alternative Flow

None

    - Post-conditions

LFN at adjacent intersection control traffic lights that are located in the intersection.

    - High Level Illustration

Figure 6.27.9-1: High Level Illustration - Edge/Fog Traffic Light Control Application

    - Potential requirements

- The oneM2M System shall enable Edge/Fog Nodes to have service communications with multiple other Edge/Fog Nodes to meet reliability requirements.

The oneM2M System shall enable Edge/Fog Nodes to detect the failure of other Edge/Fog Nodes.

The oneM2M System shall enable the sharing and discovery of service capability information across Fog/Edge networks.

The oneM2M System shall enable requests for services to be provided by Fog/Edge Nodes.

The oneM2M System shall enable service migration among Fog/Edge Nodes.

  - Vulnerable Road Users Discovery Service using Edge/Fog Computing

    - Description

The Vulnerable Road User (VRU) use case is a vehicle domain service to detect pedestrians and cyclists on a road. The VRUs application should use accurate positioning information provided by various traffic participants. The information used for VRUs services has to be shared by VRUs. The VRUs make their presence/location known through their mobile devices (e.g., smartphone, tablets), along with vehicle’s use of that information [i.22].

In this example, the driver of a Host Vehicle (HV) intends to make a left turn. If there is a vulnerable user or cyclist passing the place where HV is going to pass, HV is alerted to the presence of a VRU in a safety and/or awareness message.

An offloading concept locating tasks and resources to a place where close to users can be applied to this VRU use case so that a service can be provided to users with very short delay. Introducing the offloading concept to oneM2M requires  new features to oneM2M system such as offloaded resources need to be kept in sync with the original ones.

    - Source

REQ-2018-0047R05 Use case for Vulnerable Road Users Discovery using Edge/Fog Computing

    - Actors

VRU Application: An application analyses various information to detect and warn potential conflicts caused by discovered VRU and HVs that it is supporting.

Cloud Node: A node with cloud or fog capabilities that manages the operations of other fog nodes lower in the deployment hierarchy. It also interacts with service providers and has service layer agreements and can authorize requests from fog nodes related to applications that are hosted on the fog nodes.

Edge/Fog Node: A node that has any fog resource such as compute, storage, communication, analytics, etc.. For this use case,  Fog/Edge Nodes are assumed to be deployed with offloading capability. This node has its VRU service zone so that it detects all HVs entering and leaving its service zone. When a HV enters its service zone, the node is able to retrieve relevant information from the Cloud Node and analyses VRU conflicts to avoid any accidents.

Vulnerable Road Users (VRUs): A user who has mobile devices (e.g., smartphone, tablets) and entering a VRU zone managed by Edge/Fog Node. The user allows their presence/location to be known by HVs to be used for VRU discovery service.

    - Pre-conditions

A VRU service zone is identified and Edge/Fog nodes are configured to support VRU service.

VRUs allow their present/location information to be shared by IoT service provider to be used for VRU discovery service.

    - Triggers

A VRU discovery application detects one of its serving HV enters a VRU service zone.

An Edge/Fog node detects that a new HV enters its service area.

    - Normal Flow

- Figure 6.28.6-1 illustrates the high-level flows of smart transportation use case, which consists of the following steps:

- Step 001: Application-1 requests offloading to the cloud node with the following information:

  - Offloading indication, Target offloading resources (a list of target resources), Contact of address of the Destination Edge/Fog, Status of offloading resources (block, readable, writable)

    - Block – The source resources are invisible while the resources are being offloaded

    - Readable – The source resources are readable at the source node even they are offloaded to other Edge/Fog nodes

    - Writable – The source resources are writable at the source even they are offloaded to other Edge/Fog nodes

  - If there is no application logic running on the target edge/fog node to handle the VRU discovery service, an algorithm or a service logic for the VRU discovery is also delivered to the edge/fog node.

  - For example, Application-1 sends an offloading request to several resources or group resources to be offloaded

- Step 002: The cloud node configures offloading resources

  - Each offload resource is marked as an offloaded resource with additional information, e.g., the contact of the offloaded edge/fog node.

- Step 003: The cloud node registers its offloaded resources to the destination Edge/Fog node (PUSH offloaded resources to Edge/Fog)

- Step 004: The Edge/Fog creates offloading resources based on the request from the cloud node

- Step 005: Application-2 is now able to retrieve information from the offloaded resources on Edge/Fog to perform VRU discovery and warning.

- Step 006: Application-2 updates one of offloaded resources (or a set of offloaded resources). This updates can be performed from VRUs as they moved within the service zone.

- Step 007: Edge/Fog updates values of the given offload resources from Application-2

- Step 008: Edge/Fog updates the updated values to the cloud node for synchronization. If the resources are blocked to other users, UPDATE (i.e., synchronization) is being paused until the offloaded resources are all released from Edge/Fog.

- Step 009: Application-1 requests to stop offloading and synchronizing to specific resources with the following information:

  - Target resources to be stopped

  - Indication to stop the offloading service

- Step 010: The cloud node gets information about where the requested resources are stored and retrieves values from the requested resources stored in the given Edge/Fog. Edge/Fog deletes the offloaded resources as the offloading service is now stopped.

- Step 011: The cloud node updates the values from Edge/Fog for synchronization.

Figure 6.28.6-1: Normal Flow – Offloading for VRU discovery service

    - Alternative Flow

None

    - Post-conditions

- None

    - High Level Illustration

Figure 6.28.9-1: High Level Illustration - Edge/Fog VRU discovery Application

    - Potential requirements

- The oneM2M System shall be able to send the information about offloading (e.g., offloading indication, a service logic, task, target offloading resources)

The oneM2M System shall be able to synchronize the data between source and offloaded resources.

The oneM2M System shall be able to manage offloaded resources based on given properties from the users, e.g., blocking the offloaded resources to be accessed while the resources are offloaded to other oneM2M nodes.

  - Car-to-Home service based on Geo-Fence

    - Description

Car-to-Home services that provide convenience to users are gradually expanding.

In the case of a vehicle moving to a destination, it can be given a distance from the home, in order to activate some functions in the house. Typically this kind of service can be provided based on Geo-Fence, and it triggers pre-defined actions. (e.g. turn on a light, turn on a heater)

However, there is a case when a vehicle approaches to the given distance from the home, it is necessary to adjust the timing of the action triggering according to the approach direction.

For example, as shown in Figure 6.29.1-1, if the road of Direction 1 is under construction, its Estimated Time Arrival (ETA) would be delayed more than the ETA of Direction 2. (e.g. Direction 1: 15 minutes, Direction 2: 5 minutes)

In order to determine optimal action trigger based on inconsistent ETA, each vehicle must be able to control their devices in time sequence or re-adjust their geo-fence boundary.

Figure 6.29.1-1: Generic Geo-Fence Scheme for Car-to-Home Service

In the time sequence case, a designated vehicle for time sequence sends an action trigger based on the time boundary which can be set multiple. To be designated for time sequence, the vehicle must enable the time setting for time sequential control. In the matter of necessity, this time sequence setting is generated by M2M infrastructure or Location Based Service Provider which owns all information of ETA for each direction.

Figure 6.29.1-2: Time Sequential Geo-Fence Scheme for Car-to-Home Service

The geo-fence boundary reshaping method is that the boundary module in the vehicle or M2M Infrastructure readjusts the destination boundary by the ETAs of all directions. In order to reshape, it must acquire information of boundary area (or diameter), ETAs and road conditions of all directions. After re-adjusting the boundary, vehicle or M2M infrastructure must notify the owner of the vehicle.

Figure 6.29.1-3: Boundary Reshaping Geo-Fence Scheme for Car-to-Home Service

In both cases, final confirmation can be done by the owner. The owner confirms a final decision of both action trigger time sequence setting and boundary reshaping.

In order to implement IoT Car-to-Home, M2M application needs to be mounted in a vehicle together with associated servers (e.g. service provider, location server). The vehicle must allocate its computing resources for M2M application.

    - Source

REQ-2018-0070R04-Use case on Car-to-Home based on Geo-Fence

    - Actors

Vehicle

Vehicle is a nomadic object monitored by M2M System node or Location Server. It is equipped with localization components as a form of hardware. (E.g. GPS, Cellular modem)

Hub

Hub receives access information within a reference distance from the vehicle, and triggers action to other nodes in the hub. (e.g. House, Building)

Action Node under Hub

Action Node under Hub operates when it is triggered from the Hub. (e.g. Light, Heater)

Location Server

A Location Server can localize the target device(s) using collected measurements (e.g. RSSI, AP ID) and detect Geo-Fence event based on set configurations.

M2M Service Platform

In charge of providing common functionalities for the M2M services.

Car-to-Home Service provider

Car-to-Home Service Provider supports the Car-to-Home service to stakeholders.

    - Pre-conditions

- Vehicle has embedded telematics unit for network communication.

The destination building (or house) has wireless IoT hub and devices for Car-to-Home Service.

Telematics server and IoT service server support authentication process for integration with high security.

Location server has the Boundary calculation module.

Vehicle has a screen or voice interface for owner’s decision of the new boundary.

M2M Application provides a dedicated IoT Car-to-Home service and mounted in vehicle, location server, and service provider.

    - Triggers

None.

    - Normal Flow

Figure 6.29.6-1: Normal Flow - Car-to-Home service based on Geo-Fence

- A vehicle sends a geo-fence request to M2M Service Platform , which contains parameters of hub and targeted action nodes for control.

- M2M Service Platform sends an analysis request to location server to determine trigger count, timing for control.

- Location server responds to M2M Service Platform with the result of analysis containing trigger count and time.

- M2M Service Platform sends a trigger request to a service provider with specified hub and node control information.

- Car-to-Home Service Provider sends the node trigger request to a hub for selected node control.

- Hub controls action nodes. Note that hub and each action node is connected.

- Once hub sends the control request, it receives a control result containing error code and parameters.

- Hub reports to the service provider for node control information.

- Service provider transfers node control information to M2M Service Platform .

- M2M Service Platforms ends geo-fence result to vehicle and location server.

- Location server uses geo-fence result for analysis and boundary update.

- When boundary update is completed, the location server sends provisioning to M2M Service Platform for update.

- M2M pushes the provisioning to the vehicle for geo-fence boundary update.

    - Alternative Flow

None.

    - Post-conditions

None.

    - High Level Illustration

Figure 6.29.9-1: High Level Illustration – Car-to-Home based on Geo-Fence

    - Potential Requirements

- The oneM2M System shall support dynamic and variable Geo-Fence settings configuration for location-based services (e.g. boundary reshaping).

The oneM2M System shall enable mechanisms for sequential triggering of operations (e.g. time-based, event-based) based on requirements defined by M2M applications.

3The oneM2M System shall enable mechanisms to expose policies about the  current and future resource needs of M2M nodes   for resource allocation and management purposes at the system level.

  - Vehicle Remote Control using External Platform

    - Description

- Vehicle remote control is a function or service to access the vehicle for its control or necessary information. Vehicle remote control services are currently used with a wide range of functions from ignition to destination setting in navigation platform.

In general, a vehicle is equipped with a telematics unit which has a modem for the connection of telematics services. Protocols of telematics communications can be consisted of various forms, such as TCP and HTTP. With respect to security and financial reasons, telematics communication tends to be highly secure and only remain short period of time.

Figure 6.30.1-1: Generic Diagram of Telematics System for Vehicle Remote Control

In Figure 6.30.1-1, the external platform conducts vehicle remote control by accessing to telematics server to control vehicles. Note that the external platform can be forms of application, server, or other vehicle platform.

In recent days, Artificial Intelligence (AI) is widely popular in information technology industry and various studies focus on in-depth about the integration of AI to nodes including vehicles. External platform with AI has potentials for object management in a sophisticated manner. However, it has issues including security.

For instance, if the platform is granted to access vehicle ignition or auto-driving mode, it may control ignition and drive on the highway under no driver presence without any intention of the driver. To prevent this kind of cases, the level of authorization must be established based on the security level of vehicle functions.

In order to determine the level of authorization, vehicle functions have to be categorized by following criteria;

- Autonomous capability and notification requirement,

- Visual activity and self-damage possibility,

- Vehicle component access (e.g. fuel information, Engine, transmission, and accelerator).

Autonomous capability and notification requirement is a credential to review the capabilities and seriousness of vehicle functions. If they are capable of operating on its own with risk-free, it can be in a pool of authorization granted. Visual activities and self-damage possibilities are looking in the aspects of whether or not the operation of vehicle functions is visible and harmful to surrounding environments.  Vehicle component access is a direct access right of the vehicle and it is the most critical in authorization perspective, because of that hacker or unexpected outsider can access and control vehicle for any damage and casualty.

Based on three criteria, the authorization level can be categorized as follows.

- Required Authorization (Level 1)

  - No autonomous. Required approvals in every step

  - Visual activities, Self-damage

  - Vehicle access  including engine and transmission (e.g. engine ignition, air-conditioning, Autonomous Parking)

- Partially Authorized (Level 2)

  - Partial autonomous. At least should be approved by users

  - Visual activities, No self-damage

  - Vehicle access  excluding engine and transmission (e.g. light, horn & light, door lock, Electricity charging)

- Fully Authorized (Level 3)

  - Fully autonomous. No requirement for user notification

  - No visual activities, No self-damage

  - Information access and transfer only(e.g. fuel gauge, Navigation Location, Over-The-Air update)

Each component is granted to one of three levels and it can set their authorization level up to the granted level. For instance, if the navigation information is granted to level 3, it can set its authorization level 1, 2, or 3. However, if the electric charging is granted to level 2, it can only set its level 1 and 2, but not 3.

Figure 6.30.1-2: Example of Vehicle Remote Control for External Platform with AI Unit

Figure 6.30.1-2 is an example of vehicle remote control that the driver checks the fuel gauge of the vehicle using external platform. In this scene, AI speaker is a form of voice interface between driver and the platform. Assuming the fuel gauge information is with level 3, the vehicle and the platform communicates each other to transfer fuel gauge information. At final, the platform informs the driver with current fuel status of the vehicle.

    - Source

REQ-2018-0071R05- Use case- Vehicle Remote Control using External Platform

    - Actors

Vehicle

- Vehicle is a nomadic object monitored by M2M System node through telematics server. It is equipped with a telematics unit as a form of modem hardware. (e.g. 3G, LTE modem).

External Platform

- External platform contains the capabilities to control vehicle and access its information. It can be a form of device or server. Also, it can include an intelligence unit, such as artificial intelligence module linked to a smart speaker.

Telematics Server

- Telematics server is connected to a telematics unit in a vehicle for external communication. It also manages traffic and security controls of a telematics system

M2M Service Platform

- In charge of providing common functionalities for the M2M services.

    - Pre-conditions

Vehicle must contain telematics unit for network communication.

Telematics server and external platform must complete authentication process for integration with high security.

Vehicle command must contain control parameters and values for accurate control of the vehicle.

External platform must be linked to the device as an interface for owner’s command and response.

External platform may contain AI units for autonomous control and access.

    - Triggers

None.

    - Normal Flow

Figure 6.30.6-1: Normal Flow – Vehicle Remote Control using External Platform

- An owner requests an authentication process to a telematics server by sending Identification and password.

- Telematics server sends a response with other request for authorization approval to external platform.

- External platform processes the authorization and requests to telematics server for access token issue.

- Telematics server issues an access token to external platform.

- External platform sends an owner for authentication response completion.

- Owner sends voice command for vehicle control.

- External platform processes the voice command and sends a request of vehicle control to the telematics server. In some cases, handshaking process may be required.

- Telematics server sends a control command to the vehicle for command execution.

- Once the command is completed, the vehicle replies to the external platform for command result with error code.

- External platform sends a response to the owner for the result of the command. Before that, the platform may send a request to M2M Service Platform  for verification or validation in few cases.

    - Alternative Flow

None

    - Post-conditions

- None

    - High Level Illustration

Figure 6.30.9-1: High Level Illustration – Vehicle Remote Control using External Platform

    - Potential requirements

- The oneM2M System shall support the management and configuration of authorization level setting for device remote control, based on the device functionality .

The oneM2M System shall enable mechanisms to expose  device policies regarding access and communication  for device security and safety.

  - Vehicular Data Service with a vehicular data model and Edge/Fog Computing

    - Description

Connected vehicles collect data on the surroundings of the vehicle and in-vehicle data by its on-board cameras/sensors (e.g. video camera, LIDAR, GPS, OBU). In this use case, those collected data are transferred periodically to Fog Nodes for analysis and sent them to a Cloud Node for vehicle data services (e.g. vehicle maintenance, Dynamic Map, ADAS). However, the vehicle equips with many sensor devices which generate various types of data, and vehicle sensor data exists in different formats across automakers. It might take time to process them even if we apply for Fog Node deployments. If a vehicular data model is available, Fog Nodes may be able to categorize the data into some event types (e.g. rain condition, warning engine control, traffic congestion) and prioritize the event types for processing and/or transfer according to the policy of Service Providers. Therefore, the Fog Nodes can efficiently and accurately process the data, as well as handle low latency requirements of the Service Providers. This use case provides the vehicular data service using the vehicular data model and Edge/Fog Computing. Following sub-clause shows the summary of Sensor Data Ingestion Interface (SDII) [i.23] as an example of the vehicular data model.

      - Sensor Data Ingestion Interface (SDII)

Sensor Data Ingestion Interface (SDII) [i.23] is based on Vehicle Sensor Data Cloud Ingestion Interface Specification published in 2015 [i.20]. It describes data models for exchanging data between vehicles and cloud. Sensor data packages are contained in messages that may provide various type of content using data elements. The data elements may be grouped in different combinations. In the SDII, a message contains one Envelop and one Path. The message also optionally contains PathEvents and PathMedia.

The Envelop provides fundamental information about the individual sender (e.g. vehicle). The data may contain a version of interface specification in use, the name of the company that submits the data, vehicle metadata (length, width, height, primary/secondary fuel tank volume, etc), vehicle identifier, map information (map provider, map version, and map standard format, etc).

The Path is the logical data type containing one or more position estimates of the vehicle ordered by timestamp. The position estimates are ordered starting with the oldest position estimate towards the newest position estimate. It may contain timestamp, position type (GPS, etc), longitude, latitude, altitude, the speed of the vehicle (mps).

The PathEvents provide additional information along a Path. They may be singular events such as a change of vehicle operation mode or information about an electronic stability program event, and. They may also be continuously collected information such as velocity or curvature measurements. They can contain different types of events (e.g. vehicle status, environment status, detection of a traffic signal, vehicle dynamics, detection of road sign).

The PathMedia provides additional media content along a Path. It may collect information that refers to certain PathEvents, but it may also include content requested at a certain location. It may contain media type (e.g. image, video, audio), media format, duration of a media and view angle of a sensor. It can hold information for exactly one type of media, for example an image or a video clip of an image sensor.

Figure 6.31.1.1-1 shows the overview for SDII messages. For more information about each data element, see Sensor Data Ingestion Interface (SDII) [i.23].

Figure 6.31.1.1-1: Overview for SDII messages

    - Source

REQ-2018-0085R01- Vehicular data service with a vehicular data model and Edge/Fog Computing.

    - Actors

Cloud Node: It is the node which manages FNs and LFNs and interacts with Service Providers.

FN (Fog Node): It is the Node which computes, stores and analyzes data collected by sensors in a vehicle. It is located between the Cloud Node and vehicles. If a vehicle is equipped with a LFN, the FN aggregates the data sent by the LFN, and sends it to the Cloud Node.

LFN (Local Fog Node): It is the Node which computes, stores and analyzes data collected by sensors in a vehicle. It is deployed in a vehicle as a gateway.

Service Provider: It provides its own services for vehicles such as automobile dealers.

    - Pre-conditions

Vehicles, LFNs, FNs and Cloud Node support a vehicular data model.

The prioritization for data processing/transfer has been assigned to each event type represented by the vehicular data model according to the policy of the Service Provider.

    - Triggers

None.

    - Normal Flow

- Figure 6.31.6-1 illustrates the high-level flows of the use case, which consists of the following steps:

Step 1: The vehicle collects data on the surroundings of the vehicle and in-vehicle data by their on-board cameras/sensors (e.g. video camera, LIDAR, GPS, OBU).

Step 2a: The vehicular sensors send the collected data in Step1 to a LFN.

Step 2b: The vehicular sensors send the collected data in Step1 to a FN.

Step 3a, 3b: The LFN/FN categorizes the data into some event types associated with the vehicular data model, and prioritizes them for processing and/or transfer according to the policy of the Service Provider. For example:

  - Rain condition: It may be represented by a precipitation type and/or a vehicular wiper state (e.g. fast). The event type may be categorized as a low-priority.

  - Warning engine control or battery charging alert: It may be represented by maintenance light status. The event type may be categorized as a hign-priorty.

  - Traffic congestion: It may be represented by the speed of a vehicle (e.g. slow down) and an object recognition (e.g. preceding vehicles). The event type may be categorized as a medium-priority.

Step 4a: The LFN sends the data to the Cloud Node according to the priorities in Step 3a.

  - When the event type indicates a low-priority, the LFN stores the data in their temporary storage and transfer the data to the Cloud Node via the FN once an hour or at a low network congestion area.

  - When the event type indicates a high-priority, the LFN sends the data to the Cloud Node via FN immediately.

  - When the event type indicates a medium-priority, the LFN sends the data to the Cloud Node via the FN on the best effort basis.

Step 4b: The FN sends the data to the Cloud Node according to the priorities in Step 3b.

  - When the event type indicates a low-priority, the FN stores the data in their temporary storage and transfer the data to the Cloud Node once an hour or at a low network congestion area.

  - When the event type indicates a high-priority, the FN sends the data to the Cloud Node immediately.

  - When the event type indicates a medium-priority, the FN sends the data to the Cloud Node on the best effort basis.

Figure 6.31.6-1: Normal Flow - Vehicular data service with a vehicular data model and Edge/Fog

    - Alternative Flow

None

    - Post-conditions

The Cloud Node handles the data in Step 4a/4b according to the policy of the Service Provider (e.g. sending the data to the Service Provider and/or other FNs).

    - High Level Illustration

Figure 6.31.9-1: High Level Illustration - Vehicular data service with a vehicular data model and Edge/Fog

    - Potential requirements

- The oneM2M system shall enable Edge/Fog Nodes to support the data delivery and processing based on the event prioritization associated with an information model.

  - Protocol Selection in Gateway for Vehicle

    - Description

Vehicle has many different communication and application interface protocols to communicate with external platforms. Throughout physical layers to application layers, vehicle has different characteristics as compared to conventional devices like smartphone and small Internet of Things (IoT) devices. First of all, vehicle must assure its security due to the fact that the first priority of the vehicle is to offer transportation capability. Any defection in the vehicle can cause human injury or casualty even in very small malfunction in the vehicle. In addition, vehicle has to assure that it can operate under any weather, traffic circumstance such as rainy/snowy weather, high speed/temperature situations. In order to handle all of those, vehicle must have capabilities of environmental endurance and, especially, adaptive communication abilities.

In order to adaptively change communication modules and guarantee its own security, it is reasonable that vehicle needs to apply multiple different interface protocols to communicate with external services and application. To secure the vehicles from hackers and intruders, the vehicle also needs a gateway system that handles traffic condition between services and vehicles, and prevents hacking intrusion attacks towards vehicles. Its overall structure is shown on Figure 6.32.1-1.

Figure 6.32.1-1: Example oneM2M Structure of Protocol Selection in Gateway for Vehicle

Figure 6.32.1-1 shows the connection between applications and vehicles. Once the application is connected to the platform, the application sends information through platforms and gateway for the vehicle. Note that the vehicle has no prior-information about application, and vehicle initializes an agreement of protocol selection with the gateway.

However, some of the services need constant streaming and pre-selected protocol may not be sufficient to communicate between applications and vehicle. In addition, if the traffic cost becomes a matter to maintain infrastructure, high data rate protocol can result the high cost of operation and management. In order to resolve the issues, M2M platform may need to adaptively select suitable protocols based on the throughput rate from application. We have two scenarios for this.

1. Music streaming case

In case of music streaming, streaming connection has to be constant and persistent. If the protocol supports only TCP, it is not suitable for streaming cases since the quality of service is not determined by guaranteed receives. In that case, the protocol needs to switch from TCP to UDP.

2. Home monitoring case

Home monitoring case is also another case that intrusion recording has to be sent to the vehicle. If the protocol does not sufficiently support to transmit large quantity of data such as GSM network, it has to switch to 3G or LTE. Note that the type of data can be any kind of format including binary data.

    - Source

REQ-2018-0089R02- Protocol selection in gateway for vehicle

    - Actors

Vehicle

- Vehicle is an entity monitored by M2M System. It is equipped with a telecommunication unit as a form of modem hardware. (E.g. 3G, LTE modem).

Application

- Application is the entity monitored by M2M platform. This application provides services for the vehicle. Note that this application can be various from chatting to video streaming.

Gateway

- Gateway is the collection of protocols saved for vehicle communication. Note that the protocol can be updated and other protocol can be applicable when the vehicle has the capability of doing that.

M2M Platform

- In charge of providing common functionalities for the M2M services.

    - Pre-conditions

Vehicle must contain communicative modules that externally connect to other platforms.

Vehicle must be connected to the gateway for communication.

    - Triggers

- None.

    - Normal Flow

Figure 6.32.6-1: Normal Flow – Protocol Selection in Gateway for Vehicle

- Once Application is connected to M2M Platform, it sends the data.

- After M2M Platform receives the data, it analyses headers, data types and anticipated throughput rate of it. Based on the analysis result, it determines the optimal protocol in the gateway.

- M2M platform sends the request to the gateway for deployment of selected protocols.

- Gateway sends the request of select protocols to establish the new connection of selected protocols

- Vehicle analyses the request, and decide the usage of selected protocols.

- Once the decision is made, the vehicle sends the response to the gateway for the decision of the protocols.

- Gateway sends the response to M2M platform for the usage of protocols.

- M2M platform establishes the new protocol connection to the vehicle.

- Once the connection is established, application streams the data to the vehicle.

    - Alternative Flow

None

    - Post-conditions

- None

    - High Level Illustration

Figure 6.32.9-1: High Level Illustration – Protocol Selection in Gateway for Vehicle

    - Potential requirements

- 1) 	The oneM2M System shall enable infrastructure to connect multiple protocols to devices for sustainable device connection. (OSR-100)

- 2) 	The oneM2M System shall support the capability of selecting communication protocols between entities in a flexible manner.

- 3) 	The oneM2M System shall support dynamic management and configuration of protocols in entities for sustainable device connection.

- 4) 	The oneM2M System shall enable infrastructure to connect multiple underlying networks to devices for sustainable device connection. (OSR-100)

- 5) 	The oneM2M System shall support the capability of selecting underlying networks between entities in a flexible manner.

- 6) 	The oneM2M System shall support dynamic management and configuration of underlying networks in entities for sustainable device connection.

  - Software-Defined Radios Application for Vehicle

    - Description

Software Defined Radio (SDR) is the radio device that can be updated by software packages for their internal radio module such as modulation, sampling rate, frequency shifting, and so on. By doing this, the radio can secure their flexibility encompassing a wide range of radio techniques, and adaptability given their circumstances including underground, tunnel, or heavy network traffic condition.

Since the vehicle radio standard becomes various and costly to apply in a form of hardware, there is a need for a radio module that can handle various radio techniques. SDR is able to be one of the solutions for that. However, we have to overcome a few obstacles before implementing SDR in the vehicle.

At first, the concept of SDR is to develop the communication modules as software packages, and the range of modules does not include radio front-end, such as antenna, carrier frequency oscillators, and maximum sampling rate. This hardware limitation has to be considered before applying SDR. However, most of mobile and vehicle communication standards such as LTE, V2X, WAVE, do not require excessive performance of hardware as compared to current hardware level. It means existing hardware still can deal with those communication standards.

Other obstacle is for the software update and switch. An SDR module in the radio has to be adaptive given situations, and the radios need to update or switch their communication modules by changing their radio software packages. In other words, unless certain triggering is made from base centres or the radio itself, the SDR module will not make any change at all.

In order to overcome this situation, we write use cases of the vehicle where the mounted SDR module is updated given certain network situation. In this case, we have two specific scenarios, and the whole structure is shown on Figure 6.33.1-1.

Figure 6.33.1-1: Example Structure of SDR Application for Vehicle

1. Heavy network situation

In case of vehicle populated area, the situation of specific communication traffics can be very crowded, and it often causes network congestion and shutdown in the worst case. In order to avoid that situation, the vehicle should be able to distribute their traffic to other communication systems so that the network traffic can be eased and sustained. In that case,

i) Vehicle receives the network traffic condition, and determines the standard by analysing the throughput rate and spectrum condition

ii) Once it is determined, the vehicle decides to update software of SDR module to that standard in the vehicle. After update, the vehicle re-confirms the software update to make sure it is the right version.

iii) After re-confirmation, it communicates with M2M platform and notify about its change of SDR software and M2M platform notifies that to M2M infrastructure.

2. Limited infrastructure situation

In some rural and no-populated area, the lack of infrastructure can disconnect the vehicle communication at all. The desert or agricultural fields are the prime examples of very few infrastructures. To solve this problem, other communication system such as EDGE or satellite communication, can be options for that situation.

i) Vehicle receives the network traffic condition, and determines how many base stations and infrastructure existed in that area.

ii) Once it is determined, the vehicle decides the viable standard that has maximum base station near around the vehicle, and sends the software update requests to SDR module in the vehicle. After update, the vehicle re-confirms the software update to make sure it is the right version.

iii) After re-confirmation, it communicates with M2M platform and notify about its change of SDR software and M2M platform notifies that to M2M infrastructure.

To handle both cases, M2M platform reserves the trigger to either switch or update software of the network. When the M2M platform sends the trigger to the vehicle, the vehicle determines the types of triggers, and starts to switch to the different communication systems or request the new software for the vehicle. Note that once the new software request has to be one of the software packages existed in the M2M platform database.

    - Source

REQ-2018-0090R02- Use case: Software Defined Radio Application for Vehicle

    - Actors

Vehicle

- Vehicle is an entity monitored by M2M System. It is equipped with a telecommunication unit as a form of modem hardware. (E.g. 3G, LTE modem).

Software-Defined Radio (SDR) application

- SDR application is the application monitored by the middle node in the vehicle. This application handles the update and switch of communication systems of the vehicle. Note that low-level firmware is updated to the underlying network entity through Mcn and this software is connected to the firmware for the communication.

M2M Infrastructure

- In charge of providing common functionalities for the M2M services.

M2M Service Platform

- In charge of providing common functionalities for the M2M services.

    - Pre-conditions

Vehicle must contain communicative modules that externally connect to other platforms.

Vehicle must be capable of updating software of the device.

Vehicle must verify that the software which is about to be updated has no risk of hacking from outbound.

Vehicle must contain service entity, network entity and application entity for software update.

    - Triggers

- None.

    - Normal Flow

Figure 6.33.6-1: Normal Flow – SDR Application for Vehicle

- M2M Infrastructure sends the vehicle network traffic notice to M2M platform if there is any significant change happens.

- M2M platform sends a traffic condition notice of vehicle network to the vehicle node in vehicle.

- Using the information in the notice, the vehicle node in vehicle conducts the analysis to decide the software update of SDR module in the vehicle.

- If decided, the node sends the requests of SDR update/switch to SDR application for the change.

- SDR application sends a response with SDR info including SDR version and types to the node.

- The vehicle node sends the request to SDR Modem (Network) for SDR change.

- Once SDR Modem (Network) receives the request, it processes SDR changes.

- After change is completed, SDR Modem (Network) sends the completion response to the node.

- The vehicle node notifies SDR application and SDR application feedbacks with re-confirmation response.

- The vehicle node in vehicle sends the response to the M2M platform for communication node change.

- M2M platform sends the response to M2M infrastructure about the communication module situation of the vehicle node in the vehicle.

    - Alternative Flow

None

    - Post-conditions

- None

    - High Level Illustration

Figure 6.33.9-1: High Level Illustration – SDR Application for Vehicle

    - Potential requirements

- 1) 	The oneM2M System shall support the management and configuration of software module in network entity settings for device control and communication

- 2) 	The oneM2M System shall support methods for managing and configuring the change of the underlying network interfaces.

  - Dashcam Application

    - Description

A dashcam, also known as vehicle traveling data recorder, is an onboard camera that continuously records the view through a vehicle's front windscreen, sometimes through rear window, and sometimes inside the car.

A dashcam usually has two kinds of storage medium, one is built-in flash memory being used for buffering the video data, and another is external flash memory (usually 16 GB or higher MicroSDHC or MicroSDXC cards) for storing video data. Once the storage capacity is full, dashcams usually erase the oldest data and store the latest records.

Dashcams are usually made to provide video evidence in the event of a road accident. When crashed, dashcams can record video of the accident, and keep recording a short time video before and after the timepoint of the accident. When parked, dashcams can capture video and picture evidence if vandalism is detected, and some key video and picture recordings could be automatically sent to mobile phones once the phone connected to the dashcam (usually employing WiFi). These features are described in this use case of dashcam application.

    - Source

- REQ-2018-0094R01-Use Case of Dashcam Application

    - Actors

Dashcam Device

- This is a M2M Device installed inside the car, recording the view through a vehicle's front windscreen and sometimes rear window. Additionally, this device can have a G-sensor inside.

Camera Module

- This is a M2M Entity sending video data to the Dashcam Device continuously.

G-sensor

- This is a M2M Entity within the Dashcam Device. After receiving the request of the sensor, related data are not overwritten by the new data.

Use Equipment

- This is a M2M Entity requesting dashcam video and interacting with the dashcam.

    - Pre-conditions

None

    - Triggers

In a car crash or hard brake scenario, the gravity sensor can trigger this use case.

    - Normal Flow

- Figure 6.34.6-1: Normal Flow – Dashcam Application

- Registration for all necessary entities.

- (As the device is working, dashcam records the video data sent from the camera.)

- A hard brake situation occurs. G-sensor detects the hard braking, sends a request to Dashcam which wants this video clip to be locked.

- Based on the request, the dashcam locks corresponding data.

- (As the device is working, dashcam records the video data sent from the camera.)

- If the capacity of the memory inside the dashcam is full, then the retention policy like maxByteSize goes into effect. But according to the lock setting of the data, some specific data would not be overwritten by the new data, ensuring locked data is protected from being erased.

- Driver’s equipment connects the dashcam, retrieving his interested video clip on the dashcam memory automatically to his use equipment.

- Once the locked data is considered to be useless, the corresponding lock settings are removed.

    - Alternative flow

None.

    - Post-conditions

None.

    - High Level Illustration

None.

    - Potential requirements

oneM2M shall support retention of data for protecting specific content resource based on the retention policy in effect.

  - Volatile Data Management for Instant IoT Services

    - Description

Currently, oneM2M system supports expiration timer to delete the resource from the Hosting CSE after a certain amount of time/date. This attribute can be used by IoT services utilizing temporary access. Using the expiration timer, a resource can be deleted after a given time duration. However, the current oneM2M mechanism deleting a specific resource is very limited to support various emerging IoT/M2M applications.

Now a days many IoT or new services supporting self-deleting feature because of security and privacy reason. Several examples are listed below:

- “Snapchat” similar IoT applications: if received data is read by user, the data is deleted within 10 sec. if not read by the user, data is deleted after 24 hours.

- Security & Privacy related IoT services:  any data associated with personal information shouldn’t be shared by others. Data shouldn’t be stored in a platform.

- Medical and wearable IoT services: only a dedicated doctor can read medical data from the platform. Then the data should be removed.

As many IoT services need a feature handling some data with an instant manner, volatile data management function can be considered as a common function.

    - Source

RDM-2019-0046R01-Use_case_for_Volatile_Data_Management_Service

    - Actors

Vehicle: An application sending various data generated from a vehicle and driver.

Cloud Node: A node that handles and manages data from vehicles.

Instant Application: An application that consumes data from the Cloud Node.

    - Pre-conditions

A vehicle is equipped with sensor and device to collect data from the driver and the vehicle.

Data stored in the Cloud IoT platform should be consumed by a designated application.

Data annotated with a volatile feature should be stored in a platform after a certain criteria is satisfied. For example, data should be removed from the platform after it is read by the designated application.

    - Triggers

A vehicle measures various data from its internal sensors and sends the measured information to the IoT cloud platform with additional information classifying volatile data and criteria for self-deletion.

    - Normal Flow

- Figure 6.x.6.1 illustrates the high-level flows of EDR service with volatile feature use case, which consists of the following steps:

- Step 001: IoT sensors equipped with a moving vehicle send measured data to the IoT platform with a volatile feature that is comprised of indication of volatile data type, criteria describing the condition for performing self-deletion and access right.

- Step 002: IoT service layer platform creates the request resource to store measured data. Data configured with the volatile feature should be distinguished from other normal resources so that these resource can be deleted once the given condition is satisfied. For example, the criterial can be given such as “Delete the resource when just after the resource is read by Application #2”.

- Step 003: IoT service layer platform returns response message to Application #1

- Step 004: Application #2 retrieves the data from the resource previously configured with the volatile feature.

- Step 005: IoT service layer platform returns response message containing measured data to Application #2. IoT service layer platform increases a read counter for the resource. This read counter is an additional attribute for the resource to provide information how many times read by consumers.

- Step 006: As the resource is configured with the volatile feature and the condition is satisfied (i.e., the resource is read by Application #2 that has a proper access right to the resource), the IoT service layer platform delete the resource.

- Figure 6.35.6-1: Normal flow for volatile Event Data Recorder (EDR) data management service

    - Alternative Flow

None

    - Post-conditions

- None

    - High Level Illustration

Figure 6.35.9-1: High level illustration of EDR data management with volatile feature

    - Potential requirements

- oneM2M System shall be able to track the number and types of operations on oneM2M resources

oneM2M System shall be able to delete a resource based on resource deleting policy (e.g., delete a resource when the resource is read by a specific application)

  - Offloading Service Continuity between Edge/Fog Nodes

    - Description

Vehicular application require fast and reliable communications to ensure safety in very dynamic scenarios (e.g. highway driving). In such conditions supporting continuity of service relies upon optimizations throughout the platform or system, including deployments using offloading at Edge/Fog nodes.

For example, in a VRU (Vulnerable Road User) situation where a vehicle communicates temporarily with Edge/Fog Node A .

The Edge/Fog Node A receives data from the Cloud Node by offloading service. The Edge/Fog Node A analyses the road information data (e.g. other vehicles, pedestrian) which is collected by the vehicle. When the vehicle moves from the Edge/Fog Node A to the Edge/Fog Node B, the offloaded instances running on Edge/Fog Node A should be moved to Edge/Fog Node B without affecting the service. At the same time, when the vehicle is connected to Edge/Fog Node B, it should receive the service from Edge/Fog Node B not from Edge/Fog A.

Edge/Fog offloading should be enabled in order to reduce communication loads and reduce latencies while also ensuring service continuity. by enabling direct communication between adjacent Edge/Fog nodes and synchronization with the central cloud platform .

    - Source

RDM-2019-0048R03 Use case of Offloading Service Continuity between Edge/Fog Nodes

    - Actors

Vehicle: Mobile unit travelling along  the road.

Edge/Fog Node: Node which computes, stores and analyses data. It is located between Cloud Nodes and end devices.

Cloud Node: Centralized Nodes which manages Edge/Fog Nodes, maintains database of Edge/Fog Nodes and interacts with Application Provider.

RSU: It is located along vehicular paths and provides connection between vehicles and hosts an Edge/Fog Node in a RSU network.

    - Pre-conditions

A vehicle is equipped with sensor for road data collect and analysis and communication capabilities.

Service-related resources are offloaded to RSU hosting Edge/Fog A by the IoT Cloud Node.

    - Triggers

When the vehicle moves from the Edge/Fog Node A to the Edge/Fog Node B, the Edge/Fog Node A sends the data to the Edge/Fog Node B directly without passing the Cloud Node.

    - Normal Flow

- The Cloud Node offloads service information to the Edge/Fog Node A.

A vehicle collects new data on the road and sends the data to the RSU hosting Edge/Fog Node A who analyses the road information data (e.g. other vehicles, pedestrian) which is collected by the vehicle.

As the vehicle comes in range of other RSUs, the Cloud Node and Edge/Fog Node A make a decision to move offloaded resources to Edge/Fog Node B Then the Edge/Fog Node A sends the data to Edge/Fog Node B.

The Edge/Fog Node A sends some parts of collected data to the Cloud Node for synchronization.

The Cloud Node offloads the Edge/Fog Node information to the Edge/Fog Node B to support offloading with service continuity.

The Edge/Fog Node B sends the data to the application on the moving vehicle.

The vehicle collects another new data on the road and sends the data to RSU hosting Edge/Fog Node B.

After the Edge/Fog Node B analyses the road information data which is collected by the vehicle, the Edge/Fog Node B sends the data to Edge/Fog Node C.

The Edge/Fog Node B sends the data to the Cloud Node for synchronization.

The Cloud Node offloads the Edge/Fog Node information to the Edge/Fog Node C.

The Edge/Fog Node C sends the data to the moving application.

The Edge/Fog Node C sends the data to the Cloud Node for synchronization.

Figure 6.36.6-1: Normal Flow – Offloading with Service Continuity between Edge/Fog Nodes

    - Alternative Flow

None

    - Post-conditions

- None

    - High Level Illustration

- Figure 6.36.9-1: High Level Illustration - Offloading with Service Continuity between Edge/Fog Nodes

    - Potential requirements

- The oneM2M System shall enable migration of data and context information between Edge/Fog Nodes for continuous services support.

- The oneM2M System shall enable synchronization of data between Edge/Fog Nodes and Cloud Node when migrating data and context information between Edge/Fog Nodes.

  - Handling Data Protection-related Regulations

    - Description

Data protection (in particular personal data) is one of important issues to be managed by data handling systems including IoT service layer platforms. In order to protect privacy related data, many countries prepare and mandate personal data protection related regulations such as EU’s General Data Protection Regulation (GDPR) and Korea’s Personal Information Protection Act (PIPA).

- GDPR: The General Data Protection Regulation is a regulation in EU law on data protection and privacy for all individual citizens of the European Union and the European Economic Area. Business processes that handle personal data must be designed and built with consideration of the principles and provide safeguards to protect data (for example, using pseudonymization or full anonymization where appropriate), and use the highest-possible privacy settings by default, so that the data is not available publicly without explicit, informed consent, and cannot be used to identify a subject without additional information stored separately.

- PIPA: The Personal Information Protection Act (PIPA) is a framework act on data protection both in the public sector and private sector in Korea. The act is applied to various areas as follows:

  - the Act on Promotion of Information and Communications Network Utilization and Information Protection, etc.

  - the Act on the Use and Protection of Credit Information

  - the Act on the Protection, Use, etc. of Location Information

  - the Act on the Development of Cloud Computing and the Protection of its Users

  - and so on..

After GDPR is mandated, any web-based services handling privacy data (such as subscription based web-service, online banking) have applied techniques (getting users’ agreement and pseudonymization of privacy data) to their website to follow the GDPR.

Currently, oneM2M system supports pseudonymization technique on the address of oneM2M resources but not on data and name of resource. Therefore, oneM2M system needs to consider handling different countries’ data protection related regulations as a common function.

    - Source

RDM-2019-0067R02-Handling privacy related regulation (Requirements)

    - Actors

Vehicle: An application sending various data generated from a vehicle and driver.

Cloud Node: A node that handles and manages data from vehicles.

    - Pre-conditions

A vehicle is equipped with sensor for road data collect and analysis and communication capabilities.

Service-related resources are offloaded to RSU hosting Edge/Fog A by the IoT Cloud Node.

    - Triggers

- A vehicle measures various data from its internally equipped sensors, a driver and passengers. As data from the vehicle contains personal data from the driver or passengers, the IoT service cloud platform process these personal data based on a mandated regulation once it receives such data.

    - Normal Flow

- Figure 6.37.6-1 illustrates the high-level flows of a use case showing how IoT service platforms process personal data based on a data protection related regulation (e.g., GDPR), which consists of the following steps:

Step 001: IoT sensor (i.e., Application #1) in a car measuring personal data (e.g., driving patterns or health condition) sends measured personal data to the IoT platform, which should handle personal data in compliance to a proper data protection related regulation. The message from the application includes which regulation need to be applied and a protection mechanism such as pseudonymization.

Step 002: IoT service layer platform creates the requested resource with the information how to process personal data. The created resource is configured with the indication about data protection regulation and mechanism. For example, a regulation is ‘gdpr’ and a mechanism is pseudonymization.

Step 003: IoT service layer platform returns response message to Application #1

Step 004: Application #1 sends a request to create a resource which is a placeholder for new data measurement.

Step 005: As the requested resource contains personal data that falls under GDPR IoT service layer platform process data with the specified mechanism (in this case pseudonymization). The created resource then contains pseudonymized data instead of raw input data.

Step 006: IoT service layer returns response message to Application #1.

Step 007: Application #2 tries to retrieve the created resource.

Step 008: The pseudonymized data in the requested resource is returned to Application #2.

Figure 6.37.6-1: Normal Flow –Managing privacy data that need to be handled based on a data protection regulation

    - Alternative Flow

None

    - Post-conditions

- None

    - High Level Illustration

Figure 6.37.9-1: High level illustration of privacy data protection management based on mandated regulation

    - Potential requirements

The oneM2M system considers regulations on handling privacy related data and applying proper data protection techniques (such as anonymization and pseudonymization) to personal identifiable information.

  - Vehicle Idling Alarming Service

    - Description

When a vehicle using a push button starter does not turn off its engine in a parking lot harmful exhaust fume from the vehicle is emitted, which have been linked to many human health problems such as cancer and asthma. The vehicle idling (i.e. running a vehicle’s engine while it is in stationary) causes exhaust gas poisoning and air pollution. It is one of the important environment issues in many countries. Therefore, many countries have regulations limiting idling of some or all vehicles. Nowadays, several car makers provide a function called “automatic ignition shut-off”, which automatically shut-off the engine when the vehicle is at rest during the given amount of time from the user. However, if the user is not staying in the vehicle, there is no way to know whether the operation is successful.

Therefore, if the engine of vehicle is “on” in a parking lot during the configured amount of time and the automatic ignition shut-off function is properly executed, the driver better to know about the status of the execution. For example, when the time for Idling is set to 5 minutes, the user receives an alarm about the Idling via the IoT platform.

For this automatic ignition shut-off, an idling vehicle sends the data (e.g. status of engine, idling time) to the IoT platform periodically. Then the IoT platform internally checks the current idling time with the configured the idling time of the vehicles (e.g. idling duration more than 5 minutes). If the condition is satisfied, the IoT platform sends an alarm to the user’s device about the result of the automatic ignition shut-off execution.

-

Figure 6.38.1-1: Vehicle Idling problem -

    - Source

RDM-2019-0089R01-Use_case_of_vehicle_idling_alarming_service

    - Actors

Vehicle: A mobile unit travelling along the road.

IoT Platform: A platform collecting data related to vehicle idling time.

User: A driver who receives an alarm about the status of vehicle idling.

    - Pre-conditions

The data (e.g. ID) of vehicle is registered in the IoT platform.

The user sets the notification condition about idling time of the vehicle in the IoT platform.

The vehicle can confirm its idling status.

    - Triggers

If the vehicle idles more than pre-defined time, the notification condition is satisfied. When the condition of the notification is satisfied, the IoT platform notifies to the user’s device.

    - Normal Flow

- The user subscribes to notify the idling time of the vehicle to the IoT platform if the notification condition is satisfied.

- The vehicle sends the data (e.g. status of engine, idling time) to the IoT platform periodically.

- The IoT platform checks the idling time of the vehicle and notification condition (e.g. idling duration more than 5 minutes).

- If the condition is satisfied, the IoT platform notifies to the user’s device.

Figure 6.38.6-1: Normal Flow –-

-Vehicle Idling Alarming Service-

    - Alternative Flow

None

    - Post-conditions

- None

    - High Level Illustration

Figure 6.38.9-1: High Level Illustration – Vehicle Idling Alarming Service

    - Potential Requirements

- The oneM2M system shall enable notification when the condition is maintained for pre-defined time.

- Overview of Potential Requirements

Potential requirements from all vehicular domain use cases collected in this technical report are summarized as follows,

- The M2M application System shall enable the M2M Devices to exchange M2M application to diagnostic data periodically with the M2M Application in the network domain ([i.2] OSR-118).

NOTE 1:	This requirement addresses the use case 6.1.

The M2M System shall enable the M2M Application to configure the notification interval in the M2M Devices ([i.2] OSR-112).

NOTE 2:	This requirement addresses the use case 6.1.

The M2M system shall support a mechanism to describe the syntax and semantics format of the M2M application diagnostics data exchanged between the M2M Devices and the M2M Application in the network domain ([i.2] OSR-119).

NOTE 3:	This requirement addresses the use case 6.1.

The M2M service shall be able to provide the mechanism for authorization for integrity-checking and installing processes of software/hardware/firmware component(s) on M2M Device(s) ([i.2] SER-013).

NOTE 4:	This requirement addresses the use case 6.2.

The M2M system shall be able to support authentication using device key on the integrity check for M2M Device(s) ([i.2] SER-073).

NOTE 5:	This requirement addresses the use case 6.2.

The M2M Device shall be able to support HSM (Hardware Security Module) to protect its integrity depending on the security level requirement ([i.2] SER-023).

NOTE 6:	This requirement addresses the use case 6.2.

A M2M System shall support communication between M2M Platform and a M2M device either directly or via a gateway ([i.2] OSR-113).

NOTE 7:	This requirement addresses the use case 6.3.

A M2M System shall be able to exchange information between M2M applications via M2M Platform ([i.2] OSR-114).

NOTE 8:	This requirement addresses the use case 6.3.

A M2M System shall be able to take actions according to the received service requests from M2M Applications.

NOTE 9:	This requirement addresses the use case 6.3.

A M2M system shall be able to support service requests from M2M applications for communication with QoS requirement, such as, higher delivery priority, reliable delivery, etc. ([i.2] OSR-115).

NOTE 10:	This requirement addresses the use case 6.3.

A M2M System shall support mutual-authentication among M2M device, M2M gateway, M2M platform and M2M Application ([i.2] SER-040).

NOTE 11:	This requirement addresses the use case 6.3.

The information sent by a M2M device or the M2M platform or a M2M application shall use cryptographic technology to ensure information authentication and information integrity ([i.2] SER-068).

NOTE 12:	This requirement addresses the use case 6.3.

A M2M system shall permit information being provided in anonymous way ([i.2] SER-074).

NOTE 13:	This requirement addresses the use case 6.3.

A command issued by a M2M System shall be able to have time expiration or geography restriction ([i.2] OSR-116).

NOTE 14:	This requirement addresses the use case 6.3.

Provisioning, installation, configuration and registration method of terminal system:

Especially for the case of overlapping two different system for DTG management system (owns and manages the device) and the application system using DTG data (utilizing the data from the device) ([i.2] OSR-134).

NOTE 15:	This requirement addresses the use case 6.4.

DTG/FMS data storing method and delivery protocol:

There is no dominant standard specifying data formats and protocols for vehicle related applications.

NOTE 16:	This requirement addresses the use case 6.4.

Vehicle location based service method:

M2M service platform is expected to provide the service capability supporting location based service ([i.2] OSR-120).

NOTE 17:	This requirement addresses the use case 6.4.

Control, configuration, error logging, and management method for the terminal system Over-The-Air:

M2M service platform is expected to provide the service capability supporting the Over-The-Air management ([i.2] OSR-121).

NOTE 18:	This requirement addresses the use case 6.4.

The M2M system shall provide the capability for an M2M device to maintain registration with multiple entities simultaneously ([i.2] OSR-122).

NOTE 19:	This requirement addresses the use case 6.5.

The registration shall be able to include information that identify the peer entity, and other information such as its management privilege, subscription etc., that are necessary for the conduct of the respective peer relationships([i.2] OSR-135).

NOTE 20:	This requirement addresses the use case 6.5.

It shall be possible for some registrations to hold the complete set of information context about the peer entity. This is referred to as "full registration"([i.2] OSR-136).

NOTE 21:	This requirement addresses the use case 6.5.

It shall be possible for some registrations to hold only a subset of information context about the peer entity. This is referred to as "lightweight registration"([i.2] OSR-137).

NOTE 22:	This requirement addresses the use case 6.5.

It shall be possible for "lightweight registration" at different entities that pertain to a common peer entity, to hold different sets of information, if needed, about the common peer entity([i.2] OSR-138).

NOTE 23:	This requirement addresses the use case 6.5.

It shall be possible to correlate the "full registration" and the "lightweight registration" that pertain to a common peer entity([i.2] OSR-139).

NOTE 24:	This requirement addresses the use case 6.5.

It shall be possible to distinguish the "full registrations" and the "lightweight registrations" that pertain to a common peer entity([i.2] OSR-140).

NOTE 25:	This requirement addresses the use case 6.5.

The M2M service platform shall be able to support the time-based policies to access the Underlying network ([i.2] CMR-014).

NOTE 26:	This requirement addresses the use case 6.6 "Use cases for Taxi Advertisement".

The oneM2M System shall enable discovery of M2M Application Servers, M2M Management Servers and M2M Devices available to an M2M Gateway for data exchange ([i.2] OSR-086).

NOTE 27:	This requirement addresses the use case 6.7.

The oneM2M System shall enable discovery of M2M Gateways available to a M2M Management Server and an M2M Device for data exchange ([i.2] OSR-087).

NOTE 28:	This requirement addresses the use case 6.7.

The oneM2M System shall be able to support the capabilities for data repository (i.e. to collect/store) and for data transfer from one or more M2M Devices or M2M Gateways, for delivery to one or more M2M Gateways via M2M Area Network without any assistance or instruction of M2M Management Servers and M2M Application Servers ([i.2] OSR-088).

NOTE 29:	This requirement addresses the use case 6.7.

Upon request from M2M Application Server, an M2M Gateway or M2M device shall enable functions that pre-process (e.g. average) M2M data before providing them to the recipient ([i.2] 0SR-104).

NOTE 30:	This requirement addresses the use case 6.7.

Upon request, an M2M Gateway or M2M device shall enable functions that erase M2M data (e.g. that have been sent or could not be sent to the recipient within a certain time) based on criteria from an M2M Application ([i.2] OSR-105).

NOTE 31:	This requirement addresses the use case 6.7.

An M2M Gateway and/or an M2M Device shall be able to broadcast to all M2M Devices and/or M2M Gateways in the vicinity its need to receive/deliver specific data ([i.2] OSR-106).

NOTE 32:	This requirement addresses the use case 6.7.

M2M Gateway and/or M2M Device shall be able to establish a connection to each other if it is able to receive/deliver the required specific data ([i.2] OSR-107).

NOTE 33:	This requirement addresses the use case 6.7.

The oneM2M System shall enable M2M Gateways to set conditions used for processing jointly data subscriptions and distribute the resulting notifications according to the set conditions ([i.2] OSR-108).

NOTE 34:	This requirement addresses the use case 6.7.

The oneM2M System shall enable subscriptions to changes to multiple resources which aim to generate notifications if and only if the expected changes to those resources occur concurrently ([i.2] OSR-109 and OSR-110).

NOTE 35:	This requirement addresses the use case 6.7.

OneM2M System shall be able to send the information to intended vehicles by unicast, multicast and/or broadcast ([i.2] OSR-123).

NOTE 36:	This requirement addresses the use case 6.8.

oneM2M System shall be able to securely transfer the information by using an appropriate method such as digital signature ([i.2] SER-069).

NOTE 37:	This requirement addresses the use case 6.8.

oneM2M System shall be able to transfer the information on real-time basis for feeding back current road states to automatic driving control. The feedback time should be less than a few seconds (the distance between vehicles normally corresponds to a few seconds) to avoid unnecessary speed down/stop of following vehicles ([i.2] OSR-124).

NOTE 38:	This requirement addresses the use case 6.8.

OneM2M system shall be able to guarantee its reliability in order to receive/feedback massages from/to related vehicles ([i.2] OSR-125).

NOTE 39:	This requirement addresses the use case 6.8.

oneM2M System shall enable sharing of service information between devices/GWs based on proximity ([i.2] OSR-126).

NOTE 40:	This requirement addresses the use case 6.8.

oneM2M System shall enable sending and receiving of service information between devices/GWs with minimized interruption ([i.2] OSR-127).

NOTE 41:	This requirement addresses the use case 6.8.

The oneM2M System shall enable the cancellation of continuous data collection and/or the deletion of collected data when pre-defined conditions are met ([i.2] OSR-089).

NOTE 42:	This requirement addresses the use case 6.9 "Use Case on Vehicle Data Wipe Service".

The oneM2M System shall enable pre-defined conditions to be protected from unauthorized modification ([i.2] SER-050).

NOTE 43:	This requirement addresses the use case 6.9.

The oneM2M System shall enable the deletion of M2M data produced/stored by the M2M Devices/Gateways based on request from an authorized entity ([i.2] SER-051).

NOTE 44:	This requirement addresses the use case 6.9.

The oneM2M System shall support reporting of Geo-Fence based Location event of the target M2M Device to the M2M Application based on the Application's configuration. ([i.2] OSR-047)

NOTE 45:	This requirement addresses the use case 6.10 "Vehicle Management based on Geo-Fence".

The oneM2M System shall support the M2M Application setting the configuration for Geo-Fence based location service ([i.2] OSR-117).

NOTE 46:	This requirement addresses the use case 6.10.

The oneM2M System shall be able to prevent unauthorized modification of the firmware of M2M Device ([i.2] SER-064).

NOTE 47:	This requirement addresses the use case 6.11.

The oneM2M System shall be able to detect unauthorized modification of the firmware of M2M Device ([i.2] SER-065).

NOTE 48:	This requirement addresses the use case 6.11.

The oneM2M System shall be able to stop operation of M2M device when it is updated with wrong firmware ([i.2] SER-066).

NOTE 49:	This requirement addresses the use case 6.11.

The oneM2M System shall be able to support security mechanisms to protect their cryptographic keys and cryptographic operations by using tamper resistant elements such as TPM (Trusted Platform Module), HSM (Hardware Security Module) and SIM (Subscriber Identity Module) ([i.2] SER-070).

NOTE 50:	This requirement addresses the use case 6.11.

The oneM2M System shall be able to prevent malfunction of M2M Device caused by receiving unsolicited messages or information ([i.2] SER-067).

NOTE 51:	This requirement addresses the use case 6.11.

The M2M System shall support mobile/portable M2M Gateway and/or Device ([i.2] OSR-128).

NOTE 52:	This requirement addresses the use case 6.12.

The M2M System shall support to distinguish the event levels for reporting and handle it differentially.

EXAMPLE:	The event levels may be divided into normal and urgent event ([i.2] OSR-032).

NOTE 53:	This requirement addresses the use case 6.12.

Based on the condition of the M2M Gateway and/or Device, the M2M System shall change the reporting (or subscription) mechanisms and/or configurations related to a service ([i.2] OSR-033).

NOTE 54:	This requirement addresses the use case 6.12.

The M2M System shall support to process access right requests of a resource and grant the requests if the required conditions are met ([i.2] SER-071).

NOTE 55:	This requirement addresses the use case 6.12.

The M2M System shall provide capabilities  to request from the underlying network either the last known location information or current location information, if supported by the underlying networks ([i.2] OSR-181).

- NOTE 56:	This requirement addresses the use case 6.12.

The M2M System shall support mechanisms to correlate charging data/records from different M2M Application Service Providers ([i.2] CHG-002 and CHG-007).

NOTE 57:	This requirement addresses the use case 6.13.

The M2M System shall support triggering M2M Devices to report on-demand regarding collected data from other M2M Devices ([i.2] OSR-072).

NOTE 58:	This requirement addresses the use case 6.13.

The oneM2M system shall enable the M2M Infrastructure to facilitate direct communication between two or more different M2M devices without having registered with one another ([i.2] OSR-130).

NOTE 59:	This requirement addresses the use case 6.14.

The oneM2M system shall be able to support the enforcement and management (e.g. update) of the privacy policies for vehicle location information ([i.2] SER-026).

NOTE 60:	This requirement addresses the use case 6.15.

The oneM2M system shall be able to apply the privacy policies configured in the oneM2M system to those vehicles whose location information is obtained by the oneM2M system itself ([i.2] SER-026).

NOTE 61:	This requirement addresses the use case 6.15

The oneM2M system shall be able to request the underlying network to perform, on behalf of the oneM2M system, the privacy policy decision concerning the location information if the location information is obtained by the underlying network location service ([i.2] SER-062 and SER-063).

NOTE 62:	This requirement addresses the use case 6.15.

The oneM2M system shall enable continuity of services to M2M devices as they move across various geographic points in the oneM2M system ([i.2] OSR-099).

NOTE 63:	This requirement addresses the use case 6.16.

The oneM2M system shall support real time data transmission for moving object to have enough time to react to the received information.

NOTE 64:	This requirement addresses the use case 6.17.

The oneM2M system shall be able to verify geographical location information from moving objects regardless of information accuracy ([i.2] OSR-131).

NOTE 65:	This requirement addresses the use case 6.17.

The oneM2M system shall be able to verify time synchronization among multiple sources that provide inputs for a single output ([i.2] OSR-132).

NOTE 66:	This requirement addresses the use case 6.17.

The oneM2M system shall be able to guarantee network stability and reliability for applications that can have safety impacts ([i.2] OSR-133).

NOTE 67:	This requirement addresses the use case 6.17.

The oneM2M system shall apply high-level security (e.g. protection from malicious hacker) for applications that can have safety impacts ([i.2] SER-075).

NOTE 68:	This requirement addresses the use case 6.17.

The oneM2M system shall provide privacy protection mechanism at the central server ([i.2] SER-072).

NOTE 69:	This requirement addresses the use case 6.17.

The M2M system shall support the data to be transmitted to IoT platform with strict timing and packet loss requirements, with requirements defined by application determined by application ([i.2] CMR-016).

NOTE 70:	This requirement addresses the use case 6.18.

The M2M system shall support the data to be transmitted from IoT platform to subscribed devices with highest priority, with strict timing and packet loss requirements, determined by application ([i.2] CMR-017).

NOTE 71:	This requirement addresses the use case 6.18.

For each source of relevant autonomous driving data which is sent to the IoT platform, the oneM2M System shall be able to detect and report the missing data in time series. For each vehicle receiving data from the Autonomous Driving Application, the oneM2M System shall be able to detect and report the missing data in time series ([i.2] CMR-018 and CMR-019).

NOTE 72:	This requirement addresses the use case 6.18.

The oneM2M System shall enable pool-based functionality sharing and scaling between Edge/Fog Nodes ([i.2] OSR-0142).

- NOTE 73:	This requirement addresses the use case 6.19.

The oneM2M System shall be able to trigger priority services from the underlying network (e.g. 3GPP MPS) ([i.2] OSR-0143).

- NOTE 74:	This requirement addresses the use case 6.19.

The oneM2M System shall enable detection of network bandwidth between Edge /Fog Nodes and M2M devices in order to provide necessary quality of service according to the bandwidth ([i.2] OSR-0144).

- NOTE 75:	This requirement addresses the use case 6.19.

The oneM2M System shall enable Edge/Fog Nodes to provide system metrics and diagnostic information to other Edge/Fog Nodes, as required to ensure reliable operations within the oneM2M System ([i.2] OSR-0145).

- NOTE 76:	This requirement addresses the use case 6.19.

The oneM2M System shall enable Edge/Fog Nodes to alert other suitable Edge/Fog Nodes that are unable to perform specific services ([i.2] OSR-0146).

- NOTE 77:	This requirement addresses the use case 6.19.

The oneM2M System shall enable data continuity services to be provided between Edge/Fog Nodes by enabling the discovery, retrieval, and combination of data sets dispersed across the Edge/Fog network ([i.2] OSR-0147).

- NOTE 78:	This requirement addresses the use case 6.20.

The oneM2M System shall enable data optimization services to be provided at Edge/Fog Nodes including aggregation, stale or redundant data identification and removal, integrity check, validation, etc. even if the data sets are dispersed across the Edge/Fog network ([i.2] OSR-0148).

- NOTE 79:	This requirement addresses the use case 6.20.

The oneM2M System shall enable categorization of the data collected by M2M devices  (e.g. high priority data, low priority data) for differential delivery and processing ([i.2] OSR-0149).

- NOTE 80:	This requirement addresses the use case 6.20.

The oneM2M System shall enable timestamp synchronization of the data collected by M2M devices between Edge/Fog Nodes for data synchronization ([i.2] OSR-0150).

- NOTE 81:	This requirement addresses the use case 6.20.

The oneM2M System shall enable services to receive and utilize location-based information about available access networks, their congestion level and other related network information, when the information is provided by the Underlying Network ([i.2] OSR-0151).

- NOTE 82:	This requirement addresses the use case 6.20.

The oneM2M System shall enable differential processing of data at different nodes, e.g. Edge/Fog node , Cloud node ([i.2] OSR-0152).

- NOTE 83:	This requirement addresses the use case 6.20.

The oneM2M System shall enable the remote instantiation of services across Fog/Edge networks as well as the remote provisioning of information required to instantiate the services ([i.2] OSR-161).

- NOTE 84:	This requirement addresses the use case 6.21.

The oneM2M System shall enable the sharing and discovery of service capability information across  Fog/Edge networks ([i.2] OSR-162).

- NOTE 85:	This requirement addresses the use case 6.21.

The oneM2M System shall enable requests for services to be provided by specific Fog/Edge nodes  ([i.2] OSR-163).

- NOTE 86:	This requirement addresses the use case 6.21.

The oneM2M System shall enable service continuity and migration among Fog/Edge nodes ([i.2] OSR-164).

- NOTE 87:	This requirement addresses the use case 6.21.

The oneM2M System shall enable the orchestration of services provided by Fog/Edge nodes in a dynamic fashion to satisfy operational requirements for availability, scalability, interoperability, etc ([i.2] OSR-165)..

- NOTE 88:	This requirement addresses the use case 6.21.

- The oneM2M system shall support the Autonomous Parking (AP) related data (for example available parking places and their location) exchange between the vehicle and the platform.

- NOTE 89:	This requirement addresses the use case 6.22.

- The oneM2M system shall support a common information model for AP including model for parking places – both in garage and in streets.

- NOTE 90:	This requirement addresses the use case 6.22.

- The oneM2M system should support a profile for parking with a consistent information model based on existing information models (e.g. ETSI ITS, DATEX and Sensoris)

- NOTE 91:	This requirement addresses the use case 6.22.

- The oneM2M system shall support handover (e.g. east-west communication) over platoon relevant data from one Platooning Manager (running on edge node) to next Platooning Manager (running on neighbouring edge node) ([i.2] OSR-154).

- NOTE 92:	This requirement addresses the use case 6.23.

- The oneM2M system shall support the Platooning data exchange between the vehicles, Platooning Service provider , Platooning Manager and oneM2M platform  ([i.2] OSR-156).

- NOTE 93:	This requirement addresses the use case 6.23.

- The oneM2M system  shall support a common information model for Platooning including vehicle state and platooning state  ([i.2] OSR-155).

- NOTE 94:	This requirement addresses the use case 6.23.

- The oneM2M system should support a profile for Platooning with a consistent information model based on existing information models (e.g. ETSI ITS, DATEX and Sensoris) ([i.2] OSR-156).

- NOTE 95:	This requirement addresses the use case 6.23.

- The oneM2M system shall support group management (e.g. joining, leaving and changing vehicle’s role within the platoon) and group message communication for platooning service ([i.2] OSR-157 and OSR-158).

- NOTE 96:	This requirement addresses the use case 6.23.

- The oneM2M system shall support Vehicle-to-Vehicle (V2V) communication via different network interfaces  ([i.2] OSR-159).

- NOTE 97:	This requirement addresses the use case 6.23.

- The oneM2M system shall support management of Vehicle-to-Vehicle (V2V) network interface switching ([i.2] OSR-160).

- NOTE 98:	This requirement addresses the use case 6.23.

- The oneM2M system shall support the Highway Pilot data (for example road surface state, detected obstacles) exchange between the vehicle and oneM2M platform.

- NOTE 99:	This requirement addresses the use case 6.24.

- The oneM2M system shall support a common information model for Highway Pilot including vehicle state, detected obstacles, road conditions.

- NOTE 100: This requirement addresses the use case 6.24.

- The oneM2M system should support a profile for Highway Pilot with a consistent information model based on existing information models (e.g. ETSI ITS, DATEX and Sensoris).

- NOTE 101: This requirement addresses the use case 6.24.

- The oneM2M system shall support the Car Rebalancing data (vehicle’s next location, ...) exchange between the vehicle and oneM2M platform.

- NOTE 102: This requirement addresses the use case 6.25.

- The oneM2M system shall support a common information model for Car Rebalancing including vehicle state and target destination (endpoint).

- NOTE 103: This requirement addresses the use case 6.25.

- The oneM2M system should support a profile for Car Rebalancing with a consistent information model based on existing information models (e.g. ETSI ITS, DATEX and Sensoris).

- NOTE 104: This requirement addresses the use case 6.25.

- The oneM2M system shall support the Urban Driving data (for example city’s traffic management system, traffic lights state, detected VRUs, …..) exchange between the vehicle and oneM2M platform.

- NOTE 105: This requirement addresses the use case 6.26.

- The oneM2M system shall support a common information model for Urban Driving including vehicle state, city traffic infrastructure state and detected VRUs.

- NOTE 106: This requirement addresses the use case 6.26.

- The oneM2M system should support a profile for Urban Driving with a consistent information model based on existing information models (e.g. ETSI ITS, DATEX and Sensoris).

- NOTE 107: This requirement addresses the use case 6.26.

- The oneM2M System shall enable Edge/Fog Nodes to have service communications with multiple other Edge/Fog Nodes to meet reliability requirements ([i.2] OSR-165).

- NOTE 108: This requirement addresses the use case 6.27.

The oneM2M System shall enable Edge/Fog Nodes to detect the failure of other Edge/Fog Nodes.

- NOTE 109: This requirement addresses the use case 6.27.

The oneM2M System shall enable the sharing and discovery of service capability information across Fog/Edge networks ([i.2] OSR-162).

- NOTE 110: This requirement addresses the use case 6.27.

The oneM2M System shall enable requests for services to be provided by Fog/Edge Nodes ([i.2] OSR-163).

- NOTE 111: This requirement addresses the use case 6.27.

The oneM2M System shall enable service migration among Fog/Edge Nodes ([i.2] OSR-164).

- NOTE 112: This requirement addresses the use case 6.27.

- The oneM2M System shall be able to send the information about offloading (e.g., offloading indication, a service logic, task, target offloading resources) ([i.2] OSR-178).

- NOTE 113: This requirement addresses the use case 6.28.

- The oneM2M System shall be able to synchronize the data between source and offloaded resources ([i.2] OSR-179).

- NOTE 114: This requirement addresses the use case 6.28.

- The oneM2M System shall be able to manage offloaded resources based on given properties from the users, e.g., blocking the offloaded resources to be accessed while the resources are offloaded to other oneM2M nodes ([i.2] OSR-180).

- NOTE 115: This requirement addresses the use case 6.28.

The oneM2M System shall support dynamic and variable Geo-Fence settings configuration for location-based services (e.g. boundary reshaping).

- NOTE 116: This requirement addresses the use case 6.29.

The oneM2M System shall enable mechanisms for sequential triggering of operations (e.g. time-based, event-based) based on requirements defined by M2M applications.

- NOTE 117: This requirement addresses the use case 6.29.

The oneM2M System shall enable mechanisms to expose policies about the  current and future resource needs of M2M nodes   for resource allocation and management purposes at the system level.

- NOTE 118: This requirement addresses the use case 6.29.

The oneM2M System shall support the management and configuration of authorization level setting for device remote control, based on the device functionality.

- NOTE 119: This requirement addresses the use case 6.30.

The oneM2M System shall enable mechanisms to expose  device policies regarding access and communication  for device security and safety.

- NOTE 120: This requirement addresses the use case 6.30.

The oneM2M system shall enable Edge/Fog Nodes to support the data delivery and processing based on the event prioritization associated with an information model.

- NOTE 121: This requirement addresses the use case 6.31.

The oneM2M System shall enable infrastructure to connect multiple protocols to devices for sustainable device connection. (([i.2] OSR-100)

- NOTE 122: This requirement addresses the use case 6.32.

The oneM2M System shall support the capability of selecting communication protocols between entities in a flexible manner.

- NOTE 123: This requirement addresses the use case 6.32.

The oneM2M System shall support dynamic management and configuration of protocols in entities for sustainable device connection.

- NOTE 124: This requirement addresses the use case 6.32.

The oneM2M System shall enable infrastructure to connect multiple underlying networks to devices for sustainable device connection. (([i.2] OSR-100)

- NOTE 125: This requirement addresses the use case 6.32.

The oneM2M System shall support the capability of selecting underlying networks between entities in a flexible manner.

- NOTE 126: This requirement addresses the use case 6.32.

The oneM2M System shall support dynamic management and configuration of underlying networks in entities for sustainable device connection.

- NOTE 127: This requirement addresses the use case 6.32.

The oneM2M System shall support the management and configuration of software module in network entity settings for device control and communication

- NOTE 128: This requirement addresses the use case 6.33.

The oneM2M System shall support methods for managing and configuring the change of the underlying network interfaces.

- NOTE 129: This requirement addresses the use case 6.33.

oneM2M shall support retention of data for protecting specific content resource based on the retention policy in effect.

- NOTE 130: This requirement addresses the use case 6.34.

The oneM2M System shall support methods for managing and configuring the change of the underlying network interfaces.

- NOTE 131: This requirement addresses the use case 6.35.

oneM2M System shall be able to track the number and types of operations on oneM2M resources.

- NOTE 132: This requirement addresses the use case 6.35.

The oneM2M System shall enable migration of data and context information between Edge/Fog Nodes for continuous services support.

- NOTE 133: This requirement addresses the use case 6.36.

The oneM2M System shall enable synchronization of data between Edge/Fog Nodes and Cloud Node when migrating data and context information between Edge/Fog Nodes.

- NOTE 134: This requirement addresses the use case 6.36.

The oneM2M system considers regulations on handling privacy related data and applying proper data protection techniques (such as anonymization and pseudonymization) to personal identifiable information.

- NOTE 135: This requirement addresses the use case 6.37.

The oneM2M system shall enable notification when the condition is maintained for pre-defined time.

NOTE 136: This requirement addresses the use case 6.38.

- High Level Architecture

NOTE: The architectures for Vehicular domain that include Edge/Fog technologies are covered by oneM2M TR-0052.

  - Introduction

The use cases in the vehicular domain discussed in the present document are listed in Table 8.1-1.

Table 8.1-1: Use cases in the vehicular domain

| Use case No. | Title | Description |
| --- | --- | --- |
| 1 | Vehicular Diagnostic & Maintenance Report | See clause 6.1 |
| 2 | Remote Maintenance Services | See clause 6.2 |
| 3 | Traffic Accident Information Collection | See clause 6.3 |
| 4 | Fleet Management Service using DTG (Digital Tachograph) | See clause 6.4 |
| 5 | Electronic Toll Collection (ETC) service | See clause 6.5 |
| 6 | Taxi Advertisement | See clause 6.6 |
| 7 | Vehicle Data Service | See clause 6.7 |
| 8 | Smart Automatic Driving | See clause 6.8 |
| 9 | Vehicle Data Wipe Service | See clause 6.9 |
| 10 | Vehicle Management based on Geo-Fence | See clause 6.10 |
| 11 | Secure Over-The-Air Firmware Update for Automotive ECUs | See clause 6.11 |
| 12 | Car/Bicycle Sharing Services | See clause 6.12 |
| 13 | Smart Parking | See clause 6.13 |
| 14 | Vehicle Broadcasting without Registration | See clause 6.14 |
| 15 | Vehicle location privacy protection | See clause 6.15 |
| 16 | Vehicle Domain service continuity | See clause 6.16 |
| 17 | Optimal Speed Recommendation | See clause 6.17 |
| 18 | Autonomous driving | See clause 6.18 |
| 19 | Automated Parking | See clause 6.22 |
| 20 | Platooning | See clause 6.23 |
| 21 | Highway pilot | See clause 6.24 |
| 22 | Car rebalancing service | See clause 6.25 |
| 23 | Urban driving | See clause 6.26 |
| 22 | Car-to-Home service based on Geo-Fence | See clause 6.29 |
| 23 | Vehicle Remote Control using External Platform | See clause 6.30 |
| 24 | Protocol Selection in Gateway for Vehicle | See clause 6.32 |
| 25 | Software-Defined Radios Application for Vehicle | See clause 6.33 |
| 26 | Dashcam Application | See clause 6.34 |
| 35 | Volatile Data Management for Instant IoT Services | See clause 6.35 |
| 37 | Handling Data Protection-related Regulations | See clause 6.37 |
| 38 | Vehicle Idling Alarming Service | See clause 6.38 |

Though these use cases are focusing on the vehicular domain, the ways of functional deployment are different. The following clauses provide three types of high level oneM2M architecture mapping for these use cases.

  - Vehicular Architecture Type 1

Figure 8.2-1 illustrates the first type of high level oneM2M architecture in vehicular domain. In this type, vehicular domain applications and the M2M service platform described in each use case are mapped to the IN (Infrastructure Node). Each vehicle is equipped with a communication module which supports cellular network or other wireless communication technologies, and the vehicle can connect to a service platform via the communication module. Alternatively, an external unit such as the smartphone acts as both a communication module and a GW. In this case, the external unit would connect to in-vehicle network via some network technologies such as Bluetooth.

The GW which provides some functions defined in oneM2M acts as the MN or the ASN. The GW would be the in-vehicle unit such as an ECU (Electronic Control Unit) or the external unit such as the smartphone. The GW is connecting to in-vehicle units and external sensors via an in-vehicle network such as CAN (Controller Area Network) or other communication technology such as Bluetooth.

The in-vehicle units such as ECUs or external sensors act as the ADN or the non-oneM2M device node.

The use case 1 to 4, 6 to 13, 15, 17, 18, 22 to 26, 29, 30, 32 to 35, 37 and 38 are categorized into this type.

Figure 8.2-1: High Level Architecture Type 1 in Vehicular Domain

  - Vehicular Architecture Type 2

Figure 8.3-1 illustrates the second type architecture. Similar to the first type, vehicular domain applications and the M2M service platform described in each use case are mapped to the IN. Each vehicle is equipped with a communication module to connect to a road side unit.

The road side unit which provides some functions defined in oneM2M acts as the MN.

In this type, the GW in vehicle acts as the MN or the ASN. The in-vehicle units such as ECUs or external sensors act as the ADN or the non-oneM2M device node.

The use cases 3, 5 and 16 to 18 are categorized into this type.

Figure 8.3-1: High Level Architecture Type 2 in Vehicular Domain

  - Vehicular Architecture Type 3

Figure 8.4-1 illustrates the third type. In this type, a vehicle to vehicle communication technology is supported in each vehicle. To support the vehicle to vehicle communication in the oneM2M architecture, the GW acts as the MN.

The use case 3, 14, 18, 20 and 23 are categorized into this type.

Figure 8.4-1: High Level Architecture Type 3 in Vehicular Domain

  - Vehicular Architecture Type 4

Figure 8.5-1 illustrates the fourth type of architecture - which is a variation to Architecture Type 2. The main difference is that the vehicle has no GW functionality providing oneM2M services to the in-vehicle units. However, the vehicle still has a communication module to communicate with RSUs.

In this architecture, the in-vehicle units, such as ECUs or external sensors, act as ADNs or non-oneM2M device nodes. The RSU, which provides functions defined in oneM2M, acts as an MN.

The use case 3, 5, and  16 to 18 are categorized into this type.

Figure 8.5-1: High Level Architecture Type 4 in Vehicular Domain

- Key Issues for Enablement of Vehicular Domain

This clause summarizes key issues resulting from the use cases and requirements identified. Each key issue is captured in a separate clause.

  - Key Issues 1: Location

Geographical location information is important in vehicular services, because a node in vehicular domain moves dynamically. As shown in use cases in clause 6, location is the key factor of vehicular services. For instance, the procedures of vehicular services could be triggered by the change of location. In other cases, the vehicular service would be provided to the vehicles located in its dedicated area.

In oneM2M Release 2, Location CSF (LOC CSF) gives the following location functions:

A location server in the Underlying Network.

A GPS module in an M2M device.

Information for inferring location stored in other Nodes, such as short-range communication establishment with a gateway.

Through the analysis of existing use cases in this technical report, the following sub-clauses contain consideration points about location for the vehicular domain.

    - Accuracy of geographic location

In the vehicular domain, each vehicle would generally be equipped with a device to obtain geographical location (geo-location) information. Through the location device, a CSE in a vehicle periodically updates its geo-location and forward it to the oneM2M platform. The accuracy of geo-location information and the applicable use cases are dependent on the capability of the geo-location information source. Table 9.1.1-1 summarizes the required accuracy of the geo-location information, and the methods to obtain the required information accurately.

Table 9.1.1-1: Required position accuracy of location

| Title | AREA | ROAD | LANE |
| --- | --- | --- | --- |
| Accuracy of geo-location (margin of error) | Less than dozens of meters | Less than a few meters | Less than dozens of centimetres |
| Example of methods | An Underlying Network (e.g. Cell-ID from 3GPP, ITS-G5) Information for inferring location stored in other Nodes | GPS Assisted GPS Underlying Network (e. g. Cell-ID from RSUs) Information for inferring location stored in other Nodes | RTK-GPS Recent GNSS/RNSS |
| Use cases | 6.8, 6.10 | 6.4, 6.12, 6.13 | 6.8 |

Figure 9.1.1-1: Example required position accuracy - AREA

To deliver a content to vehicles located in a dedicated area (a few hundred meters), the "AREA level" accuracy of geo-location would be required. Figure 9.1.1-1 illustrates the concept of "AREA" use case. In this case, an acceptable error range should be less than dozens of meters. For instance, an underlying network can provide location information with the required accuracy of geo-location based on Cell-ID.

Geo-Fence is also applicable for the AREA level accuracy of geo-location. In oneM2M Release 2, Geo-Fence is usable as the resource type <locationPolicy>. For instance, in use case 6.8, when a traffic accident occurs, oneM2M platform sets Geo-Fence around the accident point. Then the oneM2M platform can provide the content to the vehicles in the Geo-Fence.

Figure 9.1.1-2: Example required position accuracy - ROAD

To deliver content to vehicles located in a dedicated road, the "ROAD level" accuracy of geo-location would be required. In this case, navigation systems or services can be used and the acceptable error range should be less than a few meters. Some use cases provide route navigation for vehicles based on their location. Figure 9.1.1-2 illustrates the reason why "ROAD" accuracy of geo-location is required. As shown in the figure, the service cannot provide accurate route navigation when the location contains larger error. To obtain the location information with the required accuracy of geo-location level, GPS (Global Positioning System) or Assisted GPS are applicable. Furthermore, when the underlying network consists of RSUs, if the RSUs are located every a few meters to dozens of meters, the oneM2M platform could obtain ROAD level accuracy of geo-location.

To deliver content to vehicles located in a dedicated lane, the "LANE level" accuracy of geo-location would be required. In ADAS and autonomous driving, the vehicle needs this level of accuracy, In this case, an acceptable error range should be less than dozens of centimetres. Figure 9.1.1-3 illustrates the reason why "LANE" accuracy of geo-location is required. As shown in the figure, the service cannot guarantee the safety when the location contains larger error. To obtain the location information with the required accuracy level, a device in a vehicle and an infrastructure should support the current positioning systems such as RTK-GPS (Real Time Kinematic GPS) or the latest GNSS (Global Navigation Satellite System) /RNSS (Radio Navigation Satellite System).

Figure 9.1.1-3: Example required position accuracy - LANE

    - Latency

Since a node in vehicular domain moves fast and dynamically, the latency (e. g., network latency) causes a difference between the actual vehicle position and the assumed position in application implemented in the infrastructure. For instance, one second of latency causes dozens of meters of difference. Therefore, if the service requires higher position accuracy and time-sensitive operation, the service should take the effect of latency into consideration (for instance, extrapolating the location considering the latency).

  - Key Issue 2: Maintaining AE contact information

Clauses 6.3, 6.5, 6.8 and 6.16, as well as requirement 1 in clause require that  a moving M2M device uses the remote services offered by an M2M platform as well as the local services offered by a local M2M gateway. As a device AE moves along its path, it moves out of communication range of one MN-CSE and enters the communication range of another MN-CSE.

Assuming a Release 2 implementation, if the AE wishes to use the services provided by these MN-CSEs, the AE will perform a re-registration. At each (re)registration, a new <AE> resource is created at the new MN-CSE.

AEs are reached through their registrar CSE, by contacting the <AE> resource created during registration. This is referred to as the AE contact information. A number of oneM2M services rely on having up-to-date AE contact information in order to function properly. For these services, the remote CSE providing this service stores the URI of the <AE> resource as an attribute. A quick search of oneM2M TS-0004 [i.16] reveals that an <AE> resource URI can be used in the following services: notification list of subscribe/notify, link associated with an announced resource, notification list of a non-blocking asynchronous request, and member list of a group resource.

This results in the following issue:

If an AE changes its registration point, and by consequence its AE contact information, the oneM2M services highlighted above, will have URIs that point to the wrong Registrar CSE - that is they have stale AE contact information.

  - Key Issue 3: Registration management

Clauses 6.3, 6.5, 6.8 and 6.16, as well as requirement 1 in clause 6.16.10 require that  a moving M2M device uses the remote services offered by an M2M platform as well as the local services offered by a local M2M gateway. As a device AE moves along its path, it moves out of communication range of one MN-CSE and enters the communication range of another MN-CSE.

Assuming a Release 2 implementation, if the AE wishes to use the services provided by these MN-CSEs, the AE is required to perform a re-registration to every new MN-CSE it encounters. However, as the AE moves out of communication range of an 'old' MN-CSE, it will not be able to delete its registration hosted on that CSE.

This results in the following potential issue:

The AE might end up with multiple registrations in the oneM2M system. Although the old registration will eventually expire and be removed, there is a period where both registrations are active. In fact, the duration of this period may be quite long depending on the expiration time associated with the registration.

During this period, the old MN-CSE may consume processing and storage resources to manage the old AE registration. For example, the old MN-CSE may be monitoring a <timeSeries> resource associated with the AE, in order to track, detect and report missing data in a Time Series. Or, the old MN-CSE may receive a discovery request and it will search under the old registration <AE> resource, for resources matching the filter criteria.

  - Key Issue 4: Security

Due to the dynamic nature of vehicles, any service that intends to run in the vehicle domain will have to contend with security within the physical constraints of the vehicle as well as security when attempting communication with other vehicles and with external services. As has been shown throughout the use cases, there is a lot of sensitive data within vehicles to be secured. There are also a great number of use cases that require using this sensitive data for decision making to be transmitted in a secure manner. Furthermore, many decisions (or late decisions) by the vehicle components may impact human safety, so security and reactivity of the entire data acquisition, processing and decision chain is especially critical.

In oneM2M Release 2, Security CSF (SEC CSF) comprises the following functionalities:

Sensitive data handling.

Security administration.

Security association establishment.

Access control including identification, authentication and authorization.

Identity management.

Within security, we believe that there has been one particular area that has not been clearly defined, which we label secure communication.

    - Secure communication

The vehicular domain has been traditionally the most rapidly networked domain. With the introduction of more powerful Electronic Control Units (ECUs) and a number of high bandwidth networking technologies, such as MOST, Automotive Ethernet and LTE-A, it has become imperative that vehicle domain communications be as secure as possible. Security for the vehicle domain is realized without impacting vehicle functionality or imposing taxing resource acquisition and allocation.

In the M2M System functional architecture, there exists the scopes of Intra-M2M Service Provider (SP) and Inter-M2M SP communication. We propose creating a similar classification in order to define secure communication in the vehicle domain. The classifications we recommend are described in Table 9.4.1-1.

Table 9.4.1-1 Communication classifications

| Classification name | Description |
| --- | --- |
| External communication (EC) | Communication between a vehicle and broader network operator related services. This includes, but is not limited to Internet based services. These services may or may not be included as a part of M2M systems |
| Inter-vehicle communication (InterVC) | Communication between two or many vehicles. These vehicles may or may not be included in a M2M system |
| Intra-vehicle communication (IntraVC) | Communication within the physical boundaries of one vehicle in the M2M system. This classification extends to wireless based communication that has a source and destination within the same vehicle |

It is quickly apparent that the three classifications we have described have different requirements. We also propose that there are two major categories within these classifications. These are detailed in Table 9.4.1-2.

Table 9.4.1-2 Classification categories

| Classification category | Description |
| --- | --- |
| Software based secure communication (SBSC) | Secure communication created by using software running on any type of general purpose CPU in the system |
| Hardware based secure communication (HBSC) | Secure communication created by using specialized hardware based functions available to the system |

There is a need to consider SBSC from a standards point of view, as there are stricter resource requirements in the InterVC and IntraVC classifications. We assume that that EC will be using much more flexible and powerful vehicle gateway-like devices which will lean towards the use of HBSC, meaning that already standardized algorithms can be used. Since in InterVC and IntraVC, the devices used vary greatly in available resources and functionalities, we propose to include SBSC that can be further classified by the criteria seen in Table 9.4.1-3.

Table 9.4.1-3 SBSC recommended criteria

| Criteria | Description |
| --- | --- |
| Cryptographic method type | Symmetric or asymmetric cryptographic method |
| Program size | The non-volatile memory size of the program securing the communication (typically encrypting/decrypting messages) |
| Average CPU time per byte | The amount of time a general purpose CPU would need to successfully encrypt/decrypt a byte of a message |
| Memory use | The size of the volatile memory footprint for the method to successfully encrypt/decrypt a message |

Whenever possible, the selection of a particular encryption algorithm, be it SBSC or HBSC based, for communication is realized by the security mechanisms defined in the oneM2M release. When dealing with systems that are not part of the M2M system, the algorithm to use must be selected using other standardized security mechanisms available to both parties.

    - Lightweight Encryption

This type of encryption, discussed in Recommendation ITU-T X.1362 [i.5], significantly reduces the time consumed by a general purpose CPU for encryption, while permitting the system to pick a suitable level of protection for data confidentiality and integrity. Particularly for SBSC communications, these types of light-weight encryption algorithms shall be considered for resource constrained devices.

    - Security for credential

In almost all of the use cases described in clause 6, it is necessary to authenticate between entities before a service starts. In vehicle domain, there are many use cases that impact safety (particularly use cases at clauses 6.2, 6.8 and 6.11), so that a trusted credential should be used for authentication. shows examples of authentication between infrastructure node and communication module. If a credential in communication module is compromised, malicious node can connect with communication module. After establishment of connection, malicious node may be able to receive vehicle information such as version of installed software and may be able to send malicious message.

Figure 9.4.3-1: Example of authentication between infrastructure node and communication module

Figure 9.4.3-2 shows an example of authentication between gateway and ECU. A credential is used for authentication between gateway and ECU. If the credential in gateway or ECU is compromised, unauthorized ECU can be installed.

Figure 9.4.3-2: Example of authentication between Gateway and ECU

In addition to authentication, some data such as ECU software/middleware/firmware also impact the in-vehicle system, so the data should be verified for integrity using a trusted credential. Figure 9.4.3-3 shows an example of integrity check of software/middleware/firmware on gateway. If boot symmetric key and boot digest are compromised, unauthorized software/middleware/firmware can be installed.

Figure 9.4.3-3: Example of integrity check of software/middleware/firmware on gateway

Figure 9.4.3-3 shows an example of verification of ECU integrity. ECU creates digests of software/middleware/firmware and digital signatures of each digests using ECU private key, then ECU send the digests and signatures to gateway. Gateway verifies the digital signatures using ECU public key. If ECU private key is compromised, unauthorized ECU can be installed.

  - Key Issue 5: Cross-Resource Subscription

Clause requires that oneM2M System shall enable subscriptions to changes to multiple resources which aim to generate notifications if and only if the expected changes to those resources occur concurrently.

Assuming the scenario in clause 6.7 where concurrent changes to several resources need to be identified for a single event detection. A Release 2 implementation would require the subscriber to create several subscriptions. Both the host and the subscriber will process and monitor a potentially high number of individual notifications, although the concurrent change might be very rare. In order to prevent the loss of individual notifications to affect event evaluation, the subscriber would need to implement potentially sophisticated logic without event generation knowledge available only at the host.

This results the following issue:

Subscriptions to changes to multiple resources using separate subscriptions require a messaging overhead in processing subscription requests by the host and processing un-actionable notifications by the subscriber.

In addition transmission issues due to vehicle mobility which affect individual notifications create undue burdens on the subscriber/receiver in order to evaluate the occurrence of the event of interest.

  - Key Issue 6: Subscription Aggregation

Clause 6.7 describes two requirements related to data subscription grouping or aggregation:

- the oneM2M System shall enable M2M Gateways to group/aggregate data subscriptions to reduce the number of messages to M2M Devices; and

- the oneM2M System shall enable M2M Gateways to distribute notifications according to how data subscriptions have been grouped/aggregated.

Assuming the scenario in clause 6.7 where several remote subscribers create identical subscriptions to resources on an end M2M Device (e.g. sensor) behind an M2M gateway (e.g. the smart vehicle). A Release 2 implementation would require each subscriber to issue separate subscription request to the M2M Device via the M2M Gateway. This produces a messaging overhead between the M2M Gateway and the M2M Device due to the transmission of multiple subscription requests which could be the same. The M2M Device needs to generate independent notifications (e.g. one for each subscriber) when the event notification criteria is satisfied, which will be transmitted via the M2M Gateway separately. In addition, the M2M Device may have memory and processing constraints designed to support a limited number of resources. In this case the burden of monitoring each subscription separately, creates a processing overhead proportional with the number of subscribers. For some constrained devices, it is not feasible to design based on service demands (i.e. how many entities are interested in subscribing) rather than on the resources supported.

This results in the following issue:

Handling of multiple subscription requests with the same criteria introduces request and notification messaging overhead which is duplicated for M2M Devices behind an M2M Gateway.

  - Key Issue 7: Time synchronization

Clause 6.17 provides a requirement related to time synchronization:

- The oneM2M System shall be able to verify time synchronization among multiple sources that provide inputs for a single output.

- Other use cases such as the one in clause 6.18 rely on strict timing requirements being able to be enforced in the system.

Due to their resource constrained nature, some devices and networks lack support for a reference time or clock that can be efficiently and effectively propagated and synchronized to all the entities, in an end-to-end manner.  Time synchronization mechanisms such as NTP, PTP, and GPS are not always available and /or best-suited for all types of IoT deployments.

Without adequate time synchronization at the device level, applications hosted on different devices are unable to maintain synchronization with applications hosted on other devices in the network. For example, a lack of synchronization between the applications can result in one application not sending a message to another application in the proper scheduled time window or waking up at the proper time in order to receive a message.  In another example, applications timestamping data (e.g. sensor measurements) which they share with other applications may provide information which is incorrect or not being able to be interpreted correctly.

In release 3 implementations the existence or lack of synchronization mechanisms cannot be detected. Moreover, lack of synchronization between various entities cannot be detected and corrected.

This results in the following issue:

Services requiring time-sensitive operations rely on the various Service Layer instances being time-synchronized.

  - Key Issue 8: V2V network interface switching management

Both Clause 6.23 and TS-0002 provide the requirements related to V2V network interface switching management.

In general, a platoon is formed by a platoon manager that can be a platooning service provider (e.g. IN-AE) or the leader vehicle (e.g. an ADN-AE, a MN-CSE, or an ASN-CSE). After formation, vehicles (e.g. ADN-AE, MN-CSE, or an ASN-CSE) in the platoon can exchange messages via V2V communication and the leader vehicle relays road conditions and traffic information from roadside and network to other vehicles in the platoon.

V2V communication in a platoon can use several communication modes, including broadcast over PC5, unicast over LTE-Uu and MBMS. Limited by network deployment and environmental conditions, different areas can support different communication modes for V2V communication. As Figure 9.8-1, area2 only supports unicast over LTE-Uu and MBMS because the PC5 signal is unstable. For example, streets cut through dense blocks of tall buildings which significantly affect the reception of PC5 radio signals. Area3 has poor network coverage, hence it only supports PC5. The IoT platform (e.g. IN-CSE) has global information about network status and maps and can determine supported communication mode(s) for a specified area. So the IoT platform can plan the communication modes for different segments of the routing path by receiving information about the platoon information, network status and maps information. The platoon information includes memberID, leader vehicle identity, the supported communication mode of each vehicle and the routing path. As Figure 9.8-1 shows, the routing path (from point 1 to point 4) is divided into three segments. Then the IoT platform sends the planned communication mode information to the leader vehicle. When the platoon is moving into the corresponding segment, the communication mode of the platoon needs to switch.

The oneM2M system does not support a mechanism to determine an optimal communication mode for V2V communication in the platoon service according to the platoon member communication capability, road environment and network status. Firstly, the IN-CSE does not have the supported communication mode of each vehicle and routing path of the platoon to plan the communication mode of the routing path. In addition, the IN-CSE lacks resources to store the relationship between the planned communication mode and route segments. Finally, the IN-CSE cannot distinguish the leader vehicle role in a platooning group.

Figure 9.8-1: Communication mode for V2V communication on the platoon routing path

This results in the following issues:

- The IN-CSE does not have the supported communication mode of each vehicle and routing path of the platoon to plan the communication mode of the routing path.

- The IN-CSE lacks resources to store the relationship between planned communication mode and route segments.

- The IN-CSE cannot distinguish the leader vehicle role in a platooning group.

  - Key Issue 9: Simultaneous registration with multiple CSEs

Clause 6.5 concludes with several requirements related to registration:

The M2M System shall provide the capability for an M2M device to maintain registration with multiple entities simultaneously.

The registration shall be able to include information that identifies the peer entity, and other information necessary for the establishment of the respective peer relationships (e.g. management privilege, subscription).

The M2M System shall be able to hold the complete set of information context about the peer entity for some registrations (i.e. "full registration").

The M2M System shall be to hold only a subset of information context about the peer entity for some registration (i.e. "lightweight registration").

The M2M System shall be able to perform "lightweight registration" at different entities pertaining to a common peer entity and to hold different sets of information about the common peer entity.

The M2M System shall be able to correlate the "full registration" and the "lightweight registration" pertaining to a common peer entity.

The M2M System shall be possible to distinguish the "full registrations" and the "lightweight registrations" pertaining to a common peer entity.

Further work in clauses 6.19-6.21 considers cases based on Edge/Fog deployments, and as such assume the possibility for nodes to receive services from peer nodes, in addition to hierarchies like the ones created by single registration relationships.

Registrations are performed in order to obtain services from a Service Layer. In a Release 3 implementation after registering, an AE or CSE receives all services it is authorized for from its registrar. Therefore, allowing multiple registrations requires that registration scope (i.e. the services to be provided by each registrar) to be defined. Otherwise multiple registrations of the same entity will result in resources from all registrars to be allocated/ planned, when in reality certain services might be provided only by a single registrar.  This results in the following issue:

Support of multiple registrations for a single entity may introduce unnecessary resource allocations at the registrars, unless the registration procedure indicates which services are requested or granted, and the specific combinations of services to be provided by each registrar.

The use of multiple registrations of the same entity simultaneously (in Edge/Fog deployments ) has also been assumed in clause 6.19 in establishing requirements for reliability. Multiple registrations can ensure reliable services by using registration status e.g. active/inactive. This results in the following issue:

- The system needs to maintain registration status (e.g. active/inactive) along with context, in order to manage registrations in a multiple-registration system.

- Potential Solutions for the Key Issues

  - Solution A: Maintaining AE contact information - IN-CSE Notifies all CSEs

    - Solution Description

In order to address the Key Issues 2 and 3 we can rely on the IN-CSE to propagate the updates throughout the oneM2M system and to proactively delete the AE registration in the old MN-CSE.  The basic procedure is described below:

Step 1: The IN-CSE determines that an AE mobility event has occurred. An AE mobility event occurs when an AE has moved and has re-registered to a new ASN-CSE or MN-CSE.

For cases where the AE-ID is assigned by the IN-CSE (AE-ID-Stem starts with the letter 'S') the IN-CSE may examine the re-registration <AE> announcement. If this announcement is using a previously allocated AE-ID, but coming from a different ASN-CSE or MN-CSE, the IN-CSE can make the determination that this registration is as a result of an AE mobility event.

For cases where the AE-ID is assigned by a local CSE (an ASN-CSE or MN-CSE), then the AE-ID-Stem starts with the letter 'C'. The local CSE may examine the registration request. If the AE supplies an AE-ID-Stem and the local CSE is unaware of this AE, then the CSE can make the determination that this registration is as a result of an AE mobility event. Once this determination is made, the local CSE needs to inform the IN-CSE. This can be achieved through a Notify request, which includes in its Content the old AE contact information (old URI) and new AE contact information (new URI).

Step 2: The IN-CSE examines all its services that have the old AE contact information, and it updates these with the new AE contact information. Basically this would involve looking in the attributes of all <group>, <subscription>, and <announce> resources, and replacing the old URI with the new URI.

Step 3: The IN-CSE informs MN-CSEs and ASN-CSEs that there has been an AE mobility event for this AE. This can be achieved through a Notify request, which includes in its Content the old AE contact information (old URI) and new AE contact information (new URI).

Step 4: The MN-CSEs and ASN-CSEs examine their services to determine if they are impacted by the AE mobility event. If so, they update the old AE contact information with the new AE contact information. The Old MN-CSE will use this as an indication that it may delete the old <AE> registration.

    - Solution Applicability

This solution applies to Key Issue 2 and Key Issue 3.

  - Solution B: Maintaining AE contact information - IN-CSE Notifies only impacted CSEs

    - Solution Description

This solution addresses Key Issue 2, and avoids the need for the IN-CSE to contact all ASN/MN-CSEs in the oneM2M System, to inform them about an AE mobility event. The solution relies on the IN-CSE being aware of the ASN/MN-CSEs that have oneM2M services that use AE contact information (namely notification list of subscribe/notify, link associated with an announced resource, notification list of a non-blocking asynchronous request, and member list of a group resource). The IN-CSE keeps a master list of AE contact information for each ASN/MN-CSE in the oneM2M System.

The operation involves two processes:

- Keeping the IN-CSE in sync with all ASN/MN-CSEs that have resources that contain an <AE> resource URI.

Once an AE mobility event is detected, the IN-CSE determining the impacted ASN/MN-CSEs, and only updating these with the new AE contact information.

Process 1: Keeping IN-CSE in sync

Step 1: IN-CSE creates a resource to store AE contact information for each ASN/MN-CSE. This may be a simple list of <AE> resource URIs associated with each ASN/MN-CSE. For example, these may be stored in a <masterAEContactList> resource.

Step 2: When a service using AE contact information is created/updated/deleted at an ASN/MN-CSE, the ASN/MN-CSE notifies the IN-CSE. The notification includes the AE contact information.

Step 3: Upon reception of the notification, the IN-CSE updates the <masterAEContactList> accordingly. Either adding or deleting the entry for the impacted CSE.

Process 2: Updating AE contact information in oneM2M system

Step 1: The IN-CSE determines that an AE mobility event has occurred. An AE mobility event occurs when an AE has moved and has re-registered to a new ASN-CSE or MN-CSE.

For cases where the AE-ID is assigned by the IN-CSE (AE-ID-Stem starts with the letter 'S') the IN-CSE may examine the re-registration <AE> announcement. If this announcement is using a previously allocated AE-ID, but coming from a different ASN-CSE or MN-CSE, the IN-CSE can make the determination that this registration is as a result of an AE mobility event.

For cases where the AE-ID is assigned by a local CSE (an ASN-CSE or MN-CSE), then the AE-ID-Stem starts with the letter 'C'. The local CSE may examine the registration request. If the AE supplies an AE-ID-Stem and the local CSE is unaware of this AE, then the CSE can make the determination that this registration is as a result of an AE mobility event. Once this determination is made, the local CSE needs to inform the IN-CSE. This can be achieved through a Notify request, which includes in its Content the old AE contact information (old URI) and new AE contact information (new URI).

Step 2: The IN-CSE examines all its services that have the old AE contact information, and it updates these with the new AE contact information. Basically this would involve looking in the attributes of all <group>, <subscription>, and <announce> resources, and replacing the old URI with the new URI.

Step 3: The IN-CSE searches in the <masterAEContactList> to find all CSEs that are impacted as a result of the AE mobility event. It then informs these MN-CSEs and ASN-CSEs that there has been an AE mobility event for this AE. This can be achieved through a Notify request, which includes in its Content the old AE contact information (old URI) and new AE contact information (new URI).

Step 4: The MN-CSEs and ASN-CSEs examine their services to determine if they are impacted by the AE mobility event. If so, they update the old AE contact information with the new AE contact information. The Old MN-CSE will use this as an indication that it may delete the old <AE> registration.

    - Solution Applicability

This solution applies to Key Issue 2.

    - Solution Details

      - Impacted Resources and Attributes

        - Overview

To implement this solution,  the <AE> resource and the <AEAnnc> resource need to be modified, and two new resource types need to be defined (<AEContactList> and <AEContactListPerCSE). The <AEContactList> resource maintains a list of CSEs that may be impacted by a change in an AE registration point. For each CSE in this list,  the <AEContactListPerCSE> resource includes a list of all "references" to Application Entity resource identifiers that are included in this CSE. These "references" to Application Entity resource identifiers may occur in announcement links,  notification targets, and in group member IDs. These are further detailed in the following clauses.

        - Modified <AE> resource

New attributes as shown in Table 10.2.3.1.2-1.  The status attribute allows to transition an <AE> resource to an INACTIVE status. While INACTIVE, the resource is not discoverable. In addition as part of the registration procedure, an Application Entity may request (through the localRegistration  attribute) that the service provider track the AE as it changes registration points. If set to TRUE, the service provider will track the Application Entity, essentially keeping track of its Registrar CSE. If set to FALSE, the service provider will not track the Application Entity.

Table 10.2.3.1.2-1: New attributes of <AE> resource

| Attributes of <AE> | Multiplicity | RW/ RO/ WO | Description | <AEAnnc> Attributes |
| --- | --- | --- | --- | --- |
| status | 0..1 | RW | Denotes status of the AE registration. If ACTIVE, the <AE> resource and all its child resources may be discoverable. If INACTIVE, the <AE> resource and all its child resources shall not be discoverable. | OA |
| localRegistration | 0..1 | RW | Denotes if the Application Entity requests that its Registration Points be tracked. If TRUE, AE requests to be tracked as it changes its Registration Point. If FALSE, the AE requests not to be tracked as it changes its Registration Point. | OA |

        - Modified <AEAnnc> resource

New common attribute to allow transitioning the <AEAnnc> resource to INACTIVE status. While INACTIVE, the <AEAnnc> resource at the IN-CSE is not discoverable (see Table 10.2.3.1.3-1).

Table 10.2.3.1.3-1: New Commonly Used Attributes for Announced Resources

| Attribute Name | Mandatory/Optional | Description |
| --- | --- | --- |
| status | Optional | Only applicable to announced <AE> resource. Denotes status of the announced AE registration. If ACTIVE, the announced <AE> resource and all its child resources may be discoverable. If INACTIVE, the announced <AE> registration and all its child resources shall not be discoverable. |

        - New Resource Type: AEContactList

An <AEContactList> resource shall contain <AEContactListPerCSE> child resources, one for each CSE that has sent a NOTIFY request to the CSE about the creation, update, or deletion of a resource that references an Application Entity resource identifier. The <AEContactList> resource shall only be created in the IN-CSE.

Figure 10.2.3.1.4-1: Structure of <AEContactList> resource

The <AEContactList> resource shall contain the child resources specified in Table 10.2.3.1.4-1.

Table 10.2.3.1.4-1: Child resources of <AEContactList> resource

| Child Resources of <AEContactList> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n |  |
| [variable] | <AEContactListPerCSE> | 0..n |  |

The <AEContactList> resource shall contain the attributes specified in Table 10.2.3.1.4-2 .

Table 10.2.3.1.4-2: Attributes of <AEContactList> resource

| Attributes of < AEContactList > | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| resourceID | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| resourceName | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| parentID | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| expirationTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| accessControlPolicyIDs | 0..1 (L) | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| creationTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| Labels | 0..1 (L) | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| numberImpactedCSEs | 1 | RO | The number of Hosting CSEs that have reported that they have a reference to an Application Entity resource identifier. |

        - New Resource Type: AEContactListPerCSE

An <AEContactListPerCSE> resource shall include a list Application Entity resource identifiers (SP-relative-Resource-IDs of an AE). For example, these Application Entity resource identifiers may occur in announcement links, notification targets, and in group member IDs. The <AEContactListPerCSE> resource shall only be created in the IN-CSE.

Figure 10.2.3.1.5-1 Structure of <AEContactListPerCSE> resource

The <AEContactListPerCSE> resource shall contain the attributes specified in Table 10.2.3.1.5-1.

Table 10.2.3.1.5-1: Attributes of <AEContactListPerCSE> resource

| Attributes of <AContactListPerCSE> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| resourceID | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| resourceName | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| parentID | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| expirationTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| accessControlPolicyIDs | 0..1 (L) | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| creationTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| Labels | 0..1 (L) | RO | See clause 9.6.1.3 in oneM2M TS-0001 [i.3]. |
| CSE-ID |  | RO | The identifier of the Hosting CSE which has a reference to an Application Entity resource identifier (SP-relative-Resource-ID that points to an AE). Hosting CSEs notify the IN-CSE when they have a reference to an <AE> resource through e.g. announcements, notification targets, group member IDs. |
| AE-IDList | 0..1(L) | RO | List of Application Entity resource identifiers hosted on CSE with identifier CSE-ID. |

      - Impacted Information Flows

        - Overview

To implement this solution, the new procedure to manage change in AE Registration Point is detailed in the following clause

Other oneM2M TS-0001 [i.3] information flows/procedures will be affected as follows:

Application Entity registration procedure (oneM2M TS-0001 [i.3], clause 10.1.1.2.2): The existing 5 cases are shown to be valid for initial registration or re-registration to the same Registrar CSE. Two new  cases are added, for re-registration to a new Registrar CSE: when the AE-ID-Stem starts with 'S' and AE includes an AE-ID-Stem and when AE-ID-Stem starts with 'C' and AE includes an AE-ID-Stem

The notification procedures overview (oneM2M TS-0001[i.3], clause 10.3.1) update includes in the listing the new notifications of registration point changes and notifications to the IN-CSE for new/updated reference to an Application Entity Resource identifier

Modifications to the basic CREATE, UPDATE and DELETE procedures (oneM2M TS-0001[i.3], clause 10.1) to show that notifications are sent by Receivers when the operation changes a reference to  an Application Entity Resource Identifier, These notifications allow adding, updating, or removing entries from the <AEContactList> resource.

Modification of CSE processing at expiry of resource expirationTime (oneM2M TS-0001[i.3], clause 9.6.1.3.1) to indicate that if the 'obsolete' resource had a reference to an Application Entity Resource ID, the Hosting CSE shall send a NOTIFY request to the IN-CSE, allowing removing the entry from the <AEContactList> resource.

        - Procedure for Managing Change in AE Registration Point

          - Procedure at IN-CSE

The IN-CSE may determine that an AE has changed registration point either by:

Observing the creation on an <AEAnnc> resource with an AE-ID-Stem that it had previously assigned for a different Registrar CSE

Receiving a NOTIFY request from a Registrar CSE whose content includes the SP-relative-Resource-ID before and after the change in registration point

In both cases, the IN-CSE shall send a NOTIFY request to the CSEs, so that these may update the references to the <AE> resources for the AE that has changed its registration point. If the IN-CSE maintains an <AEContactList> resource, the IN-CSE shall determine which CSEs are effected, and shall send the NOTIFY request only to these. If the IN-CSE does not maintain an <AEContactList> resource, the IN-CSE shall send the NOTIFY request to all CSEs. The Content parameter of the NOTIFY request shall contain the SP-relative-Resource-ID at the prior registration point and the SP-relative-Resource-ID at the new registration point

          - Procedure at any CSE

Upon receiving a NOTIFY request regarding a change in AE registration point, the receiving CSE shall update all references to the SP-Relative-Resource-ID (e.g. in Announce links, Notification targets, group Member IDs) tied to the prior AE registration point, so that these refer to the new AE registration point.

  - Solution C: Cross-Resource Subscription

    - Solution Description

In order to address the Key Issue 5, a subscriber AE/CSE creates resource subscriptions where automatic notifications depend on two or more resources, not a single resource. In other words, notifications are generated when changes to multiple resources occur concurrently. This subscription type (termed here Cross-Resource Subscription) involves multiple resources which do not necessarily have parent-child relationship. The corresponding notifications are termed Cross-Resource Notifications.

The subscriber indicates specific requirements when issuing a Cross-Resource Subscription request to the resource host. The basic procedure is described below:

Step 1: The subscriber sends a Cross-Resource Subscription request to the resource host. This request message informs the resource host that the subscriber is interested in concurrent changes of multiple resources and expects to receive a single notification if all changes take place. For this purpose, the message may include parameters such as:

target resources (targetResourcesList): the list of target resources for the Cross-Resource subscription.

event notification criteria (eventNotificationCriteriaList): the list of event notification criteria (as defined in oneM2M Release 2) for all target resources. One event notification criteria could apply to each individual resource in listOfTargetResources.

time window for evaluation: Includes information about window type (windowType), e.g. periodic, sliding, etc. and a window duration (windowDuration). The time window is used by the resource host to determine if the expected changes to the target resources should generate Cross-Resource Notifications.

Step 2: The resource host processes the received Cross-Resource Subscription request. If the request is approved, the resource host creates a local subscription resource to maintain the subscription request.

Step 3: The resource host sends a response to the subscriber. The response may include a Uniform Resource Identifier (URI) of the local subscription resource created in Step 2.

Step 4: The resource host observes that events on target resources take place.

Step 5: The resource host uses the time window mechanism to determine whether a notification should be generated.  The time window mechanism implemented in the resource host enables the host to relieve the subscriber of the processing burden which would be required if using existing single-resource subscription mechanisms in oneM2M Release 2, as discussed in clause 9.5.

Step 6: The resource host sends a notification to the subscriber (and/or other entities if specified by the notificationURIs in Step 1) assuming the decision from Step 5 is yes.

Figure 10.3.1-1. Solution Procedures for Cross-Resource Subscription

As illustrated in Figure 10.3.1-1 , the proposed solution procedures use Cross-Resource Subscription and Cross-Resource Notification, which removes the messaging overhead in processing separate subscription requests by the Resource Host and processing un-actionable notifications by the Subscriber.

Also, in the proposed solution procedures in Figure 10.3.1-1 , the Resource Host is responsible for evaluating the occurrence of the event of interest , which removes the undue burdens at the Subscriber.

    - Solution Applicability

This solution applies to Key Issue 5.

    - New Resources and Procedures

      - Introduction

A new resource <crossResourceSubscription> is proposed to implement Cross-Resource Subscription functionality, together with the existing <group> resource.

For example, assume an IN-CSE has a <group> resource (e.g. <INCSEBase>/<group>) which has two members <container1> and <container2>. If an IN-AE wants to receive automatic notification when the value of both contain resources exceed a threshold, it shall create a cross-resource subscription (i.e. the new resource type  <crossResourceSubscription>) as the child resource of this <group> resource and set appropriate values for the attributes of <crossResourceSubscription> using the following CREATE command. Then IN-CSE shall issue a notification if and only if the value of both container resources change to exceed the threshold within the timeWindowSize (i.e. 60 seconds):

CREATE <inCSEBase>/<group>/<crosssResourceSubscription>; payload: timeWindowType=1 (i.e. periodical time window), timeWindowSize=60 seconds.

Here, timeWindowType, and timeWindowSize are the attributes for a <crossResourceSubscription> resources and they will be described in the clause 10.3.3.2.

The following clauses introduce new attributes for <subscription> resource supporting Cross-Resource Subscription functionality, detailed procedure  to create a cross-resource subscription as a child resource of a <group> resource and the corresponding notification processing.

      - New <crossResourceSubscription> Resource to Enable Cross-Resource Subscription Functionality

<crossResourceSubscription> extends existing <subscription> resource with the following enhancements.

<crossResourceSubscription> has the same child resources as <subscription> has.

<crossResourceSubscription> replaces "eventNotifionationCriteria" attribute of <subscription> with a new attribute "listOfEventNotificationCriteria". For other attributes of <subscription>, <crossResourceSubscription> will have the same.

<crossResourceSubscription> has several new attributes as included in Table 10.3.3.2-1.

Table 10.3.3.2-1 lists new attributes for the <crossResourceSubscription> resource. Those attributes are leveraged to trigger and create a cross-resource subscription.

Table 10.3.3.2-1: New Attributes of <crossResourceSubscription> Resource

| Attributes of <subscription> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| listOfEventNotificationCriteria | 1 (L) | RW | This attribute lists eventNotificationCriteria for each target resource involved in a cross-resource subscription. |
| numOfTargetResourcesForNotification | 0..1 | RW | This attribute indicates the required number of target resources for generating a cross-resource notification. |
| timeWindowType | 1 | RW | This attribute indicates the type of time window mechanisms (e.g. "timeWindowType=1" stands for periodical time window and "timeWindowType=2" represents sliding time window) which will be used to determine the generation of a cross-resource notification. |
| timeWindowSize | 1 | RW | This attribute indicates the size or time duration (e.g. in seconds) of the time window, based on which cross-resource notification shall be generated. Note that the maximum window size (e.g. 60 seconds) may be enforced by the Hosting CSE for a subscriber. If the timeWindowSize indicated or requested by a subscriber is larger than the maximum window size, the Hosting CSE rejects the subscriber's request. |

      - Procedure for Creating a Cross-Resource Subscription

This procedure shall be used to request the creation of a new cross-resource subscription <crossResourceSubscription> resource as a child resource of a <group> resource. As a result, cross-resource notifications shall be generated if and only if changes occur during the designated time window on each involved target resource; note that the condition for each target resource is indicated by the new attribute listOfEventNotificationCriteria.

Table 10.3.3.3-1: <crossResourceSubscription> CREATE over a <group> resource

| <crossResourceSubscription> CREATE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | All parameters defined in table 8.1.2-2 in oneM2M TS-0001 [i.3] apply with the specific details for: To: a <group> resource Content: The resource content shall provide values of attributes of <crossResourceSubscription> |
| Processing at Originator before sending Request | According to clause 10.1.1.1 in oneM2M TS-0001 [i.3] with the following additions: The Request shall include timeWindowType and timeWindowSize. The Request shall include listOfEventNotificationCriteria. The Request shall include numOfTargetResourcesForNotification. |
| Processing at Receiver (i.e. Group Hosting CSE) | According to clause 10.1.1.1 in oneM2M TS-0001 [i.3] with the following additions: Check if the resource as indicated in the To parameter in the Request is a <group> resource. Check if the Originator has privileges for creating a child resource in the To parameter in the Request. Upon successful validation, obtain the IDs of all member resources from the membersIDs attribute of the addressed <group> resource. Then, convert the Request to normal single-resource subscription request (i.e. removing numOfTargetResourcesForNotification, timeWindowType, timeWindowSize; extract eventNotificationCriterial for each member from listOfEventNotificationCriteria; change notificationURI to the Group Hosting CSE) and send the normal single-resource subscription request to the Members Hosting CSEs addressing the obtained IDs appended with the ID of the <subscription> resource to be created. After receiving the responses from the Members Hosting CSEs, respond to the Originator with the aggregated results and the associated memberIDs. If any of the checks above fails, the Hosting CSE shall send an unsuccessful response to the Originator with corresponding error information. Otherwise, the Hosting CSE shall create the <crossResourceSubscription> resource under the <group> resource as indicated in the To parameter in the Request, and send a successful response to the Originator. |
| Processing at Member Hosting CSE | For the subscribe procedure, the Members Hosting CSE shall treat the request received from the group Hosting CSE as a normal single-resource SUBSCRIBE request on the addressed member resource as if it comes from the original Originator. Therefore the members Hosting CSE shall: Check if the original Originator has the READ permission on the members resource Upon successful validation, perform the subscribe procedures for the corresponding type of member resource as described in clause 10.2.12 in oneM2M TS-0001 [i.3]. Send the corresponding response to the group Hosting CSE |
| Information in Response message | All parameters defined in table 8.1.3-1 in oneM2M TS-0001 [i.3] apply with the specific details for: Content: address of the created <crossResourceSubscription> resource, according to clause 10.1.1.1 in oneM2M TS-0001 [i.3]. |
| Processing at Originator after receiving Response | According to clause 10.1.1.1 in oneM2M TS-0001 [i.3] |
| Exceptions | According to clause 10.1.1.1 in oneM2M TS-0001 [i.3] |

      - Procedure for Generating Cross Resource Notification

After the Group Hosting CSE creates the <crossResourceSubscription> as described in the clause 10.3.3.2, it uses the designated time window mechanism to determine if a cross-resource notification shall to be issued each time when receiving a notification from Member Hosting CSEs. Only when expected changes on all target resources occur within the required time window, the Group Hosting CSE issues a notification (i.e. cross resource notification) to the Originator and/or its designated notification receivers; otherwise, the Group Hosting CSE just simply discards the received normal single-resource notification from any Member Hosting CSE.

  - Solution D: Subscription Aggregation

    - Solution Description

In order to address the Key Issue 6, a middle node (e.g. the M2M Gateway in clause 6.7) can group or aggregate subscription requests received from multiple subscribers, generate one aggregated subscription request, and only forward this aggregated subscription request to the resource host (e.g. the M2M Device in clause 6.7). The basic procedure is described below:

Step 1: The resource host may publish/announce its resources and associated event notification criteria to the middle node, which could be the registrar CSE of the resource host. This message contains a list of following parameters:

ResourceID: the identifier of the source which can be subscribed.

EventNotifCriteria: the event notification criteria associated with the resource as denoted by resourceID.

WhiteSubList: the list of subscribers which are allowed to make subscription to the resource as denoted by resourceID.

BlackSubList: the list of subscribers which are not allowed to make subscription to the resource as denoted by resourceID.

Access control criteria for allowing or disallowing subscribers. Note that the access control criteria could be based on the location of subscribers, the service or application type of subscribers, etc.

Step 2: The middle node maintains the list of resouceID and its eventNotifCriteria. It sends a response back to the resource host.

Step 3: Each subscriber sends a subscription request to the middle node. Besides resourceID, notifURI, and eventNotifCriteria, this message could optionally contain a new parameter aggrgFlag. Note that the destination of this message is the middle node:

aggrgFlag: a flag to indicate if the subscriber likes this subscription request to be aggregated (e.g. if aggrgFlag=TRUE) or not (e.g. if aggrgFlag=FALSE).

Step 4: The middle node finds that those subscription requests in Step 3 from multiple subscribers can be aggregated (e.g. they have the same resourceID and eventNotifCriteria; and the subscribers are in the whiteSubList as received in Step 1). Then it aggregates those subscription requests, creates a subscription group SG(i), and generates an aggregated subscription request. The middle node also creates a notification group NG(i), which contains all notifURI received from all subscribers in Step 3.

Step 5: The middle node sends an aggregated subscription request to the resource host. This message may contain the following parameters, which are associated with SG(i). In addition, the middle node maintains the mapping relationship between SG(i) and NG(i):

ResourceID: the identifier of resource which multiple subscribers are interested.

EventNotifCriteria: the event notification criteria multiple subscribers indicate.

NewNotifURI: indicates the address which the resource host should send the notification to (i.e. the address of the middle node or the identifier of SG(i) being created during subscription aggregation in Step 4.

SubscriberList: the list of original subscribers included in SG(i). This parameter may be optional.

Step 6: The resource host sends a response back to the middle node. If subscriberList is included in Step 5, the resource host may disapprove some subscribers. If that happens, the middle node will update NG(i) accordingly.

Step 7: An event corresponding to eventNotifCriteria in Step 3 occurs.

Step 8: The resource host sends a notification to newNotifURI which was indicated in Step 5.

Step 9: The middle node receives the notification and distributes it to all subscribers and their notification targets as indicated in Step 3 and captured in NG(i).

Step 10: Subscribers and their notification targets send a response to the middle node.

Step 11: The middle node sends a response back to the resource host.

Figure 10.4.1-1: Solution Procedures for Subscription Aggregation

    - Solution Applicability

This solution applies to Key Issue 6.

  - Solution E: Secure Channel Establishment

    - External communication and inter-vehicle communication

      - Solution Description

In order to address key issue 4: security, vehicular security requires secure communication and lightweight cryptography to protect data confidentiality, data integrity and availability. In general, assuming support of an Internet Protocol layer, Transport Layer Security (TLS) [i.6] is used to establish secure channel between entities and also supports data integrity in transport layer.

According to clauses 8 and 9, external communication (EC) is defined as communication between vehicle gateway (MN or ASN) and server at infrastructure domain (IN-CSE). For example, the UU interface in 3GPP in V2X [i.9] and [i.10]. Inter-vehicle communication (InterVC) is defined as communication between vehicle gateways (MN or ASN). For example, PC5 in 3GPP in V2X [i.9] and [i.10]. These communications could use TLS or DTLS, provided that latency constraints can be satisfied. The protection of communications is also discussed in PREparing SecuRe Vehicle-to-X (PRESERVE) Project  and 3GPP. PRESERVE proposed secure communication for EC and InterVC [i.8] and 3GPP defined specification for EC and InterVC using LTE [i.9] and [i.10].

    - Intra-vehicle communication

      - Solution Description

Intra-vehicle communication (IntraVC) is defined as communication between vehicle gateway (MN or ASN) and ECU (ADN-AE or non-oneM2M) or external sensors (ADN-AE or non-oneM2M) in clause 8.

The former is a communication in the control system, and Controller Area Network (CAN) is used as the communication protocol. Although ECU is a low resource device, a strict response latency is defined as service performance requirement because the control system is impacting human life.

The latter is a communication in information system and Wi-Fi, Bluetooth or Ethernet are used as communication protocol. The devices in this communication range from low resource device such as external sensor to relatively powerful resource such as smartphone. Service performance requirements level in the information system is not high as compared with the control system. We propose two categories from the point of view of systems in IntraVC. The classifications are described in [i.17].

Table 10.5.2.1-1: IntraVC classifications

| System | Resource of devices | Protocol | Example |
| --- | --- | --- | --- |
| Information system | High or low resource | Wi-Fi, Bluetooth, Ethernet | Vehicle gateway and smartphone Vehicle gateway and external sensor |
| Control system | Low resource | CAN | Vehicle gateway and ECUs ECU and the other ECU |

In the information system, it is possible to establish a secure channel using security protocols which are described in clause 10.5.1.1, because there is no strict requirement such as response latency within a few tens of milliseconds and it can use TCP/IP network. If external sensors have low resource, security protocols which are designed based on lightweight cryptography (i.e. lightweight encryption algorithm or lightweight MAC algorithm) may be suitable to establish secure channel. Lightweight cryptography is discussed in ISO/IEC29192 [i.7] and lightweight encryption algorithm is discussed in Recommendation ITU X.1362 [i.5].

In the control system, there are severe service performance requirements such as response latency within a few tens of milliseconds because information sent from the ECU is involved in vehicle control. Almost all communications in control system send messages by plain text. However, Hardware Security Module (HSM) was discussed in E-safety Vehicle Intrusion proTected Application (EVITA), and EVITA defined three categories of security requirements: EVITA-HSM-Full-Version, EVITA-HSM-Medium-Version and EVITA-HSM-Light-Version [i.11]. According to the definition of EVITA, ECUs correspond to EVITA-HSM-Medium. EVITA also proposed transport protocol with secure features based on EVITA HSM [i.12].

    - Solution Applicability

This solution applies to Key Issue 4.

  - Solution F: Hardware Secure Element

    - Solution Description

In some use case, credentials are critical to the authentication between entities and the verification of integrity. If the credential is compromised or tampered with, the result of authentication or verification of integrity is not reliable. Therefore, the credential should be stored in tamper resistance storage and protect cryptographic operation from side channel attack. The following hardware secure elements provide tamper resistance and the highest protection level 3 which is defined at clause 6.3.1 in oneM2M TS-0003 [i.14], and could be suitable for protection of the credentials.

Trusted Platform Module (TPM):

Trusted Computing Group (TCG) defined the specification of TPM for vehicle domain [i.9].

EVITA-Hardware Security Module (HSM) implemented in tamper-resistant hardware:

EVITA defined the specifications of EVITA-Full-HSM, EVITA-Medium-HSM, EVITA-Light-HSM [i.11].

Universal Integrated Circuit Card (UICC), embedded Universal Integrated Circuit Card (eUICC) [i.9]:

Usage of UICC is described in oneM2M TS-0003, annex D [i.14]. GSMA defined the specification of eUICC [i.9].

Other secure elements supporting basic cryptographic services as specified in oneM2M TS-0003, annex L [i.14] specifies a set of cryptographic services.to be supported on tamper-resistant IoT secure elements.

    - Solution Applicability

This solution applies to Key Issue 4.

  - Solution G: Cross-Resource Subscription #2

    - Solution Description

In order to address the Key Issue 5, a subscriber AE/CSE creates resource subscriptions where automatic notifications depend on two or more resources, not a single resource as described in the previous solution. In this clause, a new candidate solution for the same use case but with different resource types and procedures is illustrated.

In short, a new resource type subscriptionAssociation is suggested which has two-ways links from/to existing subscription resource type. When a <subscriptionAssociation> resource is created which associates existing <subscription> resources, notifications generated per <subscription> resource is sent to the <subscriptionAssociation> resource Hosting CSE to generate a single but not just aggregated notification to the cross-resource subscriber.

The main difference between the previous and this one is whether the solution relies on group functionality (i.e. group resource type) or not while making cross-resource subscription relationship.

    - Solution Applicability

This solution applies to Key Issue 5.

    - New Resources and Procedures

      - Introduction

A new resource <subscriptionAssociation> is proposed to implement Cross-Resource Subscription functionality, associating with existing <subscription> resources.

Basically two-way links are maintained between a <subscriptionAssociation> resource and <subscription> resource(s). So that individual notification per <subscription> resource is sent to the <subscriptionAssociation> resource Hosting CSE referring the link in the <subscription> resource.

This link is also used to manage the list of individual subscriptions in the <subscriptionAssociation> resource. E.g. when a <subscription> resource is removed, then the link to the resource in the <subscriptionAssociation> resource is removed also. If there is a newly created resource subscription and needs to be associated with the existing <subscriptionAssociation> resource, then the subscriber can just add the link to that <subscription> resource. There's no need to do something for the <group> resource update.

To have a single event notification from cross or multiple resource subscriptions, the timeWindow attribute is suggested to check all the individual events are occurred during the time window, so the single notification needs to be sent to the subscriber of the <subscriptionAssociation> resource.

      - New subscriptionAssociation resource type

The new resource type subscriptionAssociation is suggested to have link/association to existing <subscription> resources. The Hosting CSE of this resource receives notifications from the <subscription> resource Hosting CSEs, and generate a single notification to the subscriber when all the events from individual subscription occurred in a time window that is set by the subscriber. The subscriber considers those events occurred within the window consist of a meaningful information so the single notification for that is enough.

Table 10.7.3.2-1: Child resources of <subscriptionAssociation> resource

| Child Resources of <container> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| subDel | <subscriptionLinkDeletion> | 1 | This is the virtual resource only permits DELETE operation. The <subscription> Hosting CSE in the subsciptionIDs list is allowed to delete the <subscription> resource from the list. |

Table 10.7.3.2-2: Resource specific attributes of <subscriptionAssociation> resource

| Attributes of <subscriptionAssociation> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| subscriptionIDs | 1 (L) | RW | This attribute indicates the resource address(es) of associated <subscription> resources. |
| timeWindow | 1 | RW | This attribute indicates the time duration (e.g. in seconds) that the cross-resource subscriber considers all associated subscription events are occurred in the time window so that a single notification is need for those event. |
| notificationType | 1 | RW | This indicates the type of information for notifications of this <subscriptionAssociation> resource. Possible values are: 1) simple notice that all associated subscriptions have events in the time window; and 2) aggregation of the notifications from associated subscriptions. |

      - Extension to subscription resource type

A new attribute is added to have a link to <subscriptionAssociation> resource. This is used to remove the link from the associated subscription to the individual subscription as well as to send a notification to the <subscriptionAssociation> resource Hosting CSE.

Table 10.7.3.3-1:New attributes of <subscription> resource

| Attributes of <subscription> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| associatedSub | 0..n | RW | This attribute lists eventNotificationCriteria for each target resource involved in a cross-resource subscription. |

      - Procedure to create subscription association

When an Originator requests to create a <subscriptionAssociation> resource, after the basic checking procedures for resource creation as defined in TS-0001, the Hosting CSE creates the <subscriptionAssociation> resource. After this, the Hosting CSE sends UPDATE request(s) to <subscription> resources indicated in the subscriptionIDs attribute to include the associatedSub attribute targeting the created <subscriptionAssociation> resource.

      - Procedure to manage subscription association

After the creation of a <subscriptionAssociation> resource, when the subscriber wants to associate a new <subscription> resource to the existing subscription association, the <subscription> Hosting CSE sends UPDATE Request to the <subscriptionAssociation> resource to update the subscriptionIDs attribute.

After the creation of a <subscriptionAssociation> resource, when an associated <subscription> resource having the associatedSub attribute gets deleted, the <subscription> Hosting CSE sends DELETE Request to the virtual child resource, which has the fixed name "subDel", of the <subscriptionAssociation> resource. Then the <subscriptionAssociation> resource does not wait for notifications coming from the <subscription> Hosting CSE anymore to generated a single event to the subscriber.

      - Procedure to generate notifications of cross-resource subscription

After the creation of a <subscriptionAssociation> resource, when the subscriber wants to associate a new <subscription> resource to the existing subscription association, the <subscription> Hosting CSE sends UPDATE Request to the <subscriptionAssociation> resource to update the subscriptionIDs attribute.

  - Solution H: Time Synchronization using beacons

    - Solution Description

This solution addresses Key Issue 7 which highlights that services requiring time-sensitive operations rely on the various Service Layer entities being time-synchronized.

CSEs can be configured by one or more entities to send SL time synchronization beacon notifications. The beacons can be customized to be sent at specific frequencies (e.g. every 5 minutes) or in response to customized conditions or events (e.g. certain synchronization offset has been exceeded), and the settings can be specified by one or more entities.   The beacons can be useful for keeping an entity synchronized with the current local time of a CSE especially when the entity is communicating with the CSE on an intermittent basis where clock drift and synchronization issues can arise.

Time synchronization beacon notifications contain information such as the local time of the originator of the beacon and other coordination scheduling information e.g. information used by receivers to program their sleep and low power schedules.

The solution relies upon the following elements:

- <timeSynchBeacon> resources are created by entities requesting time synchronization services from the Hosting CSE. The Hosting CSE provides time synchronization services in the form of beacon notifications that are sent to the entities.

- The Hosting CSE provides notifications with the time-synchronization information (beacon message) based on the configured parameters. The notification includes: originator CSE-ID, target, CSE current local time, and other beacon notification event information.

- The beacon receiver can extract a CSE current local time component, compare it against its own local time and an offset is computed.  The offset can then be used to synchronize the local time of the beacon receiver to the local time of the beacon originator.

    - Solution Applicability

This solution applies to Key Issue 7

    - Solution Details

To implement this solution, the new <timeSynchBeacon> resource type is introduced, as described in

Table 10.8.3-1. Existing resource types <CSEBase>, <remoteCSE> and <AE> are modified to include <timeSynchBeacon> as optional child resources.

Table 10.8.3-1: <timeSynchBeacon> Resource Attributes

| New Attributes | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| timeSynchBeaconInterval | 0..1 | RW | Frequency of beacons expressed as the minimum time between two beacon notifications sent by the Hosting CSE to the beacon target. This attribute is used for the periodic timeSynchBeaconCriteria. If this attribute is not configured and timeSynchBeaconCriteria is set to “Periodic” then the Hosting CSE will use a default value based on local policies. |
| timeSynchBeaconThreshold | 0..1 | RW | Synchronization time offset threshold. Used for the “Loss of Synchronization” timeSynchBeaconCriteria. When the delta between the current local time of the Hosting CSE and the local time of the beacon target specified in the timeSynchBeaconTarget exceeds this defined threshold, then the Hosting CSE sends a beacon to the target. The local time of the beacon target can be extracted by the Hosting CSE using time synchronization parameters present in the request and response messages that the beacon target sends to the Hosting CSE. If this attribute is not configured and timeSynchBeaconCriteria is set to “Loss of Synchronization” then the Hosting CSE will use a default value based on local policies. |
| timeSynchBeaconTargets | 1 (L) | RW | This attribute is configured with the list of targets that the Hosting CSE sends beacon notifications to. A target is formatted as a oneM2M compliant Resource-ID or as an identifier compliant with a oneM2M supported protocol binding (e.g. http, coap, mqtt). If a target is formatted as a oneM2M compliant Resource-ID, then the target is formatted as a structured or unstructured CSE-Relative-Resource-ID, SP-Relative-Resource-ID, and/or Absolute-Resource-ID of an <AE> or <remoteCSE> resource. A Hosting CSE uses this information to determine proper pointOfAccess, requestReqchability and/or pollingChannel information needed to send a beacon notification to the target. The following is an example. /CSE0001/AE0001 For a target that is formatted as an identifier compliant with a oneM2M supported protocol binding, the details of this format are defined by the respective oneM2M protocol specification. The following is an example of an HTTP URI compliant with oneM2M HTTP protocol binding. https://172.25.30.25:7000/notification/handler A notification serialization type may be appended to a notification target. The Hosting CSE serializes notifications and sends them to the target based on this serialization type indicator (e.g. XML, JSON or CBOR). If a notification serialization type is not appended to a notification target, a default is applied based on the Hosting CSE local policy. The syntax for appending a serialization type to a notification target uses the “?” delimiter character as shown in the below examples. http://mydomain/notificationHandler?ct=json CSE02/base/ae2?ct=xml |
| timeSynchBeaconCriteria | 1 | RW | Criteria for the beacon generation. - Periodic – Beacon is sent periodically based on the period defined in timeSynchBeaconInterval - Loss of Synchronization – Beacon is sent as a result of a detection in a loss of synchronization exceeding the defined timeSynchBeaconThreshold |

To support sending time synchronization information to beacon targets within the payload of a oneM2M notification, a new data type is defined within the existing oneM2M notification data object as described in .

Table 10.8.3-2: Data Types for notification data objects

| Root Element Name | Request Optionality | Data Type | Default Value and Constraints |
| --- | --- | --- | --- |
|  | N |  |  |
| timeSynchBeacon | O | m2m:timeSynchBeaconInfo |  |

Table 10.8.3-3 defines a proposed definition for the m2m:timeSynchBeacon data type.

Table 10.8.3-3: Type Definition of m2m:timeSynchBeacon

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| currentLocalTime | m2m:timestamp | 1 | This element is configured with the local time of the Hosting CSE. |
| currentTimeOffset | xs:duration | 0..1 | This element is included only if timeSynchBeaconCriteria is set to “Loss of Synchronization”. This element is configured with the amount of time offset calculated by the Hosting CSE and that triggered the beacon. The beacon target should adjust its local time by this specified offset. |

  - Solution I: Time Synchronization using request and response parameters

    - Solution Description

This solution addresses the time synchronization Key Issue 7.

In this proposal, the Originating Timestamp message parameter is used to advertise as well as compare the current times of a CSE and the entities that communicate with it.  The Originating Timestamp parameter can be used to share current times, compute time offsets, and adjust/correct times such that they remain synchronized with one another.  The Originating Timestamp  message parameter can be included in requests as well as in response messages originated by the CSE or the entities that communicate with it.

In this solution a Service Layer Ping message is also proposed, enabling the calculation of network latency for time compensation.

    - Solution Applicability

This solution applies to Key Issue 7.

    - Solution Details

When an entity creates a <timeSynchBeacon> resource on a Hosting CSE and configures the timeSynchBeaconCriteria attribute with a value of “Loss of Synchronization”, the entity is requesting that the Hosting CSE monitor the offset between the current time of the entity and the current time of the Hosting CSE to detect if/when there is a delta that exceeds a certain threshold.  To perform this monitoring, the entity must provide its current time on a regular basis to the <timeSynchBeacon> resource Hosting CSE.  To facilitate this exchange, the Originating Timestamp request and response parameters can be used.

Note based on deployment and implementation use case requirements, the Originating Timestamp message parameter can be included in all request and response messages originated by an entity targeted towards a <timeSynchBeacon> resource Hosting CSE, or only in select messages (e.g. one every n seconds or one in every n messages).  This allows providing adequate information for maintaining proper synchronization without introducing unnecessary overhead.

Note, when an entity creates a <timeSynchBeacon> resource and configures the timeSynchBeaconCriteria attribute with a value of “Periodic”, the exchange of current timing information and the use of Originating Timestamp is not required.

When an entity configures a <timeSynchBeacon> resource with the timeSynchBeaconCriteria attribute set to a value of “Loss of Synchronization”, the Hosting CSE can optionally compute the transit time (i.e. network latency) of messages that flow between itself and the entity.  The Hosting CSE can take this time into account when calculating the offset between the current time of the entity and the current time of the Hosting CSE.  This enables the Hosting CSE to calculate the offset with added precision.   When receiving messages from the entity with the Originating Timestamp parameter configured, the Hosting CSE can first adjust this value by the transit time before comparing the value against its own current time and computing an offset.

To compute the transit time between the entity and the <timeSynchBeacon> resource Hosting CSE, a new Ping (P) operation can be defined in addition to the existing Create (C), Retrieve (R), Update (U), Delete (D), Notify (N) operations.  The <timeSynchBeacon> resource Hosting CSE can use the Ping operation along with a Originating Timestamp parameter within requests that it targets towards the pointOfAccess of an entity that created the <timeSynchBeacon> resource.  When the entity receives the Ping request and returns a Ping response, the entity can echo back the value of the Originating Timestamp parameter of the Ping request in a Originating Timestamp parameter within the Ping response. Upon receiving the Ping response, the <timeSynchBeacon> resource Hosting CSE can compute the round-trip transit time by substracting the value contained in the Originating Timestamp parameter of the Ping response from the time when it received the Ping response message.

Editor’s Note:  It is FFS whether targeting a Ping request towards the pointOfAccess of an entity is optimal or whether defining a new pingURI is required.

  - Solution J: Network Communication Management

    - Network Communication Management function overview

The Network Communication Management (NCM) function of the IN-CSE allows AEs to request communication mode planning service for a Node (e.g. ADN, ASN, MN) or a group. Such communication mode planning requests could be from an AE residing on either a local Node or a remote Node.

Step 1-2: When the Network Communication Management (NCM) of the IN-CSE receives this service request with the route information of the targeted Node or group, Network Communication Management (NCM)  shall plan the right communication mode list according to the communication mode capability of the targeted Node or the members of targeted group and the deployment information of Underlying Network along with the route information.

Step 3: The IN-CSE and the targeted Node/Group shall update the current communication mode according to the current location and planned communication mode list.

Figure 10.10.1-1: Overview of Network Communication Management

In order to address the Key Issue 8, when the service is requested for a group, the Network Communication Management (NCM) provides the communication mode for V2V communication in a platoon group, according to group information (e.g. Member ID, lead vehicle ID, supported communication mode for each member and route information of the platoon), network status information (e.g. network coverage deployment, congestion status) and map information (e.g. road topology, building location).

The solution in this TR focuses on 3GPP underlying network and static network information (e.g. network MBMS deployment). The dynamic networking information (e.g. network congestion status) is FFS.

    - Detailed Procedure

      - Network communication management procedure for a single Node

        - Network Communication Planning Procedure

Figure 10.10.2.1.1-1: Solution Procedures for Network communication planning procedure

The basic procedure is described below:

Step 1: An ADN-AE or MN/ASN-CSE creates the <node> resource hosted on the IN-CSE with a supportedCommModes attribute.

Note: The ADN-AE or ASN/MN-CSE must be the Registree of the IN-CSE to support this procedure.

Step 2: The ADN-AE or MN/ASN-CSE sends <commModePlan> CREATE request to IN-CSE, in which the value of Result Content is modified-attributes.

Step 3: The IN-CSE creates the <commModePlan> resource including planning communication modes according to the routeInformation and network information including PC5/ LTE-Uu geographical area configuration. IN-CSE configures the plannedCommModeList which are the planned node communication modes.

For example:

The supportedCommModes of the ADN-AE are LTE-Uu and PC5, the routeInformation is (A, B, C), and the network information is as below:

Table 10.10.2.1.1-1: Network deployment information

| Area information | Network deployment |
| --- | --- |
| area1 | PC5 & LTE-Uu & MBMS |
| area2 | LTE-Uu & MBMS |
| area3 | PC5 |
| area4 | PC5 & LTE-Uu |

Figure 10.10.2.1.1-2: Example of the network communication planning

The IN-CSE compares the routeInformation and networking information, decides the route segment, and determines the communication mode according to the supportedCommModes of the targeted ADN-AE and the networking information for each route segment. So the plannedCommModeList is:

Table 10.10.2.1.1-2: plannedCommModeList of the <commModePlan>

| communication mode | route information |
| --- | --- |
| LTE-Uu | (A,B) |
| PC5 & LTE-Uu | (B,D) |
| PC5 | (D,C) |

Step 4: The IN-CSE sends the response including the plannedCommModeList.

Step 5: The ADN-AE, or a MN/ASN-CSE creates <subscription> for the <commModePlan> resource.

Step 6 (optional): The IN-CSE sends a notification that includes updated plannedCommModeList to the ADN-AE, or a MN/ASN-CSE when the resource is updated.

        - Network communication mode switching procedure

Figure 10.10.2.1.2-1: Solution Procedures for Network Communication Mode Switching

Step1: The ADN-AE, or a MN/ASN-CSE determines the new communication mode according to the plannedCommModeList and current location.

Step 2: The ADN-AE, or a MN/ASN-CSE sends the UPDATE request for the currentCommMode of the <node>.

Step 3: The IN-CSE sends response to the ADN-AE, or a MN/ASN-CSE.

      - Network communication management procedure for a Group

        - Network communication planning procedure

Figure 10.10.2.2.1-1:Solution Procedures for Network communication mode planning

The basic procedure is described as below:

Pre-conditions:

The MBMS service area information and PC5/LTE-Uu geographical area information provided by the operator is configured in the oneM2M System;

Step 1: An MN/ASN-CSE creates the <node> resource on the IN-CSE with the supportedCommModes attribute. The MN/ASN-CSE creates a <subscription> resource as a child of the <node> resource.

Note: The MN/ASN-CSE must be the Registree of the IN-CSE to support this procedure.

Step 2a and 2b: An ASN/MN-AE, or an IN-AE sends a <group> resource CREATE request to the IN-CSE, including memberIDs, memberRoles (e.g. lead vehicle of the platoon). For the platooning use case, the memberIDs attribute represents the <node> resource IDs that each ASN/MN-CSE resides. The IN-CSE creates the <group> resource.

Step 3: The lead member sends <commModePlan> resource CREATE request to the IN-CSE, in which the value of Result Content is “modified-attributes”.

Step 4: The IN-CSE creates the <commModePlan> resource including targetedID attribute check. After fetching the attribute, if the targetedID attribute represents the <group> resource ID, the IN-CSE retrieves the <node> resources of each member Hosting CSE. Then the IN-CSE gets the supportedCommModes attribute of the <node> resource associated with each member resource. The IN-CSE plans the group communication modes according to the routeInformation attribute,  the <group> resource ID, and the network deployment information, and supplementary information, e.g. the map information of road topology. The IN-CSE obtains the network deployment information according to the MBMS SAI list, PC5/ LTE-Uu coverage geographical area configuration. The IN-CSE configures the plannedCommModeList attribute which are the planned group communication modes.

Step 5: The IN-CSE sends the response to the leader member Hosting CSE including the plannedCommModeList attribute.

        - V2X network communication switching procedure

          - Option 1: The lead vehicle sends switching message to each member to update the current communication mode

Figure 10.10.2.2.2.1-2: Solution Procedures for communication mode switching of Option 1

Step 1: The lead member Hosting CSE creates <subscription> resource for the <commModePlan> resource.

Step 2: The IN-CSE sends a notification that includes updated plannedCommModeList attribute to the MN/ASN-CSE hosted on the lead vehicle of the platoon when  the IN-CSE receives the UPDATE request from the IN-AE to update the routeInformation attribute of the <commModePlan> resource and then updates the plannedCommModeList attribute.

Step 3: The lead member Hosting CSE determines the new communication mode for all members according to the plannedCommModeList and current location assuming all the member Hosting CSEs follow the lead member Hosting CSE.

Step 4: The lead member Hosting CSE sends UPDATE request for the currentCommMode of the <node> resource to other members to switch communication mode by current communication mode (e.g. PC5) if the member Hosting CSEs host the <node> locally. Or the lead member Hosting CSE sends UPDATE operation for the currentCommMode of the <node> resource to the <fanoutPoint> attribute of the <group> resource created in the step 2 of 10.x.2.2.1 if the member Hosting CSEs host the <node> on the IN-CSE.

Note: In this use case, it is assumed the lead member Hosting CSE has UPDATE privileged to the other members’ <node> resources

Step 5: The other member Hosting CSEs receive the UPDATE or NOTIFY and switch to the new communication mode (e.g. LTE-Uu) according to the received request.

Note: The solution of the communication procedure between vehicle and vehicle by interworking between oneM2M and 3GPP for PC5 is outside the scope of the current document..

          - Option 2: IN-CSE updates the current communication mode for each member

Figure 10.10.2.2.2.2-1: Solution Procedures for communication mode switching of Option 2

Pre-condition:

The IN-CSE is able to detect the location of its Registree ASN/MN-CSEs.  For example, <locationPolicy> resources with Network Based mode may be used by the IN-CSE to get the location of each ASN/MN-CSE.

Step 1: The IN-CSE determines the new communication mode for all members according to the plannedCommModeList and current location of each member.

Step 2: The IN-CSE updates the currentCommMode attribute of <node> resource for each member Hosting CSE.

Note: In this use case, it is assumed the IN-CSE has UPDATE privileged to the members’ <node> resources.

Step 3: Each Member Hosting CSE of the group switches to the new communication mode according to the notification.

          - Option 3: Each member updates the current communication mode

Figure 10.10.2.2.2.3-1: Solution Procedures for communication mode switching of Option 3

Step 1: Each Member Hosting CSE of the group creates a <subscription> resource for the <commModePlan> resource.

Note: Provision the <commModePlan> resource identifier to each member is out-of-scope.

Step 2: The IN-CSE sends a notification that includes updated plannedCommModeList to the each member Hosting CSE of the group when the IN-CSE receives the UPDATE request from the IN-AE to update the routeInformation attribute of the <commModePlan> resource and then updates the plannedCommModeList attribute.

Step 3: Each member Hosting CSE determines the new communication mode for itself according to the plannedCommModeList and current location.

Step 4: Each member Hosting CSE updates the currentCommMode attribute of <node> and switches to the new communication mode.

        - V2X network communication switching procedure

Figure 10.10.2.2.3-1: Solution Procedures for Group fanout by the memberRoles

Step1: The IN-AE sends operation for the <fanOutPoint> resource of the group with the <group> resource ID.

Step2: The IN-CSE checks the <group> resource ID whether is the same with the targetedID of the <commModePlan> resource. If the same, then IN-CSE checks the memeberRoles attribute of the <group>. The IN-CSE determines the fanout mode according to the currentCommMode attributes of the <node> resource of all member Hosting CSEs:

Case A: If the memeberRoles attribute is configured and there is a lead member defined in the list, and if the currentCommMode attribute of the lead member Hosting CSE and other member Hosting CSEs are PC5,  go to Step 3a:

Step 3a:  The IN-CSE sends the operation to the lead member Hosting CSE by unicast.

Step 4a:  The lead member forwards the operation to the other Member Hosting CSEs by PC5 or multicast broadcast forcing the From in the request to be the IN-CSE ID. Go to Step 5.

Note: The procedure of communication with field nodes using PC5 is outside the scope of the current document.

Case B: If the currentCommMode attribute is Unicast over LTE-Uu,  go to Step 3b:

Step 3b: the IN-CSE sends the operation to each group Member Hosting CSE by unicast. Go to Step 5.

Case C: If the currentCommMode attributes are IP multicast or MBMS, go to Step 3c:

Step 3c:  The IN-CSE sends the operation to the group Member Hosting CSEs by IP multicast or MBMS.

Step 5: Each Member Hosting CSE sends the response to the IN-CSE.

Step 6: The IN-CSE sends the aggregated response to the IN-AE.

    - Solution Applicability

This solution applies to Key Issue 8.

    - New Resources and Procedures

      - Introduction

To implement this solution, new resource <commModePlan> is proposed, and <group>, <node> resource need to be modified. The plannedCommModelist maintains a list of communication modes that can be used in a specified area. These are further detailed in the following clauses.

      - Modified <node> resource

New child resource and attribute as shown in Table 10.10.4.2-1.

Table 10.10.4.2-1: New attributes of <node> resource

| Attributes of <node> | Multiplicity | RW/ RO/ WO | Description | <nodeAnnc> Attributes |
| --- | --- | --- | --- | --- |
| supportedCommModes | 0..1 (L) | RW | Indicates the oneM2M node supported communication mode capability, e.g. supported V2V communication mode, pre-defined values are: - PC5 - Unicast over LTE-Uu - MBMS - IP multicast | OA |
| currentCommMode | 0..1(L) | RW | Indicates the oneM2M node current supported communication mode to receive the message, e.g. supported V2V communication mode, pre-defined values are: - PC5 - Unicast over LTE-Uu - MBMS - IP multicast | OA |

Note: PC5 communication in oneM2M is outside the scope of the current document.

      - New <commModePlan> resource type

The attribute as shown in Table 10.10.4.3-1. The routeInformation attribute indicates the group or node driving area which is requested for communication mode planning. The parent resources of the <commModePlan> are <CSEBase>, <AE> and <remoteCSE>.

Table 10.10.4.3-1: Resource specific attributes of <commModePlan> resource

| Attributes of <commModePlan> | Multiplicity | RW/ RO/ WO | Description | <commModePlanAnnc> Attributes |
| --- | --- | --- | --- | --- |
| routeInformation | 1 | RW | Indicates the route information that the oneM2M node may pass through as specified in the ETSI ITS TS 102 894-2 Annex A A-113 [i.24], e.g. the route of a platoon or a geographical area where the platoon is estimated to pass. When routeInformation is exit, the IN-CSE should plan communication mode according to this attribute for the oneM2M node. | OA |
| targetedID | 1 | RW | Indicates the targeted <node> resource ID or <group> resource ID which the plannedCommModeList applies to. When the targetedID is a groupID, the member hosting CSEs are different. | OA |
| plannedCommModeList | 1 | RO | List of two elements: communication mode and route segment information. The first element indicates the communication mode that oneM2M node to be used to communicate. The pre-defined values for communication mode are: PC5, unicast over LTE-Uu and MBMS. The second element indicates the geographic area where the communication mode is used. | OA |

      - Modified <group> resource

New attribute as shown in Table 10.13.3.1-1.  The memberRoles attribute is used to indicate the leader of a group, and the leader should be one of members of the group.

Table 10.10.4.4-1: New attributes of <group> resource

| Attributes of <group> | Multiplicity | RW/ RO/ WO | Description | <groupAnnc> Attributes |
| --- | --- | --- | --- | --- |
| memberRoles | 0..1 | RW | List of arrays which consist of two elements: role and memberID. The role is the role of the member in a group: - 0: Lead - 1: reserved for the future use. The memberID should be one of the memberIDs. This attribute is used when the member roles of a group need to be specified, e.g. the lead vehicle of a platoon. | OA |

  - Solution K: Time Synchronization using currentTime

    - Solution Description

This solution addresses Key Issue 7 which highlights that services requiring time-sensitive operations rely on the various Service Layer entities being time-synchronized.  This solution complements the time synchronization beacon-based solution and is targeted towards deployments involving constrained IoT devices that only support client functionality and not server functionality.  Client-only devices cannot receive requests and therefore receiving of  synchronization beacons is not ideal for these types of devices. When hosting a oneM2M AE on these types of devices a lightweight client-only mechanism is a better fit for synchronizing an AE’s current time to the current time of its Registrar CSE.

This solution proposes introducing a new currentTime attribute to the <CSEBase> resource to provide Registrees of a CSE the capability to retrieve the current time of their Registrar CSE.  After retrieving their Registrar’s current time, a Registree can then synchronize its time to its Registrar’s time.

Note that nodes hosting oneM2M entities should keep their time synchronized to Coordinated Universal Time (UTC) using mechanisms such as NTP or GPS whenever possible.  However if this is not possible, then this proposed oneM2M time synchronization method can be used.

    - Solution Applicability

This solution applies to Key Issue 7

    - Solution Details

To implement this solution, a new currentTime attribute is proposed for the <CSEBase> resource type as described in Table 10.11.3-1.

Table 10.11.3-1: <CSEBase> Resource Attributes

| New Attributes | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| currentTime | 0..1 | RW | The current time value on the node hosting this CSE. When the CSE receives a retrieve request targeting this attribute or the <CSEBase> resource, it can trigger an action to sample its current time (e.g. make an OS call to get system time) and respond with this value. An Originator retrieving this attribute can use this time value to adjust and synchronize its time value to that of its Registrar CSE. |

  - Solution L: Cross Resource Subscription Management

    - Solution Description

This solution addresses the Key Issue 5 – cross Resource subscription by proposing three new functionalities within <crossResourcesubscription>.

- Use of <cross-notification> only when expected changes occur in sequence that refers to the order in which expected events happened.

The Hosting CSE is now defined to generate a cross-resource notifications only when expected changes occur on a designated number of target resources concurrently within a time window. But there is a case where the subscriber wants to get a cross-resource notifications only when expected changes occur in a given sequence.

But, because current cross-resource notification is generated regardless of the sequence, the recevicer should check on whether the cross-resource notification is meaningful or not based on sequence information (the sequence of eventNotificationCriteria of regularResourcesAsTarget and subscriptionResourceAsTarget). This might be burden to the receiver (application). This burden can be relieved by having sequence information of eventNotificationCriteria of <crossResourcesubsciption>.

- Independent expiration counter for <crossResourcesubsciption>

The <subscription> resource has expirationCounter attribute as a deleting policy. Similarly, the cross-resource subscriber wants to have independent deleting policy for <crossResourceSubsciption>. In this regard, it needs to define expirationCounter for <crossResourceSubsciption>

- Update expiration counter in target resource while creating cross resource subscription

The <crossResourcesubsciption> resource is related to a set of target resources which could be existing <subscription> and/or other subscribable oneM2M resources. If a target <subscription> resource is deleted by reaching expirationCounter, the <crossResourcesubsciption> shall be deleted accordingly. But while creating new <crossResourcesubsciption> resource, if the current number of notification of the target <subscription> almost reach into expirationCounter, the <crossResourcesubsciption> resource can be deleted a short time later after creating the <crossResourcesubsciption> resourece, which is not situation that the cross-resource subscriber expects. For example, if the expirationCounter for target subscription is updated by adding expirationCounter of new <crossResourcesubsciption>, above mentioned problem can not be happened. In this regard, a new attribute called expirationCounterupdatepolicy be defined.

    - Solution Applicability

This solution applies to Key Issue 5.

    - Solution Details

N/A

    - Overview

To implement this solution, <crossResourceSubscription> resource needs to be updated as follows:

- eventNotificationCriteriaSet supports new functionality regarding sequence

- new attributes such as expirationCounter and expirationCouter update policy

    - Modified attribute

The eventNotificationCriteriaSet has new functionality regarding sequence, therefore cross notifications can appear only when the given sequences are satisfied. (see Table 10.12.5-1)

Table 10.12.5-1:  Modified attribute of <crossResourceSubscription> resource

| Attributes | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| eventNotificationCriteriaSet | 0..1(L) | RW | This attribute lists eventNotificationCriteria for each regular target resource as indicated in regularResourcesAsTarget attribute and involved in a cross-resource subscription. If there is only one eventNotificationCriteria contained in this attribute, it shall be applied to all target resources as indicated by regularResourcesAsTarget attribute. If only subscriptionResourcesAsTarget attribute appears (i.e. no regularResourcesAsTarget attribute), eventNotificationCriteriaSet shall not be needed. This attribute may include an indiation of a sequence of eventNotificationCriteria. In this case, the specified eventNotificationCriteria should be satisfied in the given sequence so that the subscriber can get a notification only when mutiple eventNotificaionCriteria are met in the specified sequence. See clause 9.6.8 in oneM2M TS-0001 [i.3] for the description of eventNotificationCriteria. |

      - New attribute

The <crossResourcesubsciption> has its own deleting policy by introducing new attribute expirationCounter and update policy while creating <crossResourcesubsciption> in order to avoid unwanted deletion.

Table 10.12.5.1-1:  New attribute of <crossResourceSubscription> resource

| Attributes | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| expirationCounter | 0..1 | RW | This attribute indicates that the subscriber wants to set the life of this crossResourcesubscription to a limited number of maximum notifications. When the number of sent out notifications reaches the value of this counter, the <crossResourcesubscription> resource shall be deleted, regardless of any other policy. |
| expirationCounterUpdatePolicy | 0..1 | RW | This attribute indicates update policy on expirationCounter of the target subscription. The possible values for expirationCounterUpdatePolicy are : - “NoUpdate” - “AddUpdate” “AddUpdate” indicates that expirationCounter of <crossResourceSubscription> resource is added to expirationCounter of target <subscription>. |

  - Solution M: Time Compensation

    - Solution Description

This solution addresses the time synchronization Key Issue 7 by proposing time offset compensation functionality within a Registrar CSE.

In this proposal a new time synchronization solution involving a CSE performing time offset compensation on behalf of one of its Registrees is defined.  This time offset compensation can be performed by a CSE when it detects a lack of synchronization between itself and one of its Registrees that exceeds a specified threshold.  A CSE can use timing information that a Registree includes within the Originating Timestamp oneM2M message parameter to calculate the amount of offset between its current time and the current time of the Registree.  Based on this offset, a CSE can then compensate for the offset by making adjustments to oneM2M timing related metadata contained within the oneM2M requests it receives from the Registree.   In doing so, a Registree’s messages can be synchronized to the current time of the CSE without burdening the Registree with having to maintain synchronization with the CSE.

Some examples of timing related metadata in a oneM2M request that is expressed in absolute format (e.g. 20141003T112032) and that a Registrar CSE can perform time offset compensation includes the following.

- originatingTimestamp

- requestExpirationTimestamp

- resultExpirationTimestamp

- operationExecutionTime

- resultPersistence

- createdBefore

- createdAfter

- modifiedSince

- unmodifiedSince

- expireBefore

- expireAfter

- creationTime

- lastModifiedTime

- expirationTime

- createdBefore

- createdAfter

- modifiedSince

- unmodifiedSince

- startTime

- completeTime

- eventStart

- eventEnd

- dynamicAuthorizationLifetime

In this proposal, new resource attributes are proposed to the <AE> and <remoteCSE> resources to allow a Registree to enable its Registrar CSE to perform time offset compensation on its behalf.

    - Solution Applicability

This solution applies to Key Issue 7.

Note, this solution is applicable to the 0-hop deployment scenario involving an ADN-AE or ASN-CSE which does not support synchronizing its time reference to its Registrar MN-CSE. This is not targeting a multi-hop deployment scenario.

    - Solution Details

A Registree can enable its Registrar CSE to perform time offset compensation on its behalf by configuring the enableTimeCompensation attribute of the <AE> and <remoteCSE> resources defined in Table 10.13.3-1.

Table 10.13.3-1: Time Compensation Attribute of <AE> and <remoteCSE>

| Attributes | Multiplicity | RW/ RO/ WO | Description | Announced Attributes |
| --- | --- | --- | --- | --- |
| enableTimeCompensation | 0..1 | RW | Enables time offset compensation functionality. When set to TRUE, the Registrar CSE peforms time offset compensation and when FALSE it does not. Default is FALSE. | NA |

Figure 10.13.3-1: Solution Procedures for time compensation

Step 1: Registree AE/CSE registers to Registrar CSE by issuing a CREATE request for an <AE> or <remoteCSE> resource.  Within this request, the Registree enables time offset compensation processing by setting the  enableTimeSyncrhonization to a value of TRUE.

Step 2: The Registrar CSE creates the <AE> or <remoteCSE> resource and enables time offset compensation functionality.

Step 3: The Registrar CSE generates a response.

Step 4: The Registree sends a subsequent request to the Registrar sometime later.  In this request, the Registree includes its Originating Timestamp request parameter.

Step 5: The Registrar CSE then computes the time offset between its time and the time of the Registree specified in the Originating Timestamp request parameter.

Step 6: The Registrar CSE performs time offset compensation to the incoming request by adjusting any time stamp related metadata that is expressed in absolute time format.  The adjustment is performed using the computed time offset to adjust time stamp related metadata within the request to compensate for any offset.

Step 7: The Registrar CSE performs time offset compensation to the outgoing response by adjusting any time stamp related metadata that is expressed in absolute time format.  The adjustment is performed using the computed time offset to align time stamp related metadata within the response to account for any time offset between the Originator and Receiver.

Step 8: The Registrar CSE returns a response.  Within the response, any time stamp related metadata is compensated to account for any time offset between the Originator and Receiver.

Annex A: oneM2M data model for vehicular domain

A.1	AUTOPILOT

European Large Scale Pilot (LSP) Autopilot [i.18] is project which is focused on use of IoT for improving autonomous driving. “Automated driving Progressed by Internet Of Things” (AUTOPILOT) will bring IoT into the automotive world to transform connected vehicles — moving ”things” in the IoT ecosystem — into highly automated vehicles (towards levels 4 and 5 – see 5.4 Levels of Driving Automation on description of levels of automation). While using the IoT data for automated driving, AUTOPILOT will also make data from autonomous cars available to the Internet-of-Things platforms, thus being also source of data.

The extent and volume of information sources that can be addressed through internet of things is seamlessly unlimited, offering potential improvements of automated driving functions (including improvements in security, efficiency, accuracy, etc.) and the information will enable services involving automated driving. Various use cases are executed implemented at the 6 pilot sites of AUTOPILOT in large scale demonstrations in order to evaluate the potential and calculate the related impacts of using Internet of Things for Automated Driving.

The AUTOPILOT consortium represents all relevant areas of the IoT eco-system. Thanks to AUTOPILOT, the IoT eco-system will involve vehicles, road infrastructure and surrounding objects in the IoT, with particular attention to safety critical aspects of automated driving. AUTOPILOT IoT enabled autonomous driving cars are tested, in real conditions, at six permanent large-scale pilot sites in Finland, France, Italy, the Netherlands, Spain and South Korea.

A.2	AUTOPILOT and use of IoT

Automated driving is expected to increase safety, provide more comfort and create many new business opportunities for mobility services. The Internet of Things (IoT) is about enabling participating devices (and applications) to be able to access more data than typical connected car would, and in this way to be able to make better decisions, or create more precise world model. In both cases, idea is that having more data available, it will be possible to improve existing services as well as provide new types of services.

As example of new type of services, one can think of application which can predict movements of VRU (Vulnerable Road Users) and provide them as input to vehicle’s world model. In this example, cameras from smart city can monitor streets, and using object recognition software, detect cyclist going in particular direction. An application, let’s call it ‘motion trajectory prediction’ can use data on observed movement of cyclist and predict its trajectory. Another sapplication can take trajectory and combine it with known or prediucted trajectory of vehicle. If two would meet in the same spot and same time, that means that crash might occur between two – vehicle and cycle. This is something that will be out of scope of traditional connected car and cooperative ITS (intelligent Transport Systems) – since cyclist is not directly observable by vehicle.

A.3	AUTOPILOT, oneM2M and other IoT platforms

In real-life deployments there will be another IoT platforms deployed by other entities.

Example of multiple platforms are different IoT platforms that will be deployed by road operator and smart city operator. Typically, highways are under jusrisdiction and control of so-called road operator, or road authority, which is able to set and enforce rules for use of roads. For example, road operator can open emergency lane for traffic when road becomes too busy. Or they can reserve particular lane for platooning vehicles.

But when vehicle leaves highway and enters the city, roads / streets within city are under jurisdiction and control of other entity – smart city operator, one that has information on state of streets, parking places, roadworks, …

Depending on its location, vehicle will exchange data with different platforms. And those paltforms are not necessarily oneM2M based.

But some information about vehicle must be exchanged between these two platforms in our example, and in effect will lead to situation that we have federation of IoT platforms. And that puts emphasis on interworking, but also on data that will be exchanged via interworking interfaces.

A.4	oneM2M data model for vehicular domain

In AUTOPILOT, central IoT platform is oneM2M platform.

As currently there is not data model which is defined by oneM2M for vehicular domain, partners within AUTOPILOT are facing choice whether to go bottom-up, and to define which data is needed for given use cases, and then determine which data model to use. Or, to start from one of existing data models – for example SAREF [i.19] or Sensoris [i.20].

SAREF [i.19] stands for Smart Appliances Reference ontology, and it is OWL language (Web Ontology Language). It is conceived as a shared model of consensus that facilitates the

matching of existing assets in the smart appliances domain, reducing the effort of translating from one asset to another,  since SAREF [i.19] requires one set of mappings to each asset, instead of a dedicated set of mappings for each pair of assets.

But, SAREF [i.19] was designed initially for small applicances, and is currently being expanded (ETSI taskforce is working right now on it) to energy domain, agriculture and healthcare. But vehicular domain is still not covered.

Another development which is from automotive domain is Sensoris [i.20]. It is result of cooperation onf number of parties from automotive domain, and it proposed data model for exchanging data between vehicles and cloud. You can find more details in reference [i.20].

Currently, vehicle sensor data exists in different formats across automakers and it is typically carmaker specific. Even some protocols like CAN are defining transpoprt protocol for transporting data within vehicle, but do not go into standardizing messages that are transported. When connecting car to IoT platform - standardization is needed, as pooling analogous vehicle data from millions of vehicles will be a key enabler for bringing vehicle-to-vehicle and vehicle-to-infrastructure communication to the next level.

SENSORIS [i.20] was initiated by HERE in June 2015 when the company published the first open specification for how vehicle sensor data gathered by connected cars will be sent to the cloud (as well as between clouds) for processing and analysis.

A.5	Proposal for oneM2M data model for vehicular domain

Seeing that SAREF [i.19] is ETSI standard, but lacks vehicular data model, while Sensoris [i.20] is industrial standard which covers vehicular doimain, and of course there are others that for example cover real-time systems data models, we would like to propose to use SAREF [i.19] as the base to which we will add vehicle specific elements from Sensoris [i.20], and others namely regarding real-time systrem’s data models.

History

| Publication history |  |  |
| --- | --- | --- |
| V4.0.0 | <2018-03-23> | Release 4 baseline |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V.4.0.0 | <2018-03-23> | Merges of agreed the following contributions at REQ#34, ARC#34 and SEC#34, and editorial updates. 1. REQ-2018-0008R02 Use case for Accident Notification using Edge/Fog traffic Monitoring Service. 2. REQ-2018-0010R02 Use case for High-precision Road Map Service using Edge/Fog Computing 3. REQ-2018-0018R02- Smart Transportation with Edge/Fog deployment Merges of agreed the following contributions at REQ#34, and editorial updates. 1. REQ-2018-0012R03 Use case: Automated Valet Parking (AVP) 2. REQ-2018-0013R03 Use case: Platooning 3. REQ-2018-0014R03 Use case: Highway Pilot 4. REQ-2018-0016R04 Use case: Car Rebalancing 5. REQ-2018-0017R03 Use case: Urban Driving |
| V.4.1.0 | <2018-06-01> | Merges of agreed the following contributions at REQ#35, ARC#35 and SEC#35. REQ-2018-0031R01 Use case for reliable Edge/Fog computing. Merges of agreed the following contributions at ARC#35. ARC-2018-0106 TR-0026 Time Synch solutions |
| V.4.2.0 | <2018-07-26> | Merges of agreed the following contributions at REQ#36, ARC#36 and ARC35.2 and editorial updates. 1. REQ-2018-0047R05 Vulnerable Road User Discovery Use Case for Edge/Fog Computing. 2. ARC-2018-0208R02 TR-0026 KI_ and Solution of V2V network interface switching management 3. ARC-2018-0201R02 TR-0026 multiple registrations key issue 4. ARC-2018-0106R02 TR-0026 Time Synch solutions |
| V.4.3.0 | <2018-10-05> | Merges of agreed the following contributions at REQ#37. 1. ARC-2018-0277R01-Add some descriptions for the “LANE LEVEL” accuracy of geographic location 2. ARC-2018-0280R01- Update the Summary of levels of driving automation 3. REQ-2018-0067R01-Car_Bicycle_Sharing_Service 4. REQ-2018-0070R04-Use case Car to Home service based on Geo-Fence 5.REQ-2018-0071R05-Use case on vehicle remote control using External platform |
| V.4.4.0 | <2019-01-04> | Merges of agreed the following contributions at REQ#38. 1. REQ-2018-0085R01-Use Case for Vehicular Data Service with data model and Edge Fog 2. REQ-2018-0089R02-Use case Protocol selection in gateway for vehicle 3. REQ-2018-0090R02-Use case Software-defined radios (SDR) application for vehicle 4. REQ-2018-0094R01-Use Case of Dashcam Application |
| V.4.5.0 | <2019-03-04> | Merges of agreed the following contributions at SDS#39 and editrial update. 1. SDS-2019-0077R04-TR-0026 Overview of Network Communication Management 2. SDS-2019-0078R04-TR-0026 ServiceFlow of Network Communication Management for A Single UE |
| V.4.6.0 | <2019-05-27> | Merges of agreed the following contributions at SDS#39.x, SDS#40 and RDM#40 and editorial update. 1. SDS-2019-0062-TR-0026 timeSyncBeacon solution 2. SDS-2019-0063R01-TR-0026 time sync currentLocalTime solution 3. SDS-2019-0088R02-introduce a new attribute enhancing cross-resource subscription resource. 4. SDS-2019-0093R05-TR-0026 ServiceFlow of Network Communication Management for A Group 5. SDS-2019-0064R02-TR-0026_time_sync_request_response_params_solution 6. RDM-2019-0046R01-Use_Case_on_Volatile_Data_Management_Service 7. RDM-2019-0048R03-Use case of Offloading Service Continuity between Edge Fog Nodes |
| V.4.7.0 | <2019-07-13> | Merges of agreed the following contributions at RDM#41 and editorial update. 1. RDM-2019-0067R02-Handling privacy related regulation (Requirements) |
| V.4.7.1 | <2019-09-12> | Merges of agreed the following contributions at SDS#40 and editorial update. 1. SDS-2019-0065R04-TR-0026_time_sync_compensation_solution |
| V.4.8.0 | <2019-12-17> | Merges of agreed the following contributions at RDM#42 and editorial update. 1. RDM-2019-0089R01-Use_case_of_vehicle_idling_alarming_service |