| oneM2M Technical Report |  |
| --- | --- |
| Document Number | TR-0079-V-0.2.0 |
| Document Name: | oneM2M-ROS Interworking |
| Date: | 2025-07-25 |
| Abstract: | Technical Report on oneM2M-ROS (Robot Operating System) Interworking |
| 'Template Version: January 2019 (do not modify) |  |

The present document is provided for future development work within oneM2M only. The Partners accept no liability for any use of this report.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

© 2023, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TSDSI, TTA, TTC).

All rights reserved.

The copyright and the foregoing restriction extend to reproduction in all media.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

Contents	3

1	Scope	5

2	References	5

2.1	Normative references	5

2.2	Informative references	5

3	Definition of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Conventions	7

5	Overview of ROS	7

5.1	ROS middleware	7

5.1.1	Core concepts and components	7

5.1.2	Implementation structure	7

5.1.3	Use cases	8

5.2	ROS1	8

5.2.1	Centralized architecture	8

5.3	ROS2	8

5.3.1	Decentralized Architecture	8

5.3.3	Security framework	9

5.4	Benefits of oneM2M-ROS interworking	9

6	Interworking scenarios	10

6.1	ROS 2 protocol	10

6.1.1	ROS 2 communication types	10

6.1.2	Topic protocol	10

6.1.3	Service protocol	11

6.1.4	Action protocol	12

7	Resource mapping	13

8	Call flows	13

9	Conclusions	14

Annex <A>: Title of annex	15

Annex <y>: Bibliography	15

History	15

# 1	Scope

This document defines candidate solutions for oneM2M and ROS interworking including the folloing topics:

- Technical overview of ROS

- Interworking scenarios for ROS based robots with oneM2M applications

- Interworking procedures with resource mapping

- Identification of any additional requirements to oneM2M for candidate interworking solutions

# 2	References

The following text block applies.

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references,only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## 2.1	Normative references

As a Technical Report (TR) is entirely informative it shall not list normative references.

The following referenced documents are necessary for the application of the present document.

Not applicable.

## 2.2	Informative references

Clause 2.2 shall only contain informative references which are cited in the document itself.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	OMG, "Data Distribution Service (DDS) Specification," 2023.

NOTE:	Available at https://www.omg.org/spec/DDS

[i.2]	ROS2 Documentation, Understanding services, 2025.

NOTE:	Available at https://docs.ros.org/en/foxy/Tutorials/Beginner-CLI-Tools/Understanding-ROS2-Services/Understanding-ROS2-Services.html.

[i.3]	ROS2 Documentation, Quality of Service Settings, 2025.

NOTE:	Available at https://docs.ros.org/en/rolling/Concepts/Intermediate/About-Quality-of-Service-Settings.html.

[i.4]	ROS2 Documentation, Setting access controls, 2025.

NOTE:	Available at https://docs.ros.org/en/rolling/Tutorials/Advanced/Security/Access-Controls.html.

# 3	Definition of terms, symbols and abbreviations

Delete from the above heading the word(s) which is/are not applicable.

## 3.1	Terms

Clause numbering depends on applicability.

A definition shall not take the form of, or contain, a requirement.

The form of a definition shall be such that it can replace the term in context. Additional information shall be given only in the form of examples or notes (see below).

The terms and definitions shall be presented in alphabetical order.

For the purposes of the present document, the [following] terms and definitions [given in ... and the following] apply:

Definition format

<defined term>: <definition>

If a definition is taken from an external source, use the format below where [N] identifies the external document which must be listed in Section 2 References.

<defined term>[N]: <definition>

example 1: text used to clarify abstract rules by applying them literally

NOTE:	This may contain additional information.

## 3.2	Symbols

Clause numbering depends on applicability.

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

Symbol format

<symbol>	<Explanation>

<2nd symbol>	<2nd Explanation>

<3rd symbol>	<3rd Explanation>

## 3.3	Abbreviations

Abbreviations should be ordered alphabetically.

Clause numbering depends on applicability.

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

Abbreviation format

<ABBREVIATION1>	<Explanation>

<ABBREVIATION2>	<Explanation>

<ABBREVIATION3>	<Explanation>

# 4	Conventions

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# 5	Overview of ROS

## 5.1	ROS middleware

### 5.1.1	Core concepts and components

ROS (Robot Operating System) middleware serves as the communication backbone for distributed robotic systems, abstracting hardware complexities while enabling modular software development. ROS is not an international standard, but ROS2’s underlying middleware, DDS, complies with the OMG (Object Management Group) standard [i. 1]. Its design emphasizes flexibility, allowing integration with diverse hardware and software stacks.

Core components:

- Packages: The fundamental unit of ROS software organization. A package contains nodes, libraries, configuration files, and message/service definitions (e.g., sensor_msgs for standardized sensor data formats)

- Nodes: Nodes are lightweight, executable processes that perform specific tasks (e.g., sensor data processing, motor control). They communicate via topics or services, enabling decentralized computation

- Topics: Topics facilitate asynchronous, many-to-many communication through named channels. For example, a LiDAR node publishes point cloud data to /scan, which is consumed by mapping and obstacle detection nodes

- Services: Services enable synchronous, request-response interactions. A battery management node might expose a /get_battery_status service for periodic health checks

- Messages: Data structures defining content exchanged via topics or services. Message definitions (.msg files) are compiled into language-specific code during build time, ensuring type-safe communication.

Figure 5.1.1-1: ROS Service concept diagram [i.2]

### 5.1.2	Implementation structure

ROS middleware bridges high-level APIs and low-level communication protocols. Key technical elements include:

- RMW (ROS Middleware Interface): A language-agnostic abstraction layer that connects ROS to middleware implementations like DDS (Data Distribution Service). For example, rmw_fastrtps_cpp integrates ROS with Fast DDS.

- XML-RPC: In ROS1, XML-RPC (Extensible Markup Language Remote Procedure Call) was used as the primary protocol for communication between nodes and the centralized ROS Master. XML-RPC is a stateless, HTTP-based protocol that allows remote procedure calls encoded in XML to be transmitted over a network.

- DDS Integration: In ROS 2, Data Distribution Service (DDS) as an OMG standard provides real-time communication features such as:

  - Automatic Discovery: Nodes self-register without centralized coordination.

  - QoS Policies: Configurable parameters for reliability, latency, and security.

- Message Serialization: ROS messages are serialized using IDL (Interface Definition Language) for cross-language compatibility. For example, std_msgs/String maps to string data in C++/Python.

### 5.1.3	Use cases

ROS middleware is deployed in diverse domains:

- Autonomous Vehicles: The open-source Autoware.Auto project, built on ROS 2, planning (A* algorithm), and control modules. Its QoS policies ensure real-time obstacle detection even on unstable networks.

- Space Exploration: NASA’s VIPER lunar rover uses ROS 2’s fault-tolerant architecture to manage navigation and scientific payloads in extreme environments.

## 5.2	ROS1

### 5.2.1	Centralized architecture

ROS1 provides a centralized architecture centered around the ROS Master, a core component that is responsible for coordinating node discovery and communication routing. This design simplifies initial system setup and resource management, making it well-suited for prototypes and small-scale research applications.

The communication protocol in ROS1 utilizes XML-RPC for node registration and connection negotiation, complemented by TCP/UDP for message transport. XML-RPC was selected during ROS1’s development for its cross-language compatibility and ease of debugging. However, this protocol prioritizes simplicity over advanced functionality, omitting built-in support for dynamic QoS configurations.

In terms of operating system support, ROS1 was primarily optimized for Linux-based systems, reflecting the dominant operating system in robotics research at the time. ROS1 can run on Windows using tools like rosserial, but it works best and has more features on Linux.

## 5.3	ROS2

### 5.3.1	Decentralized Architecture

ROS2 eliminates the ROS Master, adopting DDS for peer-to-peer communication. Key advancements include:

- QoS Policies: ROS2’s Quality of Service (QoS) system enables fine-grained control over communication reliability and resource management [i.3]. Key policies include:

  - Reliability

    - Best Effort: Prioritizes timely delivery over guaranteed delivery (e.g., high-frequency sensor data).

    - Reliable: Ensures message delivery through retries (e.g., emergency stop commands).

  - Durability

    - Volatile: Discards messages for late-joining subscribers.

    - Transient Local: Retains messages for late-joining subscribers (e.g., map data).

  - Deadline: Specifies the maximum allowable latency between messages (μs precision). Critical for real-time systems like motor control.

  - Liveliness

    - Automatic: Node liveliness is inferred from message activity.

    - Manual: Requires explicit node status assertions.

- Predefined QoS Profiles: ROS2 provides optimized profiles for common use cases for example:

  - Sensor Data: Best effort reliability + small queue depth for real-time prioritization.

  - Services: Reliable delivery + volatile durability to avoid processing stale requests.

- ROS2 supports multiple DDS implementations including the open sources:

  - Fast DDS is optimized for low-latency communication (e.g., robotic arms).

  - Cyclone DDS implements OMG DDS specification with minimal resource usage and suitable for embedded systems.

- Nodes lifecycle management: Nodes with lifecycle states are called Managed Nodes, and their transitions between states (Unconfigured → Inactive → Active) enable safe firmware updates without restarting the entire system.

- Micro-ROS: A lightweight variant for embedded systems (e.g., STM32 microcontrollers). Supports real-time tasks like motor control with a memory footprint under 256KB.

### 5.3.3	Security framework

SROS2 (Secure ROS2) enforces access control through DDS-Security integration, leveraging X.509 certificates and access control polices [i.4].

- Authentication: Nodes use X.509 certificates signed by a Certificate Authority (CA)

- Access Control: SROS2 integrates DDS-Security, where access control is enforced via XML policies translated to DDS permissions.

## 5.4	Benefits of oneM2M-ROS interworking

# 6	Interworking scenarios

## 6.1	ROS 2 protocol

### 6.1.1	ROS 2 communication types

ROS 2 defines three standardized communication patterns —Topics, Services, and Actions — which are essential abstractions for enabling data exchange between nodes in a distributed robotic system. These communication types are all built upon the underlying DDS middleware and its underlying Real Time Publish Subscribe (RTPS) protocol, which ensures real-time, peer-to-peer data delivery through configurable Quality of Service (QoS) policies. Each communication type is designed for different application scenarios.

Topics are the most commonly used communication method in ROS 2 and implement a many-to-many publish/subscribe model. This model is asynchronous and unidirectional: data flows from publishers to subscribers without requiring a response. Topics are particularly well suited for scenarios that require high-frequency, continuous data streaming such as robot sensor readings (e.g., camera images, LIDAR point clouds) or actuator status updates. With appropriate QoS settings, topic communication can balance latency, throughput, and reliability depending on application requirements. For instance, a best-effort QoS can be acceptable for a high-rate depth sensor stream, while reliable delivery is more suitable for transmitting control signals.

Services provide a synchronous request-response communication pattern. A client sends a request to the service server to initiate an operation. The response from the server can be received either synchronously or asynchronously, where the client continues execution and handles the response later. This model is appropriate for atomic operations that require a response, such as querying the robot’s battery level, setting system parameters, or triggering specific routines. Because services do not inherently support long execution times or intermediate feedback, they are not ideal for complex or time-consuming tasks. Services in ROS 2 are implemented over DDS using the Requester-Replier abstraction, and use a pair of DDS topics for request and response channels.

Actions extend the service model to support asynchronous, goal-oriented tasks that may take much longer time to complete and can be preempted or monitored in progress. This is especially relevant in robotics domains where a task such as navigating to a location that could require ongoing feedback and cancellation support. Actions leverage both service and topic communication internally: a goal is sent via a service, feedback is streamed through a topic, and the final result is retrieved through another service. Each goal is uniquely identified by a UUID, enabling robust tracking of multiple concurrent tasks.

Table 6.1.1-1: Usage of ROS 2 communication types

| Use case | Communication Type | Description |
| --- | --- | --- |
| Real-time sensor streaming | Topic | Low-latency, unidirectional, supports high-frequency data |
| Immediate command execution | Service | Blocking, synchronous request-response |
| Multi-step task execution | Action | Long-duration, asynchronous, supports feedback and cancel |

### 6.1.2	Topic protocol

Topic-based communication in ROS 2 is implemented over the DDS middleware. This communication mechanism follows a publish-subscribe paradigm, enabling multiple nodes to exchange information efficiently in a decentralized and asynchronous manner. The process begins with each ROS 2 node creating a DDS DomainParticipant, which acts as an interface to the DDS infrastructure. Subsequently, the node registers the required message type through the ROS-generated TypeSupport mechanism. Once the message type is registered, the node can create a Publisher or Subscriber associated with a specific topic name.

A critical aspect of topic-based communication is the configuration of Quality of Service (QoS) policies. These policies govern communication behavior by defining delivery reliability, message durability, deadline expectations, and liveliness constraints. Based on these QoS settings, DataWriters (for publishers) and DataReaders (for subscribers) are instantiated. The actual message flow occurs via DDS's brokerless peer-to-peer transport. DDS handles discovery through the Simple Participant Discovery Protocol (SPDP) and Simple Endpoint Discovery Protocol (SEDP), allowing publishers and subscribers to match automatically based on topic names and type compatibility.

Figure 6.1.2-1: Communication type – Topic

In ROS 2 topic communication, messages are composed of two main parts:

- Header: Contains metadata such as timestamps, frame ID, and message sequence number (when applicable).

- Payload: Represents the application-specific data, which is defined by a user-defined .msg file and serialized using the Common Data Representation (CDR) standard.

### 6.1.3	Service protocol

In ROS 2, the service communication model is implemented using the request-reply paradigm of DDS. A service server exposes two topics: one for receiving requests and another for sending responses. The client constructs a request message and sends it to the server, which processes the request and sends a corresponding response.

The server is implemented using the DDS Replier interface, while the client is implemented using the DDS Requester interface. When a service client initiates communication, DDS performs discovery to locate the matching server. This is done using the SEDP (Simple Endpoint Discovery Protocol). Once the client and server are matched, a TCP channel or similar DDS-supported transport is used for message exchange.

- Request Message:

  - Header: Includes REQUEST_ID and client metadata.

  - Payload: CDR-serialized user-defined request data.

- Response Message:

  - Header: Includes matching REQUEST_ID to correlate response.

  - Payload: Serialized response data, potentially including status codes.

Figure 6.1.3-1: Communication type – Service

### 6.1.4	Action protocol

Actions in ROS 2 provide a framework for handling asynchronous and potentially long-running operations such as navigation, trajectory execution, or complex state transitions. Unlike services, which are synchronous and block until completion, actions support progressive feedback, cancellation, and goal/result separation.

The implementation of ROS 2 actions leverages both topic and service communication paradigms. When a client initiates an action, it sends a goal to a predefined send_goal service. This service processes the goal request, assigns it a unique UUID, and transitions the goal into an active state if accepted. Feedback is streamed from the server to the client using a dedicated feedback topic. Clients can subscribe to this topic to receive real-time updates. Upon completion or cancellation, the server delivers the final outcome via the get_result service. Additionally, a cancel_goal service allows the client to withdraw an ongoing goal.

Internally, ROS 2 actions use a predefined set of DDS entities derived from the action name:

- /namespace/action/_action/send_goal (Service)

- /namespace/action/_action/feedback (Topic)

- /namespace/action/_action/get_result (Service)

- /namespace/action/_action/cancel_goal (Service)

- /namespace/action/_action/status (Topic)

These endpoints are automatically managed by the action server and client classes, and developers do not need to implement them manually but typically uses abstract interfaces.

Each Action message is consists of:

- Goal Message: Contains a unique goal ID (UUID) and execution parameters.

- Feedback Message: Includes progress indicators (e.g., percent completion).

- Result Message: Provides final status (e.g., success, abort, cancel) and result values.

Figure 6.1.4-1: Communication type – Action

The use of UUID-based tracking and state machines allows ROS 2 actions to manage multiple concurrent goals, maintain goal lifecycles, and facilitate robust goal cancellation and progress tracking. Developers simply define an .action file, and the build system generates all necessary infrastructure, making it easy to use actions without deep knowledge of the underlying DDS mechanisms.

# 7	Resource mapping

# 8	Call flows

# 9	Conclusions

## Annex <A>:
Title of annex

<Text>

<PAGE BREAK>

Annex <B>:
Title of annex (style H9)

<Text>

B.1	First clause of the annex (style H1)

<Text>

B.1.1	First subdivided clause of the annex (style H2)

<Text>

<PAGE BREAK>

Annex <y>:
Bibliography

The annex entitled "Bibliography" is optional.

It shall contain a list of standards, books, articles, or other sources on a particular subject which are not mentioned in the document itself.

It shall not include references mentioned in the document.

Use the Heading 9 style for the title and B1+ or Normal for the text.

<Publication>: "<Title>".

OR

<Publication>: "<Title>".

<PAGE BREAK>

# History

This clause shall be the last one in the document and list the main phases (all additional information will be removed at the publication stage).

| Publication history |  |  |
| --- | --- | --- |
| V1.1.1 | <yyyy-mm-dd> | <Milestone> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V0.0.1 | 2025-03-10 | Skeleton of the TR. |
| V0.1.0 | 2025-07-19 | Incorporates the agreed contribution at SDS#69: - SDS-2025-0046R01-overview_of_ROS |
| V0.2.0 | 2025-07-25 | Incorporates the agreed contribution at SDS#69: - SDS-2025-0113R01-ROS_communication_types |