| oneM2M Technical Report |  |
| --- | --- |
| Document Number | TR-0071-V-0.1.0 |
| Document Name: | AI enablement to oneM2M (Stage 2) |
| Date: | 2024-06-26 |
| Abstract: | The document provides a summary of identified AI/ML-related requirements at Stage 1. It then investigates potential architectural solutions to support these identified AI/ML requirements. Based on the findings of this technical report, new features will be proposed to enable AI/ML in the oneM2M system. |
| 'Template Version: January 2019 (do not modify) |  |

The present document is provided for future development work within oneM2M only. The Partners accept no liability for any use of this report.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

© 2023, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TSDSI, TTA, TTC).

All rights reserved.

The copyright and the foregoing restriction extend to reproduction in all media.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

Contents	3

1	Scope	4

2	References	4

2.1	Normative references	4

2.2	Informative references	4

3	Definition of terms, symbols and abbreviations	4

3.1	Terms	4

3.2	Symbols	5

3.3	Abbreviations	5

4	Conventions	5

5	Introduction	5

6	Identified AI/ML Requirements	5

7	Proposed Solutions	6

7.1	AI/ML model management	6

7.1.1	Overview	6

7.1.2	Resource types	7

7.1.2.1	Resource Type modelRepo	7

7.1.2.2	Resource Type mlModel	7

7.1.2.3	Resource Type modelDeploymentList	8

7.1.2.4	Resource Type modelDeployment	9

7.1.3	Procedures	10

7.1.3.1	Procedures for <modelRepo>	10

7.1.3.2	Procedures for <mlModel>	10

7.1.3.3	Procedures for <modelDeploymentList>	10

7.1.3.4	Procedures for <modelDeployment>	10

7.n	Solution n	10

8	Conclusions	11

Annex <A>: Title of annex	12

Annex <y>: Bibliography	12

History	12

# 1	Scope

The current document provides a summary of identified AI/ML-related requirements at Stage 1. It then investigates potential architectural solutions to support these identified AI/ML requirements. Based on the findings of this technical report, new features will be proposed to enable AI/ML in the oneM2M system.

# 2	References

The following text block applies.

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references,only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## 2.1	Normative references

As a Technical Report (TR) is entirely informative it shall not list normative references.

The following referenced documents are necessary for the application of the present document.

Not applicable.

## 2.2	Informative references

Clause 2.2 shall only contain informative references which are cited in the document itself.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules  (http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc)

# 3	Definition of terms, symbols and abbreviations

Delete from the above heading the word(s) which is/are not applicable.

## 3.1	Terms

Clause numbering depends on applicability.

A definition shall not take the form of, or contain, a requirement.

The form of a definition shall be such that it can replace the term in context. Additional information shall be given only in the form of examples or notes (see below).

The terms and definitions shall be presented in alphabetical order.

For the purposes of the present document, the [following] terms and definitions [given in ... and the following] apply:

Definition format

<defined term>: <definition>

If a definition is taken from an external source, use the format below where [N] identifies the external document which must be listed in Section 2 References.

<defined term>[N]: <definition>

example 1: text used to clarify abstract rules by applying them literally

NOTE:	This may contain additional information.

## 3.2	Symbols

Clause numbering depends on applicability.

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

Symbol format

<symbol>	<Explanation>

<2nd symbol>	<2nd Explanation>

<3rd symbol>	<3rd Explanation>

## 3.3	Abbreviations

Abbreviations should be ordered alphabetically.

Clause numbering depends on applicability.

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

Abbreviation format

<ABBREVIATION1>	<Explanation>

<ABBREVIATION2>	<Explanation>

<ABBREVIATION3>	<Explanation>

# 4	Conventions

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# 5	Introduction

Editor’s Note: This section summarises the contents of this Technical Report.

# 6	Identified AI/ML Requirements

Editor’s Note: The section provides identified AI/ML related requirements at Stage 1.

# 7	Proposed Solutions

Editor’s Note: The section provides various AI/ML solutions for potential requirements.

## 7.1	AI/ML model management

### 7.1.1	Overview

In this AI/ML model repository and deployment management feature, there are four new resource types suggested.  For a CSE, there can be one or more <modelRepo> resource(s) for different services. The repository contains <mlModel> resources and each model contains an ML model and its meta data.

For an AE which represents an AIoT device (e.g. AI robot), AI service users can deploy model(s) to the device. To deploy models, a <modelDeploymentList> resource which works as the container of ML model deployments needs to be created first. A <modelDeployment> resource includes the IDs to the ML model, inference input and output resources. Multiple ML models can be deployed on an AIoT device which has one or more sensor(s) that provides inference input.

Figure 7.1.1-1: Resource tree structure for ML model management

NOTE: An alternative approach regarding the exising oneM2M device management capability is FFS

### 7.1.2	Resource types

#### 7.1.2.1	Resource Type modelRepo

The <modelRepo> resource represents a repository of ML models while each model is stored in a child <mlModel> resource. There can be more than one ML model repositories in a CSE for multiple ML-based services.

Table 7.1.2.1-1: Child resources of <modelRepo> resource

| Child Resources of <modelRepo> | Child Resource Type | Multiplicity | Description | <modelRepoAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 [1] | <semanticDescriptor>, <semanticDescriptorAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 [1] | <subscription> |
| [variable] | <mlModel> | 0..n | See clause 7.1.2.2 | <mlModelAnnc> |
| [variable] | <latest> | 1 | See clause 9.6.27 [1] | None |
| [variable] | <oldest> | 1 | See clause 9.6.28 [1] | None |

The <modelRepo> resource shall contain the attributes specified in table 7.1.2.1-2.

Table 7.1.2.1-2: Attributes of <modelRepo> resource

| Attributes of <modelRepo> | Multiplicity | RW/ RO/ WO | Description | <modelRepoAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3 [1]. | NA |
| parentID | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3 [1] | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | MA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | NA |
| announceSyncType | 0..1 | RW | See clause 9.6.1.3 [1]. | MA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | OA |
| creator | 0..1 | RO | See clause 9.6.1.3 [1]. | NA |
| custodian | 0..1 | RW | See clause 9.6.1.3 [1]. | NA |
| currentNumberOfModels | 1 | RO | The current number of ML models in the repository. | OA |
| currentByteOfModels | 1 | RO | The current total byte size of ML models in the repository. | OA |
| maxNumberOfModels | 0..1 | RW | The maximum number of ML models which can be stored in the repository. | OA |
| maxByteOfModels | 0..1 | RW | The maximum byte size of ML models which can be stored in the repository. | OA |

#### 7.1.2.2	Resource Type mlModel

The <mlModel> resource represents an ML model and is a child of a <modelRepo> resource.

Table 7.1.2.2-1: Child resources of <mlModel> resource

| Child Resources of <mlModel> | Child Resource Type | Multiplicity | Description | <mlModelAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 [1] | <semanticDescriptor>, <semanticDescriptorAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 [1] | <subscription> |

The <mlDatasetPolicy> resource shall contain the attributes specified in table 7.1.2.2-2.

Table 7.1.2.2-2: Attributes of <mlModel> resource

| Attributes of <mlModel> | Multiplicity | RW/ RO/ WO | Description | <mlModelAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3 [1]. | NA |
| parentID | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3 [1] | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | MA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | NA |
| announceSyncType | 0..1 | RW | See clause 9.6.1.3 [1]. | MA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | OA |
| creator | 0..1 | RO | See clause 9.6.1.3 [1]. | NA |
| custodian | 0..1 | RW | See clause 9.6.1.3 [1]. | NA |
| name | 0..1 | RW | The human readable name of the ML model. | OA |
| version | 1 | RW | The version of the ML model. | OA |
| platform | 1 | RW | The ML platform that trained the model. (e.g. tensorFlow) | OA |
| mlType | 1 | RW | The type of ML algorithm (e.g. regression, classification). | OA |
| description | 0..1 | RW | The human readable description of the ML model. | OA |
| inputSample | 0..1 | RW | The sample data of inference input. | OA |
| outputSample | 0..1 | RW | The sample data of inference output. | OA |
| mlModel | 0..1 | RW | The Base 64 encodedbinary ML model. When a model consists more than one file, than the zipped file is stored in this attribute. This cannot be present with mlModelURL. | OA |
| mlModelSize | 0..1 | RO | The byte size of the ML model stored in mlModel. | OA |
| mlModelURL | 0..1 | RW | The URL of the ML model. This cannot be present with mlModel. | OA |

#### 7.1.2.3	Resource Type modelDeploymentList

The <modelDeploymentList> resource represents the meta data of ML model deployements while each ML model deployment is respresented in a child <modelDeployment> resource.  This resource can be created as a child of an <AE> resource which represents an IoT device, so the deployment list resouce can work as a list of remote ML model deployments.

Table 7.1.2.3-1: Child resources of <modelDeploymentList> resource

| Child Resources of <modelDeployment>List> | Child Resource Type | Multiplicity | Description | <modelDeploymentList> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 [1] | <semanticDescriptor>, <semanticDescriptorAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 [1] | <subscription> |
| [variable] | <modelDeployment> | 0..n | See 7.1.2.4 | <modelDeploymentAnnc> |

The <modelDeploymentList> resource shall contain the attributes specified in table 7.1.2.3-2.

Table 7.1.2.3-2: Attributes of <modelDeploymentList> resource

| Attributes of <modelDeploymentList> | Multiplicity | RW/ RO/ WO | Description | <modelDeploymentListAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3 [1]. | NA |
| parentID | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3 [1] | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | MA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | NA |
| announceSyncType | 0..1 | RW | See clause 9.6.1.3 [1]. | MA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | OA |
| creator | 0..1 | RO | See clause 9.6.1.3 [1]. | NA |
| custodian | 0..1 | RW | See clause 9.6.1.3 [1]. | NA |
| numberOfDeployedModels | 1 | RO | The the number of ML models whose status is “deployed” among the models in this deployment list. | OA |
| numberOfRunningModels | 1 | RO | The the number of ML models whose status is “running” among the models in this deployment list. | OA |
| numberOfStoppedModels | 1 | RO | The the number of ML models whose status is “stopped” among the models in this deployment list. | OA |

#### 7.1.2.4	Resource Type modelDeployment

The <modelDeployment> resource represents an ML model deployment. An ML AE (e.g. AIoT device) is expected to subscribe to the parent <modelDeploymentList> resource for the newly created <modelDeployment> resource. When the AE gets the new ML model deployment, it retrieves the ML model by the modelID attribute. Once the AE serves the model, then it gets the inference input by the inputResource and run the ML model. Then the AE stores the inference result into the resource indicated by the outputResource. A referred resource by inputResource and outputResource are aresource sharing resources (e.g. <container> resource).

Another AE can remotely manipulate the model status with the modelCommand attribute. It can run or stop the deployed model.

Table 7.1.2.1-1: Child resources of <modelDeployment> resource

| Child Resources of <modelDeployment> | Child Resource Type | Multiplicity | Description | <modelDeploymentAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 [1] | <semanticDescriptor>, <semanticDescriptorAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 [1] | <subscription> |

The <modelDeployment> resource shall contain the attributes specified in table 7.1.2.1-2.

Table 7.1.2.1-2: Attributes of <modelDeployment> resource

| Attributes of <modelDeployment> | Multiplicity | RW/ RO/ WO | Description | <modelDeploymentAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3 [1]. | NA |
| parentID | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3 [1] | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | MA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | NA |
| announceSyncType | 0..1 | RW | See clause 9.6.1.3 [1]. | MA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | OA |
| creator | 0..1 | RO | See clause 9.6.1.3 [1]. | NA |
| custodian | 0..1 | RW | See clause 9.6.1.3 [1]. | NA |
| modelID | 1(L) | WO | The resource ID of the <mlModel> resource that is deployed. | OA |
| modelCommand | 1 | RW | This attribute is not returned in a response, but can be included in an Update request to update the modelStatus. Allowed values for an Update request is “run” and “stop”. | NA |
| modelStatus | 1 | RO | The status of the deployed ML model which is “deployed”, “running” or “stopped”. The default is “deployed” | OA |
| inputResource | 1 | RW | The resource ID of the inference input resource. | OA |
| outputResource | 1 | RW | The resource ID of the inference output. | OA |

### 7.1.3	Procedures

#### 7.1.3.1	Procedures for <modelRepo>

No change from the CRUD procedures in clause 10.1 [1].

#### 7.1.3.2	Procedures for <mlModel>

No change from the CRUD procedures in clause 10.1 [1].

#### 7.1.3.3	Procedures for <modelDeploymentList>

No change from the CRUD procedures in clause 10.1 [1].

#### 7.1.3.4	Procedures for <modelDeployment>

In a <modelDeployment> Update procedure, when the modelCommand attribute is included in the Update request, depending on the requested value, the Hosting CSE shall update the modelStatus attribute. The status value by default after the resource creation is “deployed”. For the start “run” and “stop” command, the status shall be updated as “running” and “stopped”, respectively. When the modelStatus attribute is updated, the Hosting CSE shall update the corresponding attributes (i.e. numberOfDeployedModels, numberOfRunningModels, numberOfStoppedModels) of a parent <modelDeploymentList> resource as well.

No change from the Create, Retrieve and Delete procedures in clause 10.1 [1].

## 7.n	Solution n

# 8	Conclusions

Editor’s Note: This section provides a summary of the conclusions drawn during the study.

## Annex <A>:
Title of annex

<Text>

<PAGE BREAK>

Annex <B>:
Title of annex (style H9)

<Text>

B.1	First clause of the annex (style H1)

<Text>

B.1.1	First subdivided clause of the annex (style H2)

<Text>

<PAGE BREAK>

Annex <y>:
Bibliography

The annex entitled "Bibliography" is optional.

It shall contain a list of standards, books, articles, or other sources on a particular subject which are not mentioned in the document itself.

It shall not include references mentioned in the document.

Use the Heading 9 style for the title and B1+ or Normal for the text.

<Publication>: "<Title>".

OR

<Publication>: "<Title>".

<PAGE BREAK>

# History

This clause shall be the last one in the document and list the main phases (all additional information will be removed at the publication stage).

| Publication history |  |  |
| --- | --- | --- |
| V1.1.1 | <yyyy-mm-dd> | <Milestone> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V0.0.1 | 2023-06-27 | Skeleton of the TR. |
| V0.1.0 | 2024-06-26 | This baseline contains agreed contributions from SDS#62: - SDS-2023-0172R01-solutions_for_ML_model_management |